3GPP TSG-WG SA2 Meeting #157	S2-2306597

# Berlin, Germany, May 22-26, 2023   		(revision of S2-230xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0824 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates on data collection from LCS system |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-04-21 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Aspect#1: S2-2303753 proposed to enable indoor/outdoor indication as an output of LMF. Therefore, it means NWDAF can also obtain this information from GMLC. It is proposed to add this parameter in section 6.2.12.1 general description part. Aspect#2: S2-2303372 (got updates in S2-2305587 in SA2#156e) proposed to enable NWDAF to request aggregated report for a single UE. It requires the NWDAF provides a reporting time as the latest time for reporting. However, TS23.288 also needs to emphasize this part for easier location info retrieval by NWDAF from GMLC. Aspect#3: Besides, it is also proposed to resolve the EN, Editor's note: Based on which analytics ID that NWDAF determines to obtain UE location from LCS system will be added in the future. Analytics IDs that utilize finer granularity location information are: - 6.7.2 UE mobility analytics - 6.9 QoS sustainability analytics - 6.17 LCS accuracy analytics - 6.19 Relative proximity analytics - 6.x Traffic flow analytics (name subjects to modification, agreed in S2-2306105 in SA2#156e) Among which, - 6.9 QoS sustainability analytics requires the Location information in Analytics filter information to be finer granularity area instead of TA/cell defined AoI. - 6.17 LCS accuracy analytics collects input data from GMLC for the pre-training model, which has no dependency on NWDAF consumer input data - 6.7.2 UE mobility analytics and 6.x traffic flow analytics both require preferred granularity of location as below cell level in Analytics reporting information. - 6.19 relative proximity analytics mainly depends on the analytics ID to determine from where to fetch UE location. Therefore, it is proposed to make above analytics ID as examples and delete the EN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Indoor/outdoor indication is added EN is deleted Description of analytics ID is added Description of requesting aggregated location report is added |  |  |  |  |  |  |  |  |
| -- |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not complete TS |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

## ****First Changes****

### 6.2.12	Data Collection from LCS system

#### 6.2.12.1	General

The NWDAF may collect location information for a target UE or a group of target UEs from LCS system. The collected location related information can include:

‐	Location estimate of the UE;

-	Time stamp of location estimate;

-	Velocity of the UE as defined in TS 23.032 [34];

-	Information about the positioning method used to obtain the location estimate of the UE;

-	Indication of Indoor/Outdoor for the location estimate

-	Indication of area event, when UE enters, is within or leaves the Geographical area;

-	Indication of motion event when UE moves by more than some predefined straight line distance from a previous location.

NOTE:	The location information that can be retrieved is defined within the location service response in clause 5.5 of TS 23.273 [39].

NWDAF shall use Ngmlc service as defined in TS 23.273 [39] to have access to the information provided by LCS system. Only Mobile Terminated Location Request (MT-LR) is supported, including both Immediate Location Request and Deferred Location Request.

NWDAF may determine to query LCS system instead of AMF to obtain UE's location information based on the following attributes as received from NWDAF consumer:

-	analytics ID (e.g. QoS sustainability analytics, LCS accuracy analytics, relative proximity analytics, UE traffic analytics);

-	Preferred granularity of location information.

#### 6.2.12.2	Procedure for data collection from LCS system

The interactions between NWDAF and LCS system for data collection are illustrated in Figure 6.2.12.2-1. The data collected depends on the use cases. This figure is an abstraction of how NWDAF collects location information from LCS system. The actual procedures that NWDAF may use are as follows:

-	For a target UE, both 5GC-MT-LR procedure for the commercial location service as specified in clause 6.1.2 and deferred 5GC-MT-LR procedure as specified in clause 6.3 of TS 23.273 [39] can be utilized;

-	For a group of target UEs, bulk operation of LCS Service Request Targeting to Multiple UEs as specified in clause 6.8 of TS 23.273 [39] can be utilized.

Figure 6.2.12.2-1: Data collection from LCS system

1.	NWDAF requests the location related information from GMLC about a target UE (that may be identified by a SUPI) or a group of target UEs (identified by a group ID). The NWDAF can also request an aggregated report of UE location related information from GMLC about a target UE.

2.	GMLC interacts within LCS system to obtain the UE's location information. If privacy verification is required, GLMC will interact with UE before sending the location information to NWDAF.

3.1	If it is Immediate Location Request, GMLC sends the location service response including the location estimate to the NWDAF.

3.2	If it is Deferred Location Request, GMLC sends the location service response including the indication of event occurrence to the NWDAF.

## ****Second Changes****

## End of Changes