3GPP TSG-WG SA2 Meeting #157	S2-2306600

# Berlin, Germany, May 22 – 26, 2023		(was_S2-2300857, 2756, 4665, 6106)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0662 | rev | 4 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New analytics ID for traffic flow use case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo, KDDI, Orange |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | TR 23.700-81 v2.0.0 has documented the conclusion for key issue#9 “Enhancement of NWDAF with finer granularity of location information”. As to the new use cases, the following conclusions are reached: “- NWDAF supports the following new Analytics IDs: - Analytics ID that supports collecting UE location when the UE is exactly in the requested area instead of the translated TA/cell list. - Analytics ID ‘Relative Proximity’ provides support for finer granularity of location information at NWDAF via relative proximity information of nearby Ues. The output analytics contain relative proximity and Time to Collision information. - Analytic ID that supports the traffic flow statistics use case. - Analytic ID that supports location accuracy estimate use case. NOTE 1: That new Analytics IDs to support these use cases are required (instead of enhancing Analytics IDs) will be confirmed during normative work. ” This contribution provides the analytics detail of the UE flow use case based on the finer granularity location information. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce a new Analytics ID for the UE flow use case in the TS 23.288. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not specified that the procedure for the “traffic flow” of “Enhancement of NWDAF with the finer granularity of location information” in R18 eNA study. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.x (new), 6.x.1 (new), 6.x.2 (new), 6.x.3 (new), 6.x.4 (new), 7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR … CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This ’CR's revision history: |  | Rev.4: - The traffic flow analytics were defined without analytics subsets but the consumer may be only interested in total number of users, average speed, speed threshold or direction. The consumer may be also interested in ordering of results of the analytics subsets. Thus, the revision proposes to add "list of analytics subsets that are requested" and "preferred level of accuracy per analytics subset" as Analytics Filter Information. - Editor’s Note from rev. 3 resolved by replacing “traffic flow” by “UE flow”. - Include AMF obtained UE location. - Include UE flow analytics in Table 7.1.2 - Modify “below cell level” to be “longitude and latitude level” based on S2-2306105 as agreed in SA2#156e. |  |  |  |  |  |  |  |  |

# * * * * First change (all new )* * * *

## 6.x	UE Flow Analytics

### 6.x.1	General

The UE Flow analytics provides analytics information (statistics or predictions) regarding the location, direction and velocity of UEs during a analytics target periodin a target area. The statistics of UE flow collects a map of UE movements in a certain region in a certain period of time. In addition, the consumers can obtain the congestion situation in the target area through statistics.

The NWDAF provides UE flow analytics to a NF (e.g. NEF, AF).

The consumer of these analytics shall indicate in the request or subscription:

-	Analytics ID = "UE flow";

-	Target of Analytics Reporting: any UE;

-	Analytics Filter Information:

-	Area of Interest (AOI): restricts the scope of the UE flow analytics to the provided area. The AOI may be described as shown in clause 5.5 of 3GPP TS 23.273 [39];

-	Optionally, the list of analytics subsets that are requested among those specified in clause 6.x.3;

-	An Analytics target period indicates the time period over which the statistics or predictions are requested;

-	Optionally, preferred level of accuracy of the analytics;

-	Preferred level of accuracy per analytics subset (see clause 6.x.3);

-	Optionally, preferred granularity of location information: "longitude and latitude level";

-	Optionally, preferred orientation of location information: ("horizontal", "vertical", "both")"; and

-	Optionally, maximum number of objects.

### 6.x.2	Input data

The NWDAF collects the UE location and velocity information within the area of interest from the sources listed in Table 6.x.2-1.

Table 6.x.2-1: Data collection by NWDAF for "UE flow" analytics

| Information | Source | Description |
| --- | --- | --- |
| UE ID | AMF | SUPI |
| UE locations (1..max) | AMF | UE positions |
| >UE location |  | TA or cells that the UE enters (NOTE) |
| >Timestamp |  | A time stamp when the AMF detects the UE enters this location |
| Fine granularity locations (1 …max) | GMLC | UE locations |
| …>Timestamp |  | Time information associated with the collected information. |
| …>LCS QoS |  | The achieved LCS QoS accuracy (see TS 23.273 [39]) |
| …>Location estimate |  | Geographical location or location in local coordinates (as defined in TS23.032) |
| …>Velocity estimate |  | Speed and direction information |
| …>Indoors/outdoor indication |  | Indicate the location estimate is indoor or outdoor |
| …>Age of location |  | Valid period of the location estimate |

### 6.x.3	Output analytics

The output analytics of UE flow provided by NWDAF is defined in Table 6.x.3-1 and Table 6.x.3-2.

Table 6.x.3-1: UE Flow statistics

| Information | Description |
| --- | --- |
| Applicable Area | A geograohical area as defined in TS 23.273 [39] that the analytics applies to. |
| Time slot entry (1..max) | List of time slots during the Analytics target period |
| > Time slot start | Time slot starts within the Analytics target period |
| > Duration | The time during the beginning and the end of UE location information collection |
| > Total number of users (NOTE) | Total number of users in the area of interest |
| > Average speed (NOTE) | Average speed of all users in the area of interest. |
| > Speed threshold (1...max) (NOTE) | Threshold utilized to fiter the users |
| >> Number of users | The number of users whose speed is faster than the speed threshold |
| >> Ratio | Percentage of users whose speed is faster than the speed threshold |
| > Direction (1…max) (NOTE) | Heading directions of the UE flow in the target area, such as north, south, east, west etc. |
| >> Number of users | The number of users in the specific direction. |
| >> Average speed | Average speed of users in the specific direction. |
| >> Ratio | Percentage of users. |
| NOTE: Analytics subset that can be used in "list of analytics subsets that are requested" and "Preferred level of accuracy per analytics subset". |  |

Table 6.x.3-2: UE Flow predictions

| Information | Description |
| --- | --- |
| Applicable Area | A geograohical area as defined in TS 23.273 [39] that the analytics applies to. |
| Time slot entry (1..max) | List of time slots during the Analytics target period |
| > Time slot start | Time slot starts within the Analytics target period |
| > Duration | The time during the beginning and the end of UE location information collection |
| > Total number of users (NOTE) | Total number of users in the area of interest |
| > Average speed (NOTE) | Average speed of all users in the area of interest. |
| > Speed threshold (1...max) (NOTE) | Threshold utilized to fiter the users |
| >> Number of users | The number of users whose speed is faster than the speed threshold |
| >> Ratio | Percentage of users whose speed is faster than the speed threshold |
| > Direction (1..max) (NOTE) | Heading directions of the UE flow in the target area, such as north, south, east, west etc. |
| >> Number of users | The number of users in the specific direction. |
| >> Average speed | Average speed of users in the specific direction. |
| >> Ratio | Percentage of users. |
| Confidence | Confidence of the prediction |
| NOTE: Analytics subset that can be used in "list of analytics subsets that are requested" and "Preferred level of accuracy per analytics subset". |  |

### 6.x.4	Procedures

Figure 6.x.4-1 depicts the procedure for UE flow analytics provided by NWDAF.

Figure 6.x.4-1: "UE flow" analytics provided by NWDAF

1.	The NWDAF receives an analytics request from the NWDAF service consumer (as defined in clause 6.1.1.1) or from an AF via NEF (as defined in clause 6.1.1.2) to request UE flow analytics within a target area for any UE.

The Analytics ID is set to "UE flow". The target for analytics reporting is set to be any UE. Analytic filters may be provided as shown in clause 6.x.1. The Area of Interest is the target area that may be defined as a geographical area or as a geopolitical name of an area, in such case, the NEF will translate it as the identities of one or more radio cells or tracking areas.

The Consumer NF can request statistics or predictions or both for a given Analytics target period.

2.	The NWDAF collects input data from AMF as defined in Table 6.x.2-1 via the AMF event exposure service.

3.	The NWDAF collects the UE location information as defined in Table 6.x.2-1 via Ngmlc_Location services.

Based on the analytics target period, the NWDAF may trigger the immediate 5GC-MT-LR procedure as defined in clause 6.1.2 or the defered 5GC-MT-LR procedure in clause 6.3.1 in TS 23.273 [39] to obtain UE’s location information.

4.	The NWDAF derives requested analytics.

5.	The NWDAF provides the analytics as defined in Table 6.x.3-1 and Table 6.x.3-2 using either the Nnwdaf_AnalyticsInfo_Request response or Nnwdaf_AnalyticsSubscription_Subscribe_Notify, depending on the service used in step 1.

# * * * * Second change * * * *

## 7.1	General

Table 7.1-1 illustrates the NWDAF Services.

Table 7.1-1: NF services provided by NWDAF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnwdaf_AnalyticsSubscription | Subscribe | Subscribe / Notify | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF |
|  | Unsubscribe |  | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF |
|  | Notify |  | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF, MFAF |
|  | Transfer | Request / Response | NWDAF |
| Nnwdaf_AnalyticsInfo | Request | Request / Response | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF |
|  | ContextTransfer | Request / Response | NWDAF |
| Nnwdaf_DataManagement | Subscribe | Subscribe / Notify | NWDAF, DCCF |
|  | Notify |  | NWDAF, DCCF, MFAF, ADRF |
|  | Fetch | Request / Response | NWDAF, DCCF, MFAF, ADRF |
| Nnwdaf_MLModelProvision | Subscribe | Subscribe / Notify | NWDAF |
|  | Unsubscribe |  | NWDAF |
|  | Notify |  | NWDAF |
| Nnwdaf_MLModelInfo | Request | Request / Response | NWDAF |
| Nnwdaf_MLModelMonitor | Subscribe | Subscribe / Notify | NWDAF |
|  | Unsubscribe |  | NWDAF |
|  | Notify |  | NWDAF |
|  | Register | Request / Response | NWDAF |
|  | Request |  | NWDAF |
| Nnwdaf_MLModelTraining | Subscribe | Subscribe / Notify | NWDAF |
|  | Unsubscribe |  | NWDAF |
|  | Notify |  | NWDAF |
| Nnwdaf_MLModelTrainingInfo | Request | Request / Response | NWDAF |
| Nnwdaf_RoamingAnalytics | Subscribe | Subscribe / Notify | H-NWDAF, V-NWDAF |
|  | Unsubscribe |  | H-NWDAF, V-NWDAF |
|  | Notify |  | H-NWDAF, V-NWDAF |
|  | Request | Request / Response | H-NWDAF, V-NWDAF |
| Nnwdaf_RoamingData | Subscribe | Subscribe / Notify | H-NWDAF, V-NWDAF |
|  | Unsubscribe |  | H-NWDAF, V-NWDAF |
|  | Notify |  | H-NWDAF, V-NWDAF |
| NOTE 1: How OAM consumes Nnwdaf services and which Analytics information is relevant is defined in TS 28.550 [7] Annex H and out of the scope of this TS. NOTE 2: How CEF consumes Nnwdaf services and which Analytics information is relevant is defined in TS 28.201 [21] and out of the scope of this TS. NOTE 3: The Nnwdaf_MLModelProvision service and the Nnwdaf_MLModelInfo service are provided by an NWDAF containing MTLF and consumed by an NWDAF containing AnLF. |  |  |  |

Editor´s note:	It is FFS whether separate Roaming Analytics and Roaming data services for inbound and outbound roaming scenarios are required.

Table 7.1-2 shows the analytics information provided by NWDAF service.

Table 7.1-2: Analytics information provided by NWDAF

| Analytics Information | Request Description | Response Description |
| --- | --- | --- |
| Slice Load level information | Analytics ID: load level information | Load level provided as number of UE registrations and number of PDU sessions for a Network Slice and Network Slice instances as well as resource utilization for Network Slice instances. |
| Observed Service experience information | Analytics ID: Service Experience | Observed Service experience statistics or predictions may be provided for a Network Slice or an Application. They may be derived from an individual UE, a group of UEs or any UE. For slice service experience, they may be derived from an Application, a set of Applications or all Applications on the Network Slice. |
| NF Load information | Analytics ID: NF load information | Load statistics or predictions information for specific NF(s). |
| Network Performance information | Analytics ID: Network Performance | Statistics or predictions on the load in an Area of Interest; in addition, statistics or predictions on the number of UEs that are located in that Area of Interest. |
| UE mobility information | Analytics ID: UE Mobility | Statistics or predictions on UE mobility. When visited AOI(s) is included in the Analytics Filter information, only statistics on UE mobility can be provided. |
| UE Communication information | Analytics ID: UE Communication | Statistics or predictions on UE communication. |
| Expected UE behavioural parameters | Analytics ID: UE Mobility and/or UE Communication | Analytics on UE Mobility and/or UE Communication. |
| UE Abnormal behaviour information | Analytics ID: Abnormal behaviour | List of observed or expected exceptions, with Exception ID, Exception Level and other information, depending on the observed or expected exceptions. |
| E2E data volume transfer time | Analytics ID: E2E data volume transfer time | Analytics on E2E data volume transfer time. |
| User Data Congestion information | Analytics ID: User Data Congestion | Statistics or predictions on the user data congestion for transfer over the user plane, for transfer over the control plane, or for both. |
| QoS Sustainability | Analytics ID: QoS Sustainability | For statistics, the information on the location and the time for the QoS change and the threshold(s) that were crossed; or, for predictions, the information on the location and the time when a potential QoS change may occur and what threshold(s) may be crossed. |
| Session Management Congestion Control Experience | Analytics ID: Session Management Congestion Control Experience | Statistics on session management congestion control experience for specific DNN and/or S-NSSAI. |
| Redundant Transmission Experience | Analytics ID: Redundant Transmission Experience | Statistics or predictions aimed at supporting redundant transmission decisions for URLLC services. |
| WLAN performance | Analytics ID: WLAN performance | Statistics or predictions on WLAN performance of UE. |
| Dispersion | Analytics ID: UE Dispersion | Statistics or predictions that identify the location (i.e. areas of interest) or network slice(s) where a UE, or a group of UEs disperse their data volume, or disperse mobility or session management transactions or both. |
| DN Performance | Analytics ID: DN Performance | Statistics or predictions on user plane performance for a specific Edge Computing application. |
| PFD Determination | Analytics ID: PFD Determination | Statistics on PFD information for a known application identifier(s). |
| UE flow | Analytics ID: UE flow | Statistics or predictions on UE flow for an applicable area |

# * * * * End of changes * * * *