SA WG2 Meeting #157	S2-2306615

Berlin, DE, May 22–26, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.503 | CR | 1050 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to UL/DL policy control based on RT latency requirement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 3GPP TS 23.501 version 18.1.0 include two separate functionalities related to round-trip time: - Related to KI#3, in clause 5.37.4: 5GS measurements of round trip delay for two QoS Flows of the XR service (when the UL and DL have different QoS requirements and are separated into two QoS flows) - Related to KI#6, in clause 5.37.6: UL/DL policy control based on round-trip latency requirement (when AF provides single service data flow UL+DL) The text in 23.503 clause 6.1.3.27.2 (UL/DL policy control based on round-trip latency requirement), includes a use case that is misplaced, as it corresponds to KI#3. This is the use case where the UL and DL traffic of the service have different QoS requirements. Note also that conclusion of TR 23.700-60 and text in TS 23.501 version 18.1.0 only define the situation where the AF provides single service data flow (UL+DL) that the PCF splits into two separate PCC rules (UL QoS Flow and DL QoS Flow). In addition, there are currently no restrictions in TS 23.503 for the AF to request to the PCF both: - UL/DL policy control based on round-trip latency requirement (new Rel-18 functionality in clause 6.1.3.27.2), and - Subscription to real-time measurements of QoS parameters for a service data flow (existing Rel-17 functionality in clause 6.1.3.21) However, the PCF cannot fulfil both AF requests at the same time. If the PCF, upon reception of RT latency indication, splits the service data flow into two separate PCC rules (UL QoS Flow and DL QoS Flow), then the PCF cannot send a QoS Monitoring policy for the original service data flow provided by the AF in a single PCC rule. Hence, QoS monitoring reports cannot be provided for a service data flow that is subject of RT latency requirement following existing Rel-17 mechanisms in clause 6.1.3.21. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the text in clause 6.1.3.27.2 about the use case where the UL and DL traffic of the service have different QoS requirements. This use case is misplaced, as it is already covered in TS 23.501 clause 5.37.4 (Network Exposure of 5GS information). In particular the text about “Round trip delay for multiple QoS Flows of the XR service (e.g. the UL and DL are separated into two flows)”, covers reporting RT delay for two QoS flows. UL/DL policy control is the functionality where PCF splits the QoS flow into two separate flows (one for UL and one for DL) in order to meet the required RT delay. The functionality is not applicable to separate QoS flows provided by the AF Make the following functionalities mutually exclusive: - UL/DL policy control based on round-trip latency requirement, and - Subscription to real-time measurements of QoS parameters for a service data flow |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect specification of the functionality of UL/DL policy control based on round-trip latency requirement |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3.27.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

##### 6.1.3.27.2	UL/DL policy control based on round-trip latency requirement

The AF may provide the round-trip (RT) latency requirement for an XR or other interactive media services with an RT latency indication via the AF session with required QoS procedure described in clause 6.1.3.22. The RT latency indication indicates the service data flow needs to meet the RT latency requirement of the service, which is twice the single direction delay requirement between the UE and the PSA UPF described by the QoS Reference parameter or individual QoS parameter.

Based on the RT latency requirement received from the AF, the PCF can split the RT latency requirement into two PDBs of two PCC rules, used for the UL QoS Flow and DL QoS Flow to carry the UL and DL traffics of the service respectively. The two PDBs can be unequal, but their sum shall not exceed the RT latency requirement.

NOTE 1:	RT latency requirement may also be locally configured in the PCF together with delay requirement.

To enable RT latency tracking, the PCF shall generate associated QoS monitoring policies for the two correlated QoS Flows. The uplink and downlink delay for the two QoS Flows shall be tracked by PCF independently with same reporting period.

When the QoS monitoring results are reported to PCF, the PCF can derive and track the RT latency by combining the QoS monitoring reports for the UL and the DL packet delay. The PCF may adjust the PDBs of the two PCC rules using SM Policy Association Modification procedure described in clause 4.16.5.2 of TS 23.502 [3] to better fit the new situation.

NOTE 2:	How the PCF derives the round-trip latency and takes policy decisions is up to implementation.

The AF cannot provide an RT latency requirement if the UL and DL traffic of the service data flow have different QoS requirements (e.g. different one-way delay).

The functionality of UL/DL policy control based on round-trip latency requirement is mutually exclusive with the subscription to real-time measurements of QoS parameters for a service data flow as defined in clause 6.1.3.21.

# * * * * End of changes * * * *