3GPP TSG-WG SA2 Meeting #157	S2-2306623

# May 22 – 26, 2023, Berlin, Germany

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0283 | rev |  | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Parameter update in 6.4.3.7.3 for Link Modification with L3 U2U Relay |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Interdigital |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For Layer3 UE-to-UE Relay, end to end link modification is agreed at SA2 #156e meeting. And it needs update parameters in 6.4.3.7.3 |  |  |  |  |  |  |  |  |
| Summary of change: |  | For adding new QoS flow or modifying existing QoS flow and for removing QoS flow(s), Description of Link Modification Request and Link Modification Accept are updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete Link Modification procedure with L3 U2U Relay. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.3.7.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

##### 6.4.3.7.3	Layer-2 link management over PC5 reference point for 5G ProSe Layer-3 UE-to-UE Relay

For the 5G ProSe Communication via 5G ProSe Layer-3 UE-to-UE Relay as described in clause 6.7.1, the description in clause 6.4.3.7.1 applies with following differences and clarifications:

-	In the Security Procedure of the first hop PC5 reference point, the source 5G ProSe End UE provides the IP Address Configuration, Link-Local IPv6 Address and QoS Info of the end-to-end QoS to the 5G ProSe Layer-3 UE-to-UE Relay.

-	In the Security Procedure of the second hop PC5 reference point, the 5G ProSe Layer-3 UE-to-UE Relay provides the IP Address Configuration, Link-Local IPv6 Address and QoS Info of the second hop QoS to the target 5G ProSe End UE.

-	The Direct Communication Accept message over the second hop PC5 reference point additionally includes IP Address Configuration, Link-Local IPv6 Address and QoS Info of the second hop QoS.

-	The 5G ProSe Layer-3 UE-to-UE Relay decides the QoS Info of the first hop QoS with considering the received second hop QoS, the Direct Communication Accept message over the first hop PC5 reference point additionally includes IP Address Configuration, Link-Local IPv6 Address, QoS Info of the first hop QoS and may include IP address of the target 5G ProSe End UE.

-	For adding new QoS flow or modifying existing QoS flow, in the Link Modification Request message over the first hop PC5 reference point, the source 5G ProSe End UE additionally includes QoS Info of the end-to-end QoS. For removing QoS flow(s), in the Link Modification Request message over the first hop PC5 reference point, the source 5G ProSe End UE includes PFI(s) of the first hop.

-	For adding new QoS flow or modifying existing QoS flow, in the Link Modification Request message over the second hop PC5 reference point, the 5G ProSe Layer-3 UE-to-UE Relay additionally includes QoS Info of the second hop QoS to the target 5G ProSe End UE. For removing QoS flow(s), in the Link Modification Request message over the second hop PC5 reference point, the 5G ProSe Layer-3 UE-to-UE Relay includes PFI(s) of the second hop.

-	For adding new QoS flow or modifying existing QoS flow, the Link Modification Accept message over the second hop PC5 reference point additionally includes QoS Info of the second hop QoS.

-	For adding new QoS flow or modifying existing QoS flow, the 5G ProSe Layer-3 UE-to-UE Relay decides the QoS Info of the first hop QoS with considering the received second hop QoS, the Link Modification Accept message over the first hop PC5 reference point additionally includes QoS Info of the first hop QoS and may include IP address of the target 5G ProSe End UE.

# * * * * End of change * * * *