SA WG2 Meeting #157	S2-2306624

22 - 26 May, 2023, Berlin, Germany

Title:	[draft]Reply LS on ProSe Secondary Authentication

Response to:	S2-2306302

Release:	Rel-18

Work Item:	PROSESA, 5G_ProSe_Ph2

Source:	SA2

To:	SA3

Cc:

Contact person:	Jung Je Son

jungje dot son at interdigital dot com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	CR S2-2303326

1.	Overall description

SA2 thanks SA3 for LS(S2-2306302, S3-232108). SA2 have discussed it and would like to provide following answers:

Q1. Relaying DNN subject to ProSe Secondary Authentication and subscription data:

- Remote UE can be configured for DNN subject to ProSe secondary authentication when the UE is authorized to act as a Remote UE which is based on PDU session subscription data and ProSe subscription data.

- Relay UE, SMF and PCF are configured with dedicated DNN(s) with indication whether ProSe secondary authentication is required.

- Please, refer to the attached CR(S2-2303326)

Q2. DNN subject to ProSe Secondary Authentication and usage in direct network connectivity:

- For Q2a, DNN for direct link and DNN used for serving Remote UE via Relay UE are different.

- For Q2b, DN-AAA server address can be locally configured in SMF of Relay UE or can be identified based on DN-specific identity which is independent of PLMN.

Q3. Determination that DNN is subject to ProSe Secondary Authentication:

- Relay UE and SMF can determine that a ProSe secondary authentication is required for a Remote UE based on configuration. Please, refer to the attached CR (S2-2303326).

Q4. Usage of Remote UE Report for ProSe Secondary Authentication initiation:

- For Q4a, Remote UE report can be used to trigger SMF to initiate a secondary authentication of the Remote UE.

- For Q4b, using each individual Remote User ID, SMF can identify whether a given Remote UE has successfully finished DN authorization with DN-AA and whether Remote UE needs DN authorization with DN-AAA. And Remote UE report per Remote UE can be used to trigger SMF to initiate a secondary authentication of the Remote UE.

Q5. ProSe Secondary Authentication and SMF and DN-AAA interface:

- For Q5a, SMF shall establish a new session with DN-AAA for each Remote UE, using existing interface definition.

- For Q5b, the interaction between SMF and DN-AAA shall be done on a per individual Remote UE basis identified by its addressing information and/or GPSI, using existing interface definition.

2. Actions:

To SA3:

Action: SA2 kindly asks SA3 to take the above information into account.

3. Date of Next TSG-SA WG2 Meetings:

|  | DATES | LOCATION | CTRY |
| --- | --- | --- | --- |
| SA2#158 | August 22-26th, 2023 | Gothenburg | Sweden |
| SA2#159 | October 09-13th, 2023 | Xiamen | China |

Please see https://portal.3gpp.org/Meetings?tbid=375&SubTB=385#/