SA WG2 Meeting #157	S2-2306672

# Berlin, DE,  2023-05-22 -- 2023-05-26

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 3973 | rev | 1 | Current version: | 18.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Including a latency parameter related to DNAI mapping |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGE_Ph2 |  |  |  |  |  | Date: |  |  | 2023-05-11 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | It can be possible to reach an EAS from several DNAIs. See example below: As shown, the EAS can be reached from both DNAI1 and DNAI2, but DNAI2 has 1 more router hop. I.e. DNAI1 would be the first choice and DNAI2 a second choice for an edge application. Since the use of DNAI1 is not guaranteed, it would be good to give 2 (or more) choices of DNAIs to an AF. But for the AF to choose, some sort of distance or latency associated with the DNAI to IP range could be provided to the AF. The proposal is to give expected latency between the DNAI and IP range. Since latency in a routed and or swithed network is not constant but can vary due to network load, re-routing due linkfailures etc, the word "expected" is used. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding a latency parameter |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Reduced information for the AF to select a desired DNAI |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.6.34.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.548, CR 0114 |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 5.2.6.34.4	Nnef_DNAIMapping_UpdateNotify service operation

Service operation name: Nnef_DNAIMapping_UpdateNotify

Description: This service operation is used by the NEF to notify NF consumer about the update of DNAI - EAS address(es) mapping information.

Inputs, Required: One or more pairs of DNAI(s) and EAS address(es).

Inputs, Optional: Cause, Expected Latency between pairs of  DNAI(s) and EAS address(es) (lower is better).

NOTE:	The Expected Latency is defined by the operator and can be either a quantitative value or relative value (against other latencies). The margin of errors is according to operator configuration.

Outputs, Required: None.

Outputs, Optional: None.