3GPP TSG SA WG-2 Meeting #157 	S2-2306692

Berlin, Germany, May 22 – 26, 2023

# Source:	Qualcomm Incorporated

# Title:	New SID on Lightweight Access Aggregation and Steering of Wi-Fi in 5GC

# Document for:	Approval

# Agenda Item:	10.5

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: Study on Lightweight Access Aggregation and Steering of Wi-Fi in 5GC

Acronym:	FS_ATSSSLite

Unique identifier:	TBD

Potential target Release:	Rel-19

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  | X |  |
| No | X |  | X |  | X |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

| X | Study |
| --- | --- |
|  | Normative – Stage 1 |
|  | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

## 2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
|  |  |  | N/A |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

# 3	Justification

The Rel-16 to Rel-18 Access Traffic Steering, Switch and Splitting feature enables simultaneous communication between UE and UPF over multiple paths. By leveraging the simultaneous communication over multiple paths, the 5G system can provide services with improved user experience, can distribute the traffic across multiple accesses in a policy-based fashion, can provide new high-data-rate services, etc.

However currently the Access Traffic Steering, Switch and Splitting feature requires that MA PDU Sessions require integrated trusted or untrusted non-3GPP accesses. This means that to enable ATSSS either a TNGF (Trusted Non-3GPP Gateway Function) or an N3IWF (Non-3GPP InterWorking Function) is deployed in the PLMN. At the same time, many network deployments do not have such nodes and it is therefore beneficial to study how to support a limited set of access traffic aggregation and steering features applicable to non-integrated Wi-Fi networks (which do not require the deployment of TNGFs/N3IWFs).

# 4	Objective

Based on the above justification, the objective of this study item is to study how to enhance the 5G system architecture to address the following aspects:

1)	Define a new architecture for Lightweight Access Aggregation and Steering of non-integrated Wi-Fi in 5GC not based on TNGF/N3IWF.

2)	Introduce the concept of Non-Integrated Aggregated (NIA) PDU Session based on the re-use of IETF RFC protocols (e.g., MPQUIC/MPTCP) and related functions in the UE and in the PSA UPF.

A)	How to establish and modify NIA PDU Sessions

B)	How to extend the usage of rules for UE and UPF to coordinate the aggregation and steering of traffic within a NIA PDU Session, including how to configure the MPTCP/MPQUIC proxy functionality in the UPF

NOTE:	It is assumed that the Lightweight Access Aggregation and Steering functionality applies to non-GBR traffic type only.

## TU estimates and dependencies

| Work Task ID | TU Estimate (Study) | TU Estimate (Normative) | RAN Dependency (Yes/No/Maybe) | Inter Work Tasks Dependency |
| --- | --- | --- | --- | --- |
| WT#1 |  |  |  |  |
| WT#2 |  |  |  |  |
| WT#3 |  |  |  |  |

Total TU estimates for the study phase:

Total TU estimates for the normative phase:

Total TU estimates: N + M =

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| Internal TR | 23.xyz | Study on upper layer traffic steering, switching and split over dual 3GPP access | TSG#XX Dec. 2024 | TSG#XX Mar. 2024 | TBD |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |

# 6	Work item Rapporteur(s)

TBD

# 7	Work item leadership

SA2

# 8	Aspects that involve other WGs

The following aspects involving other WGs may arise related to this SID:

-

# 9	Supporting Individual Members

Supporting IM name

Qualcomm Incorporated