3GPP TSG-SA2 Meeting #157	S2-2306763

Berlin, Germany, 22nd May 2023 - 26th May 2023	(revision of S2-2306041)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 3825 | rev | 5 | Current version: | 18.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of Alternative S-NSSAI replacement determined by NSSF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | [Lenovo, Nokia, Nokia Shanghai Bell, Ericsson], LG Electronics, Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS_Ph3 |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As part of the FS_eNS_Ph3 study, the TR 23.700-41 V2.0.0 has concluded in clause 8.1 for KI#1 that in one option the Alternative S-NSSAI can be determined by the NSSF. The corresponding CR to 23.501 has been approved in SP-230315. This CR attempts to implement the changes needed for the NSSF service operations. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to specify the enhancements to the NSSF service operations resulting from agreed 23.501. In Rev5, - Updates Nnssf_NSSAIAvailability_Subscribe service to cover both Network Slice Replacement and Network Slice Instance Replacement. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The TS 23.502 will not be aligned to TS 23.501. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.16.3.1, 5.2.16.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

#### 5.2.16.3	Nnssf_NSSAIAvailability service

##### 5.2.16.3.1	General

Service description: This service enables to update the AMFs and the NSSF on the availability of S-NSSAIs and NSAGs on a per TA basis. This service also provides an Network Slice Replacement and Network Slice Instance Replacement notification to NF Service Consumer in case NSSF determines that and S-NSSAI and/or Network Slice instance has to be replaced.

NOTE:	The NSSF can determine the serving network and Access Type from the TAI, as described in TS 29.571 [70].

Following NSSF event subscriptions are supported:

-	NSSAI availability status change;

-	Network Slice Replacement (see clause 5.15.19 in TS 23.501 [2]);

-	Network Slice instance Replacement (see clause 5.15.19 in TS 23.501 [2]).

## Next CHANGE

5.2.16.3.4	Nnssf_NSSAIAvailability_Subscribe service operation

Service operation name: Nnssf_NSSAIAvailability_Subscribe

Description: This service operation enables a NF Service Consumer (e.g. AMF) to subscribe to a notification of any changes in status of the NSSAI availability information (e.g. S-NSSAIs and optionally NSI IDs available per TA and the restricted S-NSSAI(s) and optionally NSI IDs per PLMN in that TA in the serving PLMN of the UE) upon this is updated by another AMF. This service also enables the NF Service Consumer (e.g. AMF or NSSF in the VPLMN) to subscribe to a notification of Network Slice Replacement and Network Slice Instance Replacement.

Inputs, Required: Callback URI of the NF Service Consumer.

Inputs, Conditional Required:

If this service operation is invoked to subscribe for changes in status of the NSSAI availability information, then the following inputs are required:

-	list of TAIs supported by the NF Service Consumer, event to be subscribed.

If this service operation is invoked to subscribe to a notification when any S-NSSAI has to be replaced by an Alternative S-NSSAI, then the following inputs are required:

-	for the S-NSSAI of the serving PLMN: set of S-NSSAIs served by the NF Service Consumer that may be replaced, NF type of the NF Service Consumer (e.g. AMF), Requester ID, event to be subscribed.

-	for HPLMN S-NSSAI: S-NSSAIs for the HPLMN, NF type of the NF Service Consumer (e.g. AMF or NSSF), Requester ID and PLMN ID, event to be subscribed.

If this service operation is invoked to subscribe to a notification when the Network Slice instance is congested or no longer available, then the following inputs are required:

-	The S-NSSAIs and NSI IDs for which the status is changed (congested or no longer available) , event to be subscribed.

Inputs, Optional: Expiry time.

Outputs, Required: Subscription ID.

Outputs, Conditional Required:

If this service operation is invoked to subscribe for changes in status of the NSSAI availability information, then the following outputs are required:

-	Expiry time (if present in the request, may be included in the response based on operator's policy and taking into account the expiry time present in the request (i.e. should be less than or equal to that value); if not present in the request, may be included in the response based on operator's policy. Whatever the case, if not included in the response, this means that the subscription is valid without an expiry time).

Outputs, Optional:

If this service operation is invoked to subscribe for changes in status of the NSSAI availability information, then the following outputs are required:

- A list of TAIs and, for each TAI, the S-NSSAIs and optionally NSI IDs supported by the AMF and 5G-AN and authorized by the NSSF for the TAI and a list of S-NSSAIs and optionally NSI IDs restricted per PLMN for the TAI.

If this service operation is invoked to subscribe to a notification when an Alternative S-NSSAI has to replace an S-NSSAI, then the following outputs are required:

-	for the S-NSSAI of the serving PLMN: an Alternative S-NSSAI and the corresponding mapping to the S-NSSAI to be replaced.

-	for HPLMN S-NSSAI: an Alternative S-NSSAI and the corresponding mapping to the S-NSSAIs of the HPLMN, PLMN ID.

If this service operation is invoked to subscribe to a notification when the Network Slice instance is congested or no longer available, then the following outputs are required:

-	The S-NSSAIs and NSI IDs.

* * * END of CHANGES * * *