3GPP TSG-WG SA2 Meeting #157 	S2-2306775

Berlin, Germany, May 22 – 26, 2023	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	Update on UE-only Operation for SL Positioning using Located UE

Document for:	Approval

Agenda Item:	9.5.2

Work Item / Release:	Ranging_SL / Rel-18

Abstract: This paper proposes to solve the EN in UE-only Operation for SL Positioning using Located UE.

1. Introduction/Discussion

This paper proposes to remove the following EN and add the list of principles in UE-only Operation for SL Positioning using Located UEs.

Editor's note:	The list of principles needs to be added.

2. Text Proposal

It is proposed to capture the following changes vs. TS 23.586.

# * * * * First change * * * *

## 5.11	UE-only Operation for SL Positioning using Located UE

When LMF is not involved for SL Positioning/Ranging, e.g. the LMF in the serving network does not support SL Positioning, UE-only Operation SL Positioning is used, including Target UE as SL Positioning Server UE and Target UE not as SL Positioning Server UE.

When Target UE as SL Positioning Server UE, the following principles applies:

-  The Target UE performs the Located UE’s discovery and selection.

-  The Target UE obtains the location of the Located UE(s), and the Located UE(s) may trigger a 5GC-MO-LR to retrieve its location. The Target UE uses the location of Located UE(s) together with the ranging/SL positioning measurement data or result to estimate its own location.

-  The Ranging/Sidelink positioning and the positioning of the Located UE(s) can be scheduled with the same scheduled location time (as per TS 23.273 [8]) to improve the Target UE positioning accuracy.

NOTE:	Security and privacy aspects require confirmation from SA WG3.

When Target UE not as SL Positioning Server UE, the following principles applies:

-  The Target UE performs the SL Positioning Server UE's discovery and selection.

-  The SL Positioning Server UE performs the result calculation, Ranging/SL Position method determination, assistant data distribution and SL Reference UE selection.

-  The SL Positioning Server UE uses the location of Located UE(s) together with the ranging/SL positioning measurement data or result to estimate the location of Target UE. The Ranging/Sidelink positioning and the positioning of the Located UE(s) can be scheduled with the same scheduled location time (as per TS 23.273 [8]) to improve the Target UE positioning accuracy.

# * * * * End of changes * * * *