3GPP TSG-WG SA2 Meeting #157 	S2-2306779

Berlin, Germany, May 22 – 26, 2023	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	Update on Network based SL Positioning for UE without NAS connection

Document for:	Approval

Agenda Item:	9.5.2

Work Item / Release:	Ranging_SL / Rel-18

Abstract: This paper proposes to update Network based SL Positioning for UE without NAS connection.

1. Introduction/Discussion

This paper proposes some updates on Network based SL Positioning for UE without NAS connection and remove two ENs.

1. Remove the EN on enhanced 5GC-MO-LR message, and clarify the Located UE initiate the 5GC-MO-LR procedure with a new type of location service (Network based SL positioning).

Editor's note:	It is FFS if an enhanced 5GC-MO-LR message is sent by the Located UE.

2. Add a NOTE to clarify the functionality of routing identifier is same with the one used in Deferred 5GC-MT-LR Procedure (clause 6.3.1 of TS 23.273). And the routing identifier can be configured in Ranging/SL Positioning policy. Each Located UE gets the routing identifier from the Target UE.

3. To clarify that LMF may interact with GMLC to get the location of Located UE.

4. It was agreed that 5GC-MT-LR is not applicable in the case Network based SL positioning for UE without NAS connection. So the following EN can be removed.

Editor's note:	It is FFS when AMF does not maintain target UE's context or GMLC cannot get target UE AMF address because of deregistration, whether and how location report from Located UE on behalf of Target UE is supported.

5. To clarify that LMF provide the resulting location via one of the Located UE(s) to the Target UE, not via all Located UE(s).

2. Text Proposal

It is proposed to capture the following changes vs. TS 23.586.

# * * * * First change * * * *

### 5.5.3	Network based SL positioning for UE without NAS connection

When Target UE cannot establish the NAS connection with AMF due to the Target UE being out of coverage and at least one of the discovered Located UEs can establish NAS connection for 5GC-MO-LR, the following principles are applied:

NOTE 1:	5GC-MT-LR is not applicable in this release of the specification.

-	The Target UE performs the Located UE's discovery and selection.

-	The Target UE and Located UE(s) perform ranging/SL positioning.

-	The Target UE selects at least one of the Located UEs that can use 5GC-MO-LR service for SL Positioning.

-	The Target UE may transmit its ranging measurements/estimation results to the Located UE(s).

-	A routing identifier for Target UE positioning can be configured in Ranging/SL Positioning policy. Each Located UE gets the routing identifier from the Target UE.

NOTE 2:	The routing identifier indicates an LMF that manages the Target UE positioning, and also used to assist the LMF in identifying the Target UE positioning, as defined in clause 6.3.1 of TS 23.273 [8].

-	The Located UE initiates the 5GC-MO-LR procedure with using Target UE Positioning assisted by Sidelink Positioning as the type of location service, which requests the location of Target UE by using Sidelink positioning.

-	Each Located UE that can establish the NAS connection may report the ranging/SL positioning measurement or estimation result and routing identifier to the LMF via the serving AMF of the Located UE. This may include ranging measurements/results received from the Target UE. The endpoints for LPP messages are the LMF and the Located UE(s).

-	In case the location of Located UE is unknown, LMF interacts with GMLC a required QoS to get the location of Located UE.

-	The LMF may use the received information (i.e., ranging/SL positioning measurement or estimation result with the same routing identifier) and the location of Located UE(s) to calculate the location of the Target UE, and provide the resulting location via one of the Located UE(s) to the Target UE.

-	If a LMF is aware of multiple Located UEs associated with the same Target UE based on the same routing identifier, it may perform the calculation using all information from these Located UEs.

Editor's note:	Security aspects require confirmation from SA WG3.

# * * * * End of changes * * * *