3GPP TSG-WG SA2 Meeting #157	S2-2306794

# Berlin, Germany, May 22 – 26, 2023		(revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4618 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Handling of PDU session for OAM system access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | VMR |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | It is proposed to update clause 5.35A.4 with the following changes: - Clarify that the AMF and SMF are configured to know whether a PDU session is for OAM system access based on DNN/S-NSSAI. - Clarify that AMF of MBSR provides MBSR authorization indication to the SMF during PDU session establishment for OAM access. - Clarify that if the MBSR authorization indication indicates the UE is not authorized for MBSR operation, the SMF shall reject the request for PDU Session Establishment for OAM system access. - Clarify that when the MBSR operation is changed from “authorized” to “not authorized” and the MBSR is still registered with existing PDU session for OAM system access, the AMF updates the SMF that MBSR is not authorized. The SMF shall release this PDU Session. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Handling of PDU session for OAM system access |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The handling of PDU session for OAM system access is not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.35A.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS23.502, CR4209 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.35A.4	MBSR authorization

For a MBSR, the subscription information stored in the HPLMN indicates whether it is authorized to operate as MBSR, and the corresponding location and time periods.

Editor's note:	The subscription information for the MBSR will be further specified.

When MBSR roaming is supported, a roaming agreement between VPLMN and HPLMN regarding MBSR operation is in place, and the 5GC can make use of it for authorization of MBSR in VPLMN. MBSR (IAB-DU) can use IAB-node integration procedure or inter-IAB-donor gNB mobility procedure to integrate into VPLMN to provide service.

The MBSR(IAB-UE) is assumed to be configured with preferred PLMN lists and forbidden PLMNs by the HPLMN for the MBSR operation.

Editor's note:	Configuration mechanism will be further described based on development of clause 5.35A.2.1.

When the MBSR (IAB-UE) performs initial registration with the serving PLMN, it indicates the request to operate as a MBSR as described in clause 5.35A.1. The AMF authorizes the MBSR based on the subscription information, and provides MBSR authorized indication to NG-RAN. The MBSR establishes the connection to OAM system using the configuration information for MBSR operation. The AMF based on DNN/S-NSSAI knows whether a PDU session is for OAM system access.

Editor's note:	It is FFS whether the MBSR indication is a new IE or part of UE 5G MM capabilities.

Editor's note:	Whether existing IAB-Operation allowed indication and IAB authorized indication can be reused for MBSR will be determined.

NOTE 1:	How the MBSR obtains the configuration information for MBSR operation is described in clause 5.35A.2.1.

The AMF of the MBSR can indicate to the MBSR IAB-UE that it is not allowed to act as an MBSR IAB node as part of registration procedure, and in this case the AMF does not include MBSR authorization indication to donor-gNB. The AMF may provide the indication either in a Registration Accept (if the PLMN allows the MBSR IAB-UE to be registered in the PLMM) or in a Registration Reject (if the PLMN does not allow the MBSR IAB-UE to be registered in the PLMN).

When receiving a request for PDU Session Establishment with the DNN/S-NSSAI for OAM system access, the AMF determines MBSR authorization state and forwards to the SMF. The SMF shall reject the request for PDU Session Establishment for OAM system access if the MBSR authorization state indicates the UE is not authorized for MBSR operation.

When the MBSR operation is changed from "authorized" to "not authorized" and the MBSR is still registered with existing PDU session for OAM system access, the AMF notifies the SMF that MBSR is not authorized to trigger PDU Session release.

Editor's note:	In what conditions and how (e.g. UCU or deregistration with re-registration) an MBSR is informed when there is a change in the MBSR authorization information is FFS.

NOTE 2:	The mechanism applies to both roaming and non-roaming MBSR operations.

# * * * * End of changes * * * *