3GPP TSG-SA WG6 Meeting #157	S2-2306801

Berlin, May 22 – 26, 2023	(revision of S2-230xxxx)

Title:	[DRAFT] Reply LS on MBS service area update failure clarification

Response to:	LS (S6-231606) on MBS service area update failure clarification

Release:	Release 17

Work Item:	5MBS, MCOver5MBS

Source:	[Huawei to be] SA2

To:	SA6

Cc:	CT3, SA4

Contact Person:

Name:	Meng Li

Tel. Number:

E-mail Address:	raymond DOT limeng AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	CR0270 and CR0271 of TS 23.247

1. Overall Description:

3GPP SA2 thanks SA6 for their LS on MBS service area update failure clarification S2-2306330/S6-231606.

SA2 provide the following answers to SA6 questions:

Q1. SA6 would like to ask SA2 whether the AF/NEF will interact with the NRF to re-select the capable MB-SMF during the MBS session update with requested MBS service area?

SA2 answer: No. During the MBS session update with requested MBS service area, the NEF/MBSF will forward the MBS session update request to the serving MB-SMF corresponding to the MBS session ID, without interacting the NRF.

Q3. SA6 would like to ask SA2 to confirm whether the scenarios described above from SA6, CT4 and CT3 can occur during MBS session update. If yes, what are the mechanism(s) defined by SA2 to be employed to overcome an MBS session update failure?

SA2 answer: During the MBS session update with MBS service area, the requested MBS service area is forwarded to the MB-SMF. If the requested MBS service area cannot be covered by the serving MB-SMF of the current MBS session, a proper error information should be provided to the NEF/MBSF/AF. The AF may create a new MBS session with the requested MBS service area, or use unicast PDU session based the error information.

SA2 discuss the potential issue and agreed on a solution in the attached CRs.

2. Actions:

To SA6, CT3, CT4 group.

ACTION: 	SA2 kindly asks SA6, CT3, CT4 group to take the above response into account.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #158		August 21 – 25, 2023			Goteborg, SE

TSG-SA2 Meeting #159            	August 9  – 13, 2023 			Xiamen, CN