3GPP TSG SA WG2 Meeting #157	S2-2306822

# May 22 - 26, 2023, Berlin, Germany											(Revision of S2-2304973)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1306 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support IMS data channel in roaming case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC |  |  |  |  |  | Date: |  |  | 2023-04-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1. There is an editor’s note on support IMS data channel in roaming case: Editor's note: It is FFS whether and how IMS Data Channel is supported in roaming cases. The roaming architecture of IMS data channel has no additional requirement to enhance existing IMS roaming architectures. When home routed roaming architecture is used, all the IMS and DC related NFs and network functional entities are located in HPLMN. When local breakout roaming architecture is used, P-CSCF is located in VPLMN. Supporting IMS data channel requires P-CSCF to support SDP of IMS data channel and provides the corresponding data channel media resource managements. Therefore during the IMS DC capability negotiation, especially in roaming case, the home IMS network needs to know if the P-CSCF supports IMS DC to decide whether IMS data channel can be provided in the registration period. It is proposed to: - clarify that existing IMS roaming architectures apply to IMS roaming architecture of supporting data channel; - update IMS DC capability negotiation procedure to allow P-CSCF indicating its IMS DC capability during registration procedure. 2. The enhancements to S-CSCF and P-CSCF is missing in current specification. This contribution adds the descriptions of enhancements to S-CSCF and P-CSCF. 3. The procedure of IMS DC capability negotiation was put in a separate caluse (AC.8) against the other procedures (in AC.7). This contribution moves the procedure of IMS DC capability negotiation into clause AC.7. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - clarify that existing IMS roaming architectures apply to IMS roaming architecture of supporting data channel; - update IMS DC capability negotiation procedure to allow P-CSCF indicating its IMS DC capability during registration procedure. - adds the descriptions of enhancements to S-CSCF and P-CSCF. - moves the procedure of IMS DC capability negotiation into clause AC.7. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AC.1, AC.2.1, AC.2.1a (new), AC.2.2.5 (new), AC.2.2.6 (new), AC.7.X (new), AC.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

Annex AC (normative):
Support of Data Channel services in IMS

# AC.1	General

# This annex describes IMS architecture enhancements to support data channel services both for non-roaming and roaming scenarios.

# Data channels are always established in the context of an IMS MMTel session.

# Service based interfaces applicable to data channel services are specified in AA.2.3

The architecture in this annex supports separation of signalling function and media function supporting data channel services. A network function for data channel management signalling, Data Channel Signalling Function (DCSF), is specified. The network function to handle data channel media can be provided in two ways:

-	by enhancing the existing IMS MRF (called MRF) to perform media functions related to DC, which interacts with the IMS AS via enhanced Mr'/Cr interfaces; and

-	by introducing a new network function called Data Channel Media Function (DCMF), which interacts with the IMS AS via the service-based interface DC2.

NOTE:	In this release of the specification the IMS AS supporting data channel services is collocated with the TAS.

# AC.2	Architecture and functions

## AC.2.1	Non-Roaming Architecture

# Figure AC.2.1-1 shows the data channel architecture when using the service-based DC media function.

# Figure AC.2.1-1: Architecture option of IMS supporting DC usage with DCMF

# Figure AC.2.1-2 shows the data channel architecture when using the MRF.

# Figure AC.2.1-2: Architecture option of IMS supporting DC usage with MRF

# NOTE 1:	MDC2 may connect different IMS security domains.

NOTE 2:	DC5 reference point is not specified in 3GPP.

NOTE 3:	MDC1, MDC2 and MDC3 reference points are not specified in 3GPP.

NOTE 4:	DC3 and DC4 reference points are not specified in this Release.

NOTE 5:	Enhancements to the N33 reference point for support of data channel services are not specified in this Release.

Editor's note:	Enhancements to other interfaces (e.g. N71/Sh, N70/Cx, Gm, Mw, ISC, Iq, Mb) for support of IMS data channel service are FFS.

AC.2.1a	Roaming Architecture

The IMS roaming architectures specified in clause 4.2.3, 4.15a and 4.15b apply to the roaming architecture of IMS supporting data channel.

In local breakout roaming architecture, the P-CSCF is in VPLMN, while in home routed architecture, all IMS functional entities or NFs are in HPLMN.

**** Second Change ****

AC.2.2.5			S-CSCF

# The S-CSCF is enhanced to support the following functionalities:

-	The S-CSCF indicates the capability of IMS home network supporting IMS data channel during IMS registration procedures.

AC.2.2.6			P-CSCF

# The P-CSCF is enhanced to support the following functionalities:

-	The P-CSCF indicates the capability supporting IMS data channel of its serving network during IMS registration procedures.

-	The P-CSCF supports SDP of IMS data channel and provides the corresponding data channel media resource managements via Iq and Policy control via Rx/N5.

**** Third Change ****

AC.7.X	IMS DC capability negotiation

The IMS network and the UE need to negotiate the capability of supporting IMS data channel mutually.

IMS data channel capability negotiation includes two aspects:

-	The network discovers the data channel capability of the UE.

-	The UE discovers the data channel capability of the network.

When the UE supporting IMS data channel registers on the IMS network, it includes the media feature tag as specified in TS 26.114 [76] in the Contact header field of the initial REGISTER request to allow the home IMS network discovers its IMS data channel capability.

If the serving IMS network supports IMS data channel, the P-CSCF includes a Feature-Caps header field indicating its data channel capability in the initial REGISTER request to allow the home IMS network discover its IMS data channel capability.

If the home IMS network, UE and P-CSCF all supports IMS data channel, the S-CSCF includes a Feature-Caps header field indicating its data channel capability in the 200 OK response to the initial and any subsequent REGISTER request, which is used by the UE to discover the IMS data channel capability of its home IMS network.

NOTE:	The UE can receive a Feature-Caps header field indicating its data channel capability in the 200 OK response to a subsequent REGISTER request when the network starts supporting IMS data channel after successful initial registration of the UE.

When the UE supporting IMS data channel initiates an IMS session, it includes the media feature tag as specified in TS 26.114 [76] in the Contact header field of the initial INVITE or a re-INVITE request to remote UE, regardless of data channel media being part of the SDP or not.

The UE shall not include data channel media description in the SDP offer of the initial INVITE request or any subsequent re-INVITE, if the S-CSCF has not included the data channel capability indication in the Feature-Caps header field in the 200 OK response either to the REGISTER or subsequent REGISTER request.

**** Forth Change ****

# AC.8	Void

**** End of Changes ****