3GPP TSG-SA2 Meeting #157 	S2-2306843

Berlin, Germany, 22 – 26 May 2023

# Source:	NTT DOCOMO

# Title:	New WID on Study on Home gNB with Local GW

# Document for:	Approval

# Agenda Item:	10.5

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:           Study on Home gNB with Local GW

Acronym:	FS_HGNB

Unique identifier:	TBD

Potential target Release:	Rel-19

1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No | X | X |  |  |  |
| Don't know |  |  | X |  |  |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a …

| X | Study |
| --- | --- |
|  | Normative – Stage 1 |
|  | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| N/A |  |  |  |

2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| N/A |  |  |

3	Justification

Insufficient use of mmWave is problematic. This is a resource our industry promised to use effectively and efficiently; a tool to improve this situation is required.

There is an opportunity where mmWave serves well; high throughput is getting required at home and at campus to enable new immersive XR applications. Operators are trying to extend use of mmWave. If there is a tool that complements operator's effort by allowing customers to boost capacity at their premises using mmWave by themselves, that would further promote the use of mmWave. Overall customer's service experience, then, is improved.

It is already possible for operator to deploy NR femto for the purpose of coverage improvement. A slight enhancement of NR femto is required so that customers are willing to put it at their premises. Such an enhanced NR femto needs to start working in a plug-and-play manner. The owner needs to be able to control who can use cells of the enhanced NR femto.

A femto (LTE femto) has sometimes caused a problem in traffic forwarding to the central network. That has often led to change of the wireline connection to the customer's home. Having a local GW next to NR femto can avoid this problem. For sure, avoiding concentration of traffic to the central site is always preferred.

NOTE:	While mmWave is often mentioned in the description above, the below objective and outcome of this study is not dependent on the choice of frequency band.

4	Objective

The following aspects will be studied:

WT#1: Study how to enable SMF to select Local GW collocated to Home gNB.

NOTE 1: Home gNB is a gNB equipped with CAG. Local GW is a UPF in this release of specification.

WT#2: Study how to enable an administrator to give a temporal Allowed CAG setting to UEs that the administrator selects, so that the UEs can access the cells of Home gNB.

NOTE 2: An administrator can be an owner of Home gNB and its collocated Local GW.

TU estimates and dependencies

| Work Task ID | TU Estimate (Study) | TU Estimate (Normative) | RAN Dependency (Yes/No/Maybe) | Inter Work Tasks Dependency Editor’s Note: This column should highlight if WT#x is self-contained, or it depends on the completion of other WTs |
| --- | --- | --- | --- | --- |
| WT#1 | 0.25 | 0.25 | Maybe | self-contained |
| WT#2 | 0.5 | 0.5 | No | self-contained |

Total TU estimates for the study phase: 0.75

Total TU estimates for the normative phase: 0.75

Total TU estimates: 0.75 + 0.75 = 1.5

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| Internal TR | 23.XXX | Study on Home gNB with Local GW | TSG#xx | TSG#xx | TBD |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |

6	Work item Rapporteur(s)

TBD

7	Work item leadership

SA2

8	Aspects that involve other WGs

RAN impacts covered by RAN WGs (RAN3).

9	Supporting Individual Members

Supporting IM name

NTT DOCOMO