3GPP TSG-WG SA2 Meeting #157	S2-2306861

# Berlin, Germany, May 22 – 26, 2023					                (revision of S2-2305634)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.316 | CR | 2087 | rev | 3 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New feature for 5G-RG to support NSWO procedure to authorize UE behind RG |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | [China Telecom], [Nokia], [Nokia Shanghai Bell], Huawei, Hisilicon, CableLabs |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5WWC_Ph2 |  |  |  |  |  | Date: |  |  | 2023-04-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For 23.700-17 has concluded that “in order to support NSWO (as defined in clause 5.42 of TS 23.501) for 3GPP UE, 5G-RG shall support the SWa' interface to NSWOF but this aspect is in the scope of BBF and/or Cable Labs specifications.” Besides, in TS 23.501 clause 4.2.15, UE is allowed to use 5G credentials based on NSWO to authenticate to the 5GC to first obtain local IP address and then to use untrusted Non-3GPP access to register to the 5GC. Similarly, when UE is behind RG, the UE should also be able to use NSWO to first authenticate to 5GC. So propose a feature for 5G-RG and FN-RG to support NSWO procedure to authorize UE behind RG. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Propose a feature for 5G-RG and FN-RG to support NSWO procedure to authorize UE behind RG. The cluse 4.10.d.1 specifies the NSWO supports for 5G-RG and the clause 4.10d.2 for FN-RG. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 5G-RG and FN-RG do not support 5G NSWO procedure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.10d (new), |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X |  |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | The rev 2 includes the support of NSWO for FN-RG per discusison and request during the meeting SA2#156e. |  |  |  |  |  |  |  |  |

# * * * * First change has been removed from CR since no changes * * * *

# * * * * Second change

# Note: the changes in yellow are related to 5G-RG

# Note: The changes in blue are related to the addition of FN-RG* * * *

## 4.10d	Support of NSWO for 3GPP UE behind a RG

In this clause the support of NSWO for 3GPP UE behind a 5G-RG and FN-RG are specified

### 4.10d.1	Support of NSWO for 3GPP UE behind a 5G-RG

In order to support NSWO defined in clause 5.42 of TS 23.501 [2] for 3GPP UE connected to a 5G-RG, the 5G-RG shall support the WLAN Access functionality defined in TS 23.501 [2], clause 4.2.15 and clause 5.42. The WLAN Access functionality includes the support of the SWa' interface to NSWOF as shown in Figure 4.10d.1-1 and Figure 4.10d.1-2.

The user plane packets of the UE  may not be transported via the 5G-RGs PDU Session and  may  also not be traversing the UE’s 5GC.In this case, the user plane for NSWO may be terminated on a BNG or on a W-AGF where NWSO traffic is offloaded without traversing the 5G network. The W-AGF should determine that the traffic from a 5G-RG not part of a PDU session shall be offloaded and not transmitted over N3 reference point and the traffic not received over N3 reference point shall be sent to the destination 5G-RG as offloaded traffic.

A 5G-RG shall be registered to 5GC to serve an UE: the 5G-RG shall not issue a DIAMETER authentication request over SWa’ for the UE if it is itself not registered to 5GC.

As specified in clause 4.2.15 of TS 23.501 [2], the SWa’ reference point corresponds to SWa reference point defined in TS 23.402 [X] but where the EAP user ID is a SUCI of the UE and not an IMSI.

The SWa’ support in Wireline access network has no impact on 3GPP specifications.

NOTE: 	The specification of support of functionalities for NSWO in the wireline access network is under BBF and CableLabs responsability. This includes potential support of AAA proxy functionality as shown in clause 4.2.15 of TS 23.501 [2] allowing to carry AAA flows between the 5G-RG and the NSWOF over the network access of the 5G-RG.

Figure 4.10d.1-1: Reference architecture to support authentication for Non-seamless WLAN offload in 5GS

Figure 4.10d.1-2: Service based reference architecture to support authentication for Non-seamless WLAN offload in 5GS

NOTE: The specification in this clause may be further revised based on feedback from BBF and CableLabs.

### 4.10d.2	Support of NSWO for 3GPP UE behind a FN-RG

In order to support NSWO defined in clause 5.42 of TS 23.501 [2] for 3GPP UE connected to a FN-RG,  the FN-RG shall support the WLAN Access functionality defined in TS 23.501 [2], clause 4.2.15 and clause 5.42.

The WLAN Access functionality includes support of the SWa' interface to NSWOF as shown in Figure 4.10d.2-1. FN-RG without the WLAN access functionality may not support 5G NSWO.

NOTE 1: 	The SWa’ specified in TS 29.273 is DIAMETER protocol with 3GPP specific extension to enable to perform authentication and authorization with 5GC.

NOTE 2: 	Whether and how the DIAMETER protocol is carried over Y4 or other mechanism are specified is out of the 3GPP scope.

The user plane packets of the UE may not be transported via the PDU Session from W-AGF acting on behalf of FN-RG and  also may not be traversing the UE’s 5GC. In this case, the user plane for NSWO may be terminated on a BNG or on a W-AGF where NWSO traffic is offloaded without traversing the 5G network. The W-AGF should determine that the traffic from a FN-RG not part of a PDU session shall be offloaded and not transmitted over N3 reference point and the traffic not received over N3 reference point shall be sent to the destination FN-RG as offloaded traffic.

A FN-RG shall be registered to 5GC to serve a UE: the FN-RG or the W-AGF shall not issue a DIAMETER authentication request over SWa’ for the UE if the FN-RG from where the UE is connecting is not registered to 5GC.

As specified in clause 4.2.15 of TS 23.501 [2], the SWa’ reference point corresponds to SWa reference point defined in TS 23.402 [X] but where the EAP user ID is a SUCI of the UE and not an IMSI.

The SWa’ support in Wireline access network has no impact on 3GPP specifications.

NOTE 3: 	The specification of support of functionalities for NSWO in the wireline access network is under BBF and CableLabs responsibility. This includes potential support of AAA proxy functionality as shown in clause 4.2.15 of TS 23.501 [2] allowing to carry AAA flows between the FN-RG and the NSWOF over the network access of the FN-RG.

Figure 4.10d.2-1: Reference architecture to support authentication for Non-seamless WLAN offload in 5GS

Figure 4.10d.2-2: Service based reference architecture to support authentication for Non-seamless WLAN offload in 5GS

Editor’s note: The specification in this clause may be further revised based on feedback from BBF and CableLabs.

# * * * * End of changes * * * *