3GPP TSG-WG SA2 Meeting #157	S2-2306905

# Berlin, Germany, May 22 – 26, 2023		(revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0831 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Procedures for network data analytics in roaming |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1. The Target of Analytics Reporting may include roaming user(s) and non-roaming user(s). Since V-NWDAF will subscribe to H-NWDAF only for inbound roaming user(s), so the V-NWDAF should determine the Target of Analytics Reporting sent by the V-NWDAF to the H-NWDAF which includes only inbound roaming user(s), and vice versa. 2. It is proposed that there is no support for analytics aggregation of roaming scenario in Rel-18, since no conclusion on analytics aggregation has been reached. Meanwhile, the NWDAF should not notify two different analytics (i.e., the HPLMN analytics and VPLMN analytics) in step 6 when the consumer only subscribes once in step 1, since the consumer may not understand it. Therefore, if the requested analytics needs to be collectively served by V-NWDAF and H-NWDAF, the V-NWDAF could derive the requested analytics based on V-NWDAF implementation, and vice versa. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Add descriptions that the NWDAF determines the Target of Analytics Reporting sent to the NWDAF in another PLMN. 2. Remove the EN under step 6, and update the procedures for analytics generation in roaming case. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The mechanisms of analytics exposure in roaming case are unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.5.2, 6.1.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.1.5.2	Analytics Exposure from HPLMN to VPLMN

Figure 6.1.5.2-1 shows the procedure that the V-NWDAF requests network analytics (i.e. slice load level analytics, NF load analytics, service experience analytics, etc.) in HPLMN from the H-NWDAF with roaming entry capability, upon receiving an analytics information/subscription request from the NF in VPLMN (e.g. AMF).

Editor's note:	The service names in the following procedure are descriptive and may need to be updated.

Figure 6.1.5.2-1: Procedure for analytics exposure from HPLMN to VPLMN

1.	Consumer NF in VPLMN (e.g. AMF) sends an Analytics request/subscribe (Analytics ID, Target of Analytics Reporting, Analytics Filter Information) to V-NWDAF by invoking a Nnwdaf_AnalyticsInfo_Request or a Nnwdaf_AnalyticsSubscription_Subscribe.

NOTE 1:	The Target of Analytics Reporting sent by the consumer NF to the V-NWDAF may include both inbound roaming user(s) and non-roaming user(s).

2.	V-NWDAF discovers the H-NWDAF with roaming entry capability as described in clause 5.2. V-NWDAF sends a roaming analytics request/subscribe (Analytics ID, Target of Analytics Reporting, Analytics Filter Information) to H-NWDAF by invoking a Nnwdaf_RoamingAnalytics_Request or a Nnwdaf_RoamingAnalytics_Subscribe, based on the Analytics request/subscribe received from the Consumer NF in VPLMN. The Target of Analytics Reporting sent by the V-NWDAF to the H-NWDAF shall only contain the inbound roaming user(s). The inbound roaming user(s) can be distinguished by the V-NWDAF according to the PLMN ID of the UE IDs. If V-NWDAF has the roaming entry capability, it checks the policies and regulatory constraints between the HPLMN and the VPLMN to determine if the roaming analytics request/subscribe can be accepted or must be rejected. If the request is rejected, the following steps can be skipped.

3.	H-NWDAF checks the policies and regulatory constraints between the HPLMN and the VPLMN, and user consent for analytics as defined in clause 6.2.9 if needed, to determine if the roaming analytics request/subscribe can be accepted or must be rejected. If the roaming analytics request/subscribe is accepted, H-NWDAF collects data from the NF(s) and/or OAM in HPLMN and derives the requested analytics.

Editor's note:	The user consent check procedure and security aspects will align with SA3 WG's conclusion.

Editor's note:	If the HNWDAF with roaming entry capability can not support to generate the requested analytics, then how to exposure the requested analytics is FFS.

4.	H-NWDAF sends the HPLMN analytics information to the V-NWDAF using either Nnwdaf_RoamingAnalytics_Request response or Nnwdaf_RoamingAnalytics_Notify, depending on the service used in step 2. The H-NWDAF may restrict the exposed analytics information based on HPLMN operator polices.

5.	[Optional] V-NWDAF derives the requested analytics based on the data collected from the NF(s) and/or OAM in VPLMN and the analytics from H-NWDAF.

NOTE 2:	Analytics aggregation for roaming scenario is not supported in this release. How the V-NWDAF derives the requested analytics based on the analytics from H-NWDAF will depend on V-NWDAF implementation that is out of 3GPP scope.

6.	V-NWDAF sends the HPLMN analytics information received in step 4, or the analytics information derived in step 5 (if available), to the Consumer NF in VPLMN using either Nnwdaf_AnalyticsInfo_Request response or Nnwdaf_AnalyticsSubscription_Notify, depending on the service used in step 1.

# * * * * Next change * * * *

6.1.5.3	Analytics Exposure from VPLMN to HPLMN

Figure 6.1.5.3-1 shows the procedure that the H-NWDAF requests network analytics (i.e. service experience analytics, slice load level analytics, etc.) in VPLMN from the V-NWDAF with roaming entry capability, upon receiving an analytics information/subscription request from the NF in HPLMN (e.g. H-PCF).

Editor's note:	The service names in the following procedure are descriptive and may need to be updated.

Figure 6.1.5.3-1: Procedure for analytics exposure from VPLMN to HPLMN

1.	Consumer NF in HPLMN (e.g. H-PCF) sends an Analytics request/subscribe (Analytics ID, Target of Analytics Reporting, Analytics Filter Information) to H-NWDAF by invoking a Nnwdaf_AnalyticsInfo_Request or a Nnwdaf_AnalyticsSubscription_Subscribe.

NOTE 1:	The Target of Analytics Reporting sent by the consumer NF to the H-NWDAF may include both outbound roaming user(s) and non-roaming user(s).

2.	The H-NWDAF checks user consent for analytics as defined in clause 6.2.9 if needed, then the H-NWDAF discovers the V-NWDAF with roaming entry capability as described in clause 5.2.

H-NWDAF sends a roaming analytics request/subscribe (Analytics ID, Target of Analytics Reporting, Analytics Filter Information) to V-NWDAF by invoking a Nnwdaf_RoamingAnalytics_Request or a Nnwdaf_RoamingAnalytics_Subscribe, based on the Analytics request/subscribe received from the Consumer NF in HPLMN. The Target of Analytics Reporting sent by the H-NWDAF to the V-NWDAF shall only contain the outbound roaming user(s). The outbound roaming user(s) can be distinguished by the H-NWDAF according to the PLMN ID of the UE IDs. If H-NWDAF has the roaming entry capability, it checks the policies and regulatory constraints between the HPLMN and the VPLMN to determine if the roaming analytics request/subscribe can be accepted or must be rejected. If the request is rejected, the following steps can be skipped.

3.	V-NWDAF checks the policies and regulatory constraints between the HPLMN and the VPLMN, to determine if the roaming analytics request/subscribe can be accepted or must be rejected. If the roaming analytics request/subscribe is accepted, V-NWDAF collects data from the NF(s) and/or OAM in VPLMN and derives the analytics.

Editor's note:	The user consent check procedure and security aspects will align with SA3 WG's conclusion.

Editor's note:	If the VNWDAF with roaming entry capability can not support to generate the requested analytics, then how to exposure the requested analytics is FFS.

4.	V-NWDAF sends the VPLMN analytics information to the H-NWDAF using either Nnwdaf_RoamingAnalytics_Request response or Nnwdaf_RoamingAnalytics_Notify, depending on the service used in step 2. The V-NWDAF may restrict the exposed analytics information based on VPLMN operator polices.

5.	[Optional] H-NWDAF derives the analytics based on the data collected from the NF(s) and/or OAM in HPLMN and the analytics from V-NWDAF.

NOTE 2:	Analytics aggregation for roaming scenario is not supported in this release. How the H-NWDAF derives the requested analytics based on the analytics from V-NWDAF will depend on H-NWDAF implementation that is out of 3GPP scope.

6.	H-NWDAF sends the VPLMN analytics information received in step 4, or the analytics information derived in step 5 (if available), to the Consumer NF in HPLMN using either Nnwdaf_AnalyticsInfo_Request response or Nnwdaf_AnalyticsSubscription_Notify, depending on the service used in step 1.

# * * * * End of changes * * * *