3GPP SA WG2 Meeting #157 	S2-2306944

# May 22 - 26, 2023, Berlin, Germany	(revision of S2-2306065)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1284 | rev | 3 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Supporting UE centric AR Telephony Communication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | [Ericsson], ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | WID NG_RTC has objective to support AR telephony communication in IMS network, which has been concluded in TR 23.700-87. This CR reflects the TR conclusions. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new Annex for both the UE centric AR Architecture and network centric AR architecture based on the DC architecture. This CR includes the UE centric architecture. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete Specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex AC.X(new), AC.X.1, AC.X.2, AC.X.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: Clarify the AR specific data used for AR rendering could be from 3 sources: UE’s local AR application, peer UE’s AR application, and AR Application Server; and for the last two cases the application data channel needs to be estabhlished to transfer the AR specific data. |  |  |  |  |  |  |  |  |

# * * * * First change * * *

AC.X Support of AR Communication

AC.X.1 General

This annex describes the enhancements to the IMS architecture supporting data channel services to additionally support  AR services. Two modes of AR services are supported; UE centric and network centric.

Editor’s Note: The definition of ‘AR media’ is for FFS.

AC.X.2 Architecture

AC.X.2.1 UE Centric AR

For the UE centric case, AR traffic is transparent to the IMS network and is exchanged between the two peer UEs via the deployed media functions (e.g. IMS AGW/TrGW).

Figure AC.X.2.1-1 shows the architecture to support UE Centric AR communication.

Figure AC.X.2.1-1: Architecture to support UE Centric AR communication

AC.X.3 Procedures

AC.X.3.1 UE Centric Procedure

Figure AC.X.3.1-1 depicts a typical call flow procedure to establish a UE centric AR IMS session from UE-A perspective. The main steps in the call flow are as follows:

1.	UE-A initiates an IMS communication with UE-B, including establishment of bootstrap data channel.

2.	The user of UE-A upgrades the IMS session with AR experience. 
UE-A initiates a re-INVITE adding media descriptors required by the AR application to be established E2E, e.g. RTP and/or application data channels. Application data channel may be anchored in the MF/MRF.

The UE performs AR media rendering based on the AR specific data which could be obtained from the following sources:

- UE’s local AR application;

- peer UE’s AR application, in this case the application data channel between two UEs is established to transfer the AR specific data;

- AR Application Server, in this case the application data channel between the UE and AR Application Server is established to transfer the AR specific data.

3.	UE-A captures and encodes AR media.

4.	UE-A sends the AR media to the peer through the established media connection(s).

5.	UE-A receives AR media from the peer through established media connection(s).

6.	UE-A decodes and displays the received  AR media on its screen.

Figure AC.X.3.1-1: Establishment of UE Centric AR communication session

# * * * * End of changes * * * *