3GPP TSG-WG SA2 Meeting #S2-157	S2-2306956

# 22 - 26 May, 2023, Berlin, Germany		(revision of)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0385 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Revisions on UE unaware positioning procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Inspur |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eLCS_Ph3 |  |  |  |  |  | Date: |  |  | 2023-05-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - According to Clause 6.1.1, 5GC-MT-LR for regulatory location service is only requested by external LCS Client. - UE should be unaware of any messages (such as paging, LCS message and LPP message) related to 5GC-MT-LR procedure for regulatory location service no matter which state the target UE is in including CM-IDLE, CM-Connected as well as CM-Connected with RRC-Inactive. - It is common procedure that the stored target UE location estimate can be sent to LCS client if the LCS QoS requested from LCS client for regualtory location service is satisfied. So there is no extra description. - A note is included in clause 6.1.1, specifying the description regarding to UE unaware postioning is only applied to reguatory location service if there is a reference to the steps in clause 6.1.1. - The target UE can be paged only if the UE/User unaware indication is not included. - The serving AMF is unaware of RRC_INACTIVE state of target UE, so the condition is not needed to decide to not page target UE. - In step 5, it is impossible that the step 5 is skipped. The skipped steps should get started from step 6. - In step 8, the UE positioning procedure in clause 6.11.2 is the Uplink Positioning method in RAN. And there is no defination about Uplink Postioning method in SA2. - In step 10, a failure of LCS should be replied to GMLC due to UE/User unawareness restriction. - In clause 6.11.2, it is possible that target UE is in CM-IDLE mode. So the AMF should reject the Uplink Positioning message from LMF if UE unawareness indication is available. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - AF is removed from 5GC-MT-LR for regulatory location service. - UE should be unaware of any LCS related messages. - There is no need to specify that stored target UE location information can be used for location response. - Specfy that the content related to UE unaware positioning is not involved if there is a reference to clause 6.11.1, TS 23.273. - Both UE unaware positioning indication and CM-IDLE stated are included to specifiy whether or not the target UE can be paged during LCS procedure. - Remove the condition that target UE should not be paged if target UE’s RRC-INACTIVE state is not needed. - Reword the description regarding removing network triggered Service Request and step 6-9. - Remove the despcripion regarding to Uplink Positioning method. - Include a LCS failure cause due to UE/User unawareness restriction. - Specify that target UE should not be paged in network assisted positioning procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - It is misleading that AF can trigger 5GC-MT-LR for regulatory location service. - Seemingly target UE can receive other LCS-related message except paging message. - Redundant descrption that stored UE location information can be used to respond LCS request. - It is misleading that the content related to UE unaware positioning is applied to other procedure if they refers to clause 6.11.1. - It is misleading that AMF initiate network triggered Service Request procedure without considering UE/User unaware indication. - It is misleading that AMF initiate network triggered Service Request procedure without considering UE’s RRC-INACTIVE state. - It is incorred that step 5 can skip step 5. - Uplink Positioning method is not specified in SA2. - The LCS failure due to UE/User unawareness restriction is missing. - Target UE can be paged in network assisted positioning procedure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.12, 6.1.1, 6.11.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.12	UE Unaware Positioning

UE Unaware Positioning is only applied to the regulatory location service. When UE Unaware Positioning is requested by LCS client external to the PLMN for the regulatory location service. . During the 5GC-MT-LR procedure, target UE should not receive any messages from the serving AMF or LMF indicating it is for the UE’s LCS procedure as specified in clause 6.1.1.

# * * * * Second change * * * *

6.1.1	5GC-MT-LR procedure for the regulatory location service

Figure 6.1.1-1 illustrates the general network positioning for the LCS clients external to the PLMN for the regulatory location service for non-roaming scenario. In this scenario, it is assumed that the target UE is identified using a SUPI or GPSI.

This procedure is applicable to a request from an LCS client for a current location of the target UE, and it is assumed that the LCS client is authorised to use the location service and no privacy verification is required.

NOTE:	The specification related to UE unaware indication is not applicable to other LCS procedures if there is a reference to the current procedure.

Figure 6.1.1-1: 5GC-MT-LR procedure for the regulatory location service

1.	The external location services client sends a request to the GMLC for a location for the target UE identified by a GPSI or a SUPI. The request may include the required QoS, UE unaware indication and Supported GAD shapes. If location is required for more than one UE, the steps following below may be repeated and in that case the GMLC shall verify whether the number of Target UEs in the LCS request is equal to or less than the Maximum Target UE Number of the LCS client. If Maximum Target UE Number is exceeded, the GMLC shall reject the LCS request, the step 2-10 are skipped, and then GMLC respond to the client with proper error cause in the step 11.

2.	The GMLC invokes a Nudm_UECM_Get service operation towards the home UDM of the target UE to be located with the GPSI or SUPI of this UE.

3.	The UDM returns the network addresses of the current serving AMF.

NOTE:	For backward compatibility, the GMLC can use a Nudm_SDM_Get service operation to retrieve the SUPI of the target UE from a Rel-15 UDM as defined in clause 5.2.3.3.2 of TS 23.502 [19].

4.	The GMLC invokes the Namf_Location_ProvidePositioningInfo service operation towards the AMF to request the current location of the UE. The service operation includes the SUPI, and client type and may include the required QoS, UE unaware indication and Supported GAD shapes.

5.	If the UE is in CM-IDLE state and UE/User unaware indication is not included, the AMF initiates a network triggered Service Request procedure as defined in clause 4.2.3.3 of TS 23.502 [19] to establish a signalling connection with the UE.

If the UE is in CM-IDLE state and UE/User unaware indication is included, the AMF should not initiate the network triggered Service Request procedure as defined in clause 4.2.3.3 of TS 23.502 [19] and step 6-9 are skippped.

6.	The AMF selects an LMF based on the available information as defined in clause 5.1 or based on AMF local configuration. The LMF selection takes the 5G-AN currently serving the UE into account. The selection may use a NRF query.

7.	The AMF invokes the Nlmf_Location_DetermineLocation service operation towards the LMF to request the current location of the UE. The service operation includes a LCS Correlation identifier, the serving cell identity of the Primary Cell in the Master RAN node and the Primary Cell in the Secondary RAN node when available based on Dual Connectivity scenarios, and the client type and may include an indication if UE supports LPP, the required QoS, UE Positioning Capability if available, UE unaware indication and Supported GAD shapes. If any of the procedures in clause 6.11.1 or clause 6.11.2 are used the service operation includes the AMF identity.

8.	The LMF performs one or more of the positioning procedures described in clauses 6.11.1, 6.11.2 and 6.11.3. During this step the LMF may use the Namf_Communication_N1N2MessageTransfer service operation to request the transfer of a Positioning related N1 message to the UE or the transfer of a Network Positioning message to the serving NG-RAN node (gNB or NG-eNB) for the UE. The LMF shall determine a geographical location and optionally a location in local coordinates.

If UE unaware indication is received from AMF in step 7, the LMF performs the positioning procedure described in clause 6.11.2.

9.	The LMF returns the Nlmf_Location_DetermineLocation Response towards the AMF to return the current location of the UE and UE Positioning Capability if the UE Positioning Capability is received in step 8 including an indication that the capabilities are non-variable and not received from AMF in step 7. The service operation includes the LCS Correlation identifier, the location estimate, its age and accuracy and may include information about the positioning method and the timestamp of the location estimate.

If UE Positioning procedure is rejected with the rejection cause in step 2 or step 4 of clause 6.11.2, the LMF should return Nlmf_Location_DetermineLocation Response towards the AMF with an indication indicating that the target UE’s location cannot be captured due to UE/User positioning unawareness restriction.

10.	The AMF returns the Namf_Location_ProvidePositioningInfo Response towards the GMLC/LRF to return the current location of the UE. The service operation includes the location estimate, its age and accuracy and may include information about the positioning method and the timestamp of the location estimate. The AMF stores the UE Positioning Capability in UE context when received from LMF.

If the target UE’s location is not available due to the UE/User positioning unawareness restriction as specified in step 5 and step 9 and the stored target UE’s location information doesn’t satisfy the LCS client’s QoS requirements, the AMF should respond location request with a failure cause indicating the UE’s location is not available due to UE/User unawareness restriction.

11.	The GMLC sends the location service response to the external location services client.

# * * * * Third change * * * *

6.11.2	Network Assisted Positioning Procedure

Figure 6.11.2-1 shows a procedure that may be used by an LMF to support network assisted and network based positioning. The procedure may be based on an NRPPa protocol in TS 38.455 [15] between the LMF and NG-RAN.

Figure 6.11.2-1: Network Assisted Positioning Procedure

Precondition:	A LCS Correlation identifier and the AMF identity have been passed to the LMF by the serving AMF.

1.	The LMF invokes the Namf_Communication_N1N2MessageTransfer service operation towards the AMF to request the transfer of a Network Positioning message to the serving NG-RAN node (gNB or ng-eNB) for the UE. The service operation includes the Network Positioning message and the LCS Correlation identifier. The Network Positioning message may request location information for the UE from the NG-RAN and should include UE unaware indication for the UE if it is received by LMF from AMF.

2.	If the UE is in CM-IDLE state and there is no UE/User unaware indication for the UE in the UE’s location context, the AMF initiates a network triggered Service Request procedure as defined in clause 4.2.3.3 of TS 23.502 [19], to establish a signalling connection with the UE.

If the UE is in CM-IDLE state and there is UE/User unaware indication for the UE in the UE’s location context, the AMF should reject the forwarding request of the Network Positioning message with a proper rejection cause value and step 3-6 are skipped.

3.	The AMF forwards the Network Positioning message to the serving NG-RAN node in an N2 Transport message. The AMF includes a Routing identifier, in the N2 Transport message, identifying the LMF (e.g. a global address of the LMF).

4.	The serving NG-RAN node obtains any location information for the UE requested in step 3.

If UE unaware indication is received in the Network Positioning message, and the UE is in RRC_INACTIVE state, the NG-RAN rejects the Network Positioning message with appropriate rejection cause (e.g. UE cannot be paged).

5.	The serving NG-RAN node returns any location information obtained in step 4 to the AMF in a Network Positioning message included in an N2 Transport message. The serving NG-RAN node shall also include the Routing identifier in the N2 Transport message received in step 3.

6.	The AMF invokes the Namf_Communication_N2InfoNotify service towards the LMF indicated by the routing identifier received in step 5. The service operation includes the Network Positioning message received in step 5 and the LCS Correlation identifier. Steps 1 to 6 may be repeated to request further location information and further NG-RAN capabilities.

# * * * * End of changes * * * *