3GPP TSG-WG SA2 Meeting #157	S2-2306980

# Berlin, Germany, May 22 – 26, 2023		(revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0388 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Removal of functional description of inter-LMF GNSS assistance data transfer |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eLCS_Ph3 |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - In SA2 156 e-meeting, inter-LMF GNSS assistance data transfer procedure was propsoed, to align with the following functional description: LMF(s) collected GNSS assistance data may update its NF profile in NRF by indicating the TAs of which corresponding GNSS assistance data exists, then the LMF can be discovered and its collected GNSS assistance data can be used by other LMFs not having corresponding GNSS assistance data. This was to enable a deployment scenario that a opertar may deploy dedicated LMFs to collect GNSS assistance data from GNSS reference network operators. However, after discussion, it was realized that if multiple types of GNSS assistance data are to be collected by different LMFs (as we cannot make it mandotary in standardization that each dedicated LMF has to collecte all types of GNSS assistance data), it will introduce unneccessary complexity on tranfsering GNSS assistance data per types. Thus, we propsoe to leave this functionality to implementation in this release and remove corresponding functional description. 2. Further, to avoid misunderstanding of how GNSS assisatance data are collected per area, a NOTE is needed to clarify that both parameters applicable to global area and parameters (e.g., RTK related parameters) applicable to specific areas can be collected by the LMF, the parameters applicable to specific areas determines the applicable area (e.g., broadcasting areas) of the collected GNSS assistance data. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Remove inter-LMF GNSS assistance data transfer functional description. - Clarify that both parameters applicable to global area and parameters (e.g., RTK related parameters) applicable to specific areas can be collected by the LMF. The parameters applicable to specific areas determines the applicable area (e.g., broadcasting areas) of the collected GNSS assistance data. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment of procudre and functional description. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.11	Collection of GNSS assistance data

LMF may collect GNSS assistance data by reusing Data Collection from AF as described in clause 5.20a of TS 23.501 [18] and Naf_EventExposure service described in clause 5.2.19.2 of TS 23.502 [19]. The collection may be performed on a periodic basis.

LMF generates a mapping table of GNSS assistant data and applicable area (e.g. TA lists). LMF may generate multiple sets of GNSS assistance data for the same location area, and each of them corresponds to a certain location accuracy. The accuracy of GNSS assistance data may vary for different areas.

NOTE 1:	The multiple accuracy level of GNSS assistance data can be the reason the collection method, and /or calculation method.

After the collection, LMF may send the collected GNSS assistance data to UEs via the procedures described in clause 6.14.1 (Broadcast of Assistance Data by an LMF) or clause 6.11.1 (UE Assisted and UE Based Positioning Procedure) during a positioning session.

In the case of using Broadcast of Assistance Data by an LMF, the LMF may provision one or more of GNSS assistance data to RAN. LMF may also provision one or more cyphered key(s) to UE from the AMF, and each cyphered key corresponds to a certain GNSS assistance data.

NOTE 2: The collected GNSS assistance data contains parameters applicable to global area (e.g., almanac) and parameters (e.g., RTK related parameters) applicable to specific areas, the parameters applicable to specific areas determines the applicable area of the collected GNSS assistance data.

# * * * * End of changes * * * *