3GPP TSG-SA2 Meeting #157	S2-2307040

Berlin, Germany, May 22 – 26, 2023	(revision of S2-2306041)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 3825 | rev | 6 | Current version: | 18.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of Alternative S-NSSAI replacement determined by NSSF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo, Nokia, Nokia Shanghai Bell, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS_Ph3 |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As part of the FS_eNS_Ph3 study, the TR 23.700-41 V2.0.0 has concluded in clause 8.1 for KI#1 that in one option the Alternative S-NSSAI can be determined by the NSSF. The corresponding CR to 23.501 has been approved in SP-230315. This CR attempts to implement the changes needed for the NSSF service operations. In rev 6, the following changes has been added: - (according to the unhandled CR in S2-2304597 from SA2#156e meeting) Nnssf_NSSAIAvailability_Notify and Nnssf_NSSAIAvailability_Unsubscribe operations. - for the case of Network Slice Replacement due to congestion: it is proposed that the NSSF provides to the AMF congestion control information which may include an indication that the Network Slice replacement applies 1) for a percentage of registered UEs or 2) only for new UEs registering to the Network Slice. - when termination of the Network Slice replacement is required, the NSSF provides to the AMF 1) indication to stop the Network Slice replacement for new UEs or 2) indication to terminate the Network Slice replacement and move back the UEs and PDU Sessions from the Alternative S-NSSAI to the S-NSSAI. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to specify the enhancements to the NSSF service operations resulting from agreed 23.501. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The TS 23.502 will not be aligned to TS 23.501. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.16.1, 5.2.16.3.1, 5.2.16.3.3, 5.2.16.3.4, 5.2.16.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## FIRST CHANGE

#### 5.2.16.1	General

The following table illustrates the NSSF Services.

Table 5.2.16.1-1: NF Services provided by NSSF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnssf_NSSelection | Get | Request/Response | AMF, NSSF in a different PLMN, SMF, NWDAF |
| Nnssf_NSSAIAvailability | Update | Request/Response | AMF |
|  | Subscribe | Subscribe/Notify | AMF, NSSF |
|  | Unsubscribe |  | AMF, NSSF |
|  | Notify |  | AMF, NSSF |
|  | Delete | Request/Response | AMF |

NOTE:	The SMF uses the Nnssf_NSSelection_Get service operation during EPS interworking.

## 2nd CHANGE

#### 5.2.16.3	Nnssf_NSSAIAvailability service

##### 5.2.16.3.1	General

Service description: This service enables to update the AMFs and the NSSF on the availability of S-NSSAIs and NSAGs on a per TA basis. This service also provides an Alternative S-NSSAI to the NF Service Consumer (e.g. AMF or NSSF in the VPLMN) when the NSSF determines that an S-NSSAI has to be replaced with an Alternative S-NSSAI according to the Network Slice Replacement as described in clause 5.15.19 [2].

NOTE:	The NSSF can determine the serving network and Access Type from the TAI, as described in TS 29.571 [70].

When this service is used by the NSSF for Network Slice Replacement, i.e. to provide to the NF Service Consumer an Alternative S-NSSAI to the S-NSSAI to be replaced, the following cases are possible:

- in non-roaming and in roaming case, NSSF in the Serving PLMN provides to the AMF the Alternative S-NSSAI of the Serving PLMN; or

- in roaming case, the NSSF in the HPLMN provides to the NSSF in the VPLMN the Alternative HPLMN S-NSSAI; and the NSSF in the VPLMN provides to the AMF the Alternative HPLMN S-NSSAI.

## 3rd CHANGE

##### 5.2.16.3.3	Nnssf_NSSAIAvailability_Notify service operation

Service operation name: Nnssf_NSSAIAvailability_Notify

Description: This service operation enables the NSSF to update a NF Service Consumer (e.g. AMF) with any S-NSSAIs restricted per TA and, if needed, subsequently lift any restriction per TA. This service operation also enables the NSSF to notify to the NF Service Consumer (e.g. AMF or NSSF in the VPLMN) that an S-NSSAI has to be replaced by an Alternative S-NSSAI.

Inputs, Required: SubscriptionId.

Inputs, Conditional Required:

If this service operation is invoked by subscription for changes in the status of the NSSAI availability information, then the following inputs are required:

- a list of TAIs and the S-NSSAIs for which the status is changed (restricted/unrestricted) per each TAI.

If this service operation is invoked by subscription for event notification for Network Slice Replacement, then the following inputs are required:

-	For replacement of an S-NSSAI of the serving PLMN: an Alternative S-NSSAI and the corresponding mapping to the S-NSSAI to be replaced.

-	For replacement of an HPLMN S-NSSAI: an Alternative S-NSSAI and the corresponding mapping to the S-NSSAIs of the HPLMN, PLMN ID.

-	For termination of the Network Slice replacement: 1) indication to stop the Network Slice replacement for new UEs or 2) indication to terminate the Network Slice replacement and move back the UEs and PDU Sessions from the Alternative S-NSSAI to the S-NSSAI.

Inputs, Optional:

If this service operation is invoked by subscription for changes in the status of the NSSAI availability information, then the following inputs are optional:

- NSI IDs in the S-NSSAI for which the status is changed (restricted/unrestricted) per each TAI.

If this service operation is invoked by subscription for event notification for Network Slice Replacement, then the following inputs are optional:

- For Network Slice Replacement due to congestion: congestion control information which may include an indication that the Network Slice replacement applies 1) for a percentage of registered UEs or 2) only for new UEs registering to the Network Slice.

Outputs, Required: None.

Outputs, Optional: None.

## 4th CHANGE

##### 5.2.16.3.4	Nnssf_NSSAIAvailability_Subscribe service operation

Service operation name: Nnssf_NSSAIAvailability_Subscribe

Description: This service operation enables a NF Service Consumer (e.g. AMF) to subscribe to a notification of any changes in status of the NSSAI availability information (e.g. S-NSSAIs and optionally NSI IDs available per TA and the restricted S-NSSAI(s) and optionally NSI IDs per PLMN in that TA in the serving PLMN of the UE) upon this is updated by another AMF. This service also enables the NF Service Consumer (e.g. AMF or NSSF in the VPLMN) to subscribe to a notification when an S-NSSAI has to be replaced by an Alternative S-NSSAI.

Inputs, Required: Callback URI of the NF Service Consumer.

Inputs, Conditional Required:

If this service operation is invoked to subscribe for changes in status of the NSSAI availability information, then the following inputs are required:

-	list of TAIs supported by the NF Service Consumer, event to be subscribed.

If this service operation is invoked to subscribe to a notification for Network Slice Replacement, then the following inputs are required:

-	for the S-NSSAI of the serving PLMN: set of S-NSSAIs served by the NF Service Consumer that may be replaced, NF type of the NF Service Consumer (e.g. AMF), Requester ID.

-	for HPLMN S-NSSAI: S-NSSAIs for the HPLMN, NF type of the NF Service Consumer (e.g. AMF or NSSF), Requester ID.

Inputs, Optional: Expiry time.

Outputs, Required: SubscriptionID.

Outputs, Conditional Required:

If this service operation is invoked to subscribe for changes in status of the NSSAI availability information, then the following outputs are required:

-	Expiry time (if present in the request, may be included in the response based on operator's policy and taking into account the expiry time present in the request (i.e. should be less than or equal to that value); if not present in the request, may be included in the response based on operator's policy. Whatever the case, if not included in the response, this means that the subscription is valid without an expiry time).

Outputs, Optional:

If this service operation is invoked to subscribe for changes in status of the NSSAI availability information, then the following outputs are optional:

- A list of TAIs and, for each TAI, the S-NSSAIs and optionally NSI IDs supported by the AMF and 5G-AN and authorized by the NSSF for the TAI and a list of S-NSSAIs and optionally NSI IDs restricted per PLMN for the TAI. The NSSS indicates to the AMF that the S-NSSAI(s) are not available in the corresponding TAIs.

If this service operation is invoked to subscribe to a notification for Network Slice Replacement, then the following outputs are optional:

-	For replacement of an S-NSSAI of the serving PLMN: an Alternative S-NSSAI and the corresponding mapping to the S-NSSAI to be replaced.

-	For replacement of an HPLMN S-NSSAI: an Alternative S-NSSAI and the corresponding mapping to the S-NSSAIs of the HPLMN, PLMN ID.

-	For Network Slice replacement due to congestion (for any of the above cases): congestion control information which may include an indication that the Network Slice replacement applies 1) for a percentage of registered UEs or 2) only for new UEs registering to the Network Slice.

## 5th CHANGE

##### 5.2.16.3.5	Nnssf_NSSAIAvailability_Unsubscribe service operation

Service operation name: Nnssf_NSSAIAvailability_Unsubscribe

Description: This service operation enables a NF Service Consumer (e.g. AMF) to unsubscribe to a notification of any previously subscribed changes to the NSSAI availability information. This service operation also enables the NF Service Consumer (e.g. AMF, NSSF) to unsubscribe to a notification for Network Slice Replacement.

Inputs, Required: SubscriptionId.

Inputs, Optional: None.

Outputs, Required: None.

Outputs, Optional: None.

* * * END of CHANGES * * *