3GPP TSG-WG SA2 Meeting #157	S2-2307074

# Berlin, Germany, May 22 – 26, 2023		(revision of S2-2304535r4)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0761 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Extending Analytics Transfer to support also accuracy checking transfer to target NWDAF. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The TS 23.288 V18.1.0 defines the new NWDAF capability: accuracy checking capability, where an NF consumer can subscribe to analtyics and accuracy information from NWDAF containing AnLF. As per R17 NWDAF definitions, analytics subscriptions can be transferred between source and target NWDAFs containing AnLF. With the addition of the new accuracy checking capability, it is also necessary to update the existing transfer menchanisms to properly handle: - The case where the target NWDAF does not support accuracy check capability - The case where the target NWDAF supports accuracy check capability and the subscription to analytics ID to be transfer includes a request for analytics accuracy generation. Please find the detailed discussion in S2-2304536. |  |  |  |  |  |  |  |  |
| Summary of change: |  | #1 – Updating analytics exposure parameters to enable the source NWDAF to notify the NF consumer that a request for analytics accuracy information has been terminated, e.g., due to the target NWDAF not supporting accuracy checking capability. #2 – Updating procedures description to consider also the analytics accuracy checking capability in the selection process of the target NWDAF, in case the source analytics provides accuracy information associated with the analytics subscription to be transferred. #3 - Update of the input parameters of the Nnwdaf_AnalyticsSubscription_Transfer Request to support the transfer of analytics accuracy related information #4 – Update of description of the Analytics Context Transfer to define the actions at the target NWDAF when it receives the analytics accuracy related information during an analytics transfer. #5 – Update of input (analytics accuracy related information as a new context type to be requested) and output (extension of Analytics context information with the list of parameters associated with the analytics accuracy related information) parameters of Nnwdaf_AnalyticsInfo_ContextTransfer service operation to support the request and transfer of analytics accuracy related information. #6 – Updating the Nnwdaf_AnalyticsSubscription_Transfer parameters to make it aligned with the description of such parameter’s usage in the procedures of clause 6.1B.2.2 and 6.1B.2.3, where the (set of) analytics context identifier(s) is an optional parameters, and therefore if this parameter is not included in the transfer request, then the analytics accuracy related information shall be included if the target NWDAF has the accuracy checking capability. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NWDAF with support for accuracy checking capability defined in R18 will not be properly provided when analytics transfers among NWDAFs are executed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3, 6.1B.2.1, 6.1B.2.2, 6.1B.2.3, 6.1B.3, 6.1B.4, 7.2.4, 7.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | The revision 1 of this CR is based on the changes discussed during SA2#156e and conslidated in S2-2304535r4. The proposed further changes are included: - Updating the description of the Analytics Accuracy Termination parameter with: “as an indication that a new target serving NWDAF supporting accuracy checking capability could not have been selected during the analytics transfer procedures” - Removing the inclusion of the inference configuration as a parameters of the “Analtyics Accuracy Related Information” to be transfer in the Analtyics Transfer Context - Inclusion in the Clause 6.1B.3 of how the target NWDAF uses the timestamps of the historical data received in the analtyics context transfer to determine the inference configuration for data collection used at the source NWDAF, and decide whether to use the same inference configuration for the generation of the analtyics accuracy information. This change reflects the email discussions during SA2#156e. |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.1.3	Contents of Analytics Exposure

The consumers of the Nnwdaf_AnalyticsSubscription_Subscribe or Nnwdaf_AnalyticsInfo_Request service operations described in clause 7 provide the following input parameters listed below.

-	A list of Analytics ID(s): identifies the requested analytics.

-	Analytics Filter Information: indicates the conditions to be fulfilled for reporting Analytics Information. This set of optional parameter types and values enables to select which type of analytics information is requested. Analytics Filter Information is defined in the analytics related clauses.

-	Target of Analytics Reporting: indicates the object(s) for which Analytics information is requested, entities such as specific UEs, a group of UE(s) or any UE (i.e. all UEs).

-	(Only for Nnwdaf_AnalyticsSubscription_Subscribe) A Notification Target Address (+ Notification Correlation ID) as defined in clause 4.15.1 of TS 23.502 [3], allowing to correlate notifications received from NWDAF with this subscription.

-	(Only for Nnwdaf_AnalyticsSubscription_Subscribe) Subscription Correlation ID: identifies an existing analytics subscription that is to be modified.

-	Related to analytic consumers that aggregate analytics from multiple NWDAF subscriptions:

-	[OPTIONAL] (Set of) NWDAF identifiers of NWDAF instances used by the NWDAF service consumer when aggregating multiple analytics subscriptions. See clause 6.1A.

-	Analytics Reporting Information with the following parameters:

-	(Only for Nnwdaf_AnalyticsSubscription_Subscribe) Analytics Reporting Parameters as per Event Reporting parameters defined in Table 4.15.1-1 of TS 23.502 [3].

NOTE 1:	When the Analytics Reporting Parameters indicates a periodic reporting mode and the periodicity of the report is equal to or greater than the Supported Analytics Delay associated with the Analytics ID (if available) defined in clause 6.2.6.2 of TS 23.501 [2], it is expected that the periodic reporting can be provided by the NWDAF as requested.

-	(Only for Nnwdaf_AnalyticsSubscription_Subscribe) Reporting Thresholds, which indicate conditions on the level of each requested analytics that when reached shall be notified by the NWDAF.

-	[OPTIONAL] Matching direction: A matching direction may be provided such as below, above, or crossed. If no matching direction is provided, the default direction is crossed.

-	[OPTIONAL] Acceptable deviation: An acceptable deviation from the threshold level in the non-critical direction (i.e. in which the QoS is improving) may be set to limit the amount of signalling.

-	Analytics target period: time interval [start..end], either in the past (both start time and end time in the past) or in the future (both start time and end time in the future). An Analytics target period in the past is a request or subscription for statistics. An Analytics target period in the future is a request or subscription for predictions. The time interval is expressed with actual start time and actual end time (e.g. via UTC time). When the Analytics Reporting Parameters indicate a periodic reporting mode, the time interval can also be expressed as positive or negative offsets to the reporting time, which indicates a subscription for predictions or statistics respectively. By setting start time and end time to the same value, the consumer of the analytics can request analytics or subscribe to analytics for a specific time rather than for a time interval.

-	Time window for historical analytics: time interval [start..end]. The time window for historical analytics indicates the time interval during which the historical analytics was generated. If the time window for historical analytics is included, the NWDAF only needs to provide the existing analytics, and does not need to generate new analytics.

-	[OPTIONAL] Data time window: if specified, only events that have been created in the specified time interval are considered for the analytics generation.

-	[OPTIONAL] Preferred level of accuracy of the analytics ("Low", "Medium", "High" or "Highest").

-	[OPTIONAL] Preferred level of accuracy per analytics subset ("Low", "Medium", "High" or "Highest"). When a preferred level of accuracy is expressed for a given analytics subset, it takes precedence for this subset over the above preferred level of accuracy of the analytics. Analytics subsets are defined in the "Output Analytics" clause of applicable analytics.

-	[OPTIONAL] Dataset Statistical Properties: information in order to influence the data selection mechanisms to be used for the generation of an Analytics ID, assuring that the generated Analytics ID reflects the statistical characteristics of the data that are relevant for the NWDAF consumer. The following dataset statistical properties are allowed:

-	Uniformly distributed datasets, which indicates the use of data samples that are uniformly distributed according to the different aspects of the requested analytics (e.g. equivalent data samples for each UE listed as a Target of Analytics Reporting or for S-NSSAIs included in the Analytics Filter Information).

-	Datasets with or without outliers, which indicates that the data samples shall consider or disregard data samples that are at the extreme boundaries of the value range.

-	Time when analytics information is needed (if applicable): indicates to the NWDAF the latest time the analytics consumer expects to receive analytics data provided by the NWDAF. It should not be set to a value less than the Supported Analytics Delay of the selected NWDAF if applicable. If the time is reached the consumer does not need to wait for the analytics information any longer, yet the NWDAF may send an error response or error notification to the consumer. "Time when analytics information is needed" is a relative time interval as the gap with respect to analytics request /subscription (e.g. "in 10 minutes").

NOTE 2:	If the analytics request contains the parameter "Time when analytics information is needed" for Analytics ID(s), this parameter takes precedence over the requested periodicity, if a periodic reporting mode is requested.

NOTE 3:	If the Time when analytics information is needed is provided and it is less than the Supported Analytics Delay per Analytics ID (if available) defined in clause 6.2.6.2 of TS 23.501 [2], it is expected that the NWDAF might not be able to treat the Analytics ID on time.

-	[OPTIONAL] Maximum number of objects requested by the consumer (max) to limit the number of objects in a list of analytics per Nnwdaf_AnalyticsSubscription_Notify or Nnwdaf_AnalyticsInfo_Request response.

-	[OPTIONAL] Preferred granularity of location information: "TA level", "cell level" or "below cell level".

Editor's note:	How to describe "below cell level" granularity is FFS and the value might be replaced by several values, or additional parameters to further qualify it might be defined.

-	[OPTIONAL] Spatial granularity size: maximum number of TA or cells used to define an area for which analytics are provided. When this parameter is provided, the NWDAF should provide analytics per group of TA of cells accordingly.

-	[OPTIONAL] Temporal granularity size: minimum duration of each time slot for which analytics are provide. When this parameter is provided, the NWDAF should provide analytics per elementary time slot accordingly.

Editor's note:	It needs to be clarified how UEs that are in an area for a shorter time than the Temporal granularity size are handled.

-	[OPTIONAL] Preferred orientation of location information: ("horizontal", "vertical", "both").

-	[OPTIONAL] Preferred order of results when a list of analytics is returned, possibly with a criterion for identifying the property of the results to which the preferred ordering is applied.

-	[OPTIONAL] Maximum number of SUPIs (SUPImax) requested by the consumer to limit the number of SUPIs in an object. When SUPImax is not provided, the NWDAF shall return all SUPIs concerned by the analytics object. When SUPImax is set to 0, the NWDAF shall not provide any SUPI.

-	[OPTIONAL] Output strategy: indicates the relevant factors for determining when the analytics reported. The following values are allowed:

-	Binary output strategy: indicates that the analytics shall only be reported when the preferred level of accuracy is reached within a cycle of periodic notification as defined in the Analytics Reporting Parameters.

NOTE 4:	If preferred level of accuracy is more important than providing an output, then the binary strategy is used so that all analytics outputs have equivalent confidence in the prediction.

-	Gradient output strategy: indicates that the analytics shall be reported according with the periodicity defined in the Analytics Reporting Parameters irrespective if the preferred level of accuracy has been reached.

NOTE 5:	If having an analytics output is more important than reaching the preferred level of accuracy, then the gradient output strategy is used so that each NWDAF will timely provide the output indicating the confidence of the prediction at the moment of the output generation.

NOTE 6:	When no output strategy is included in the subscription, the analytics output will be generated based on the gradient strategy and includes the confidence of the prediction for the reporting period.

-	[OPTIONAL] Analytics metadata request: indicates a request from one NWDAF to another NWDAF to provide the "analytics metadata information" related to the produced output analytics. This input parameter indicates which parameters in "analytics metadata information" are required to aggregate the output analytics for the requested Analytics ID(s).

-	(Only for Nnwdaf_AnalyticsSubscription_Subscribe) Consumer NF's serving area or NF ID. During a pending analytics subscription transfer, this information can be used by the NWDAF to find out if the analytics consumers may change as described in clause 6.1B.2.

-	(Only for Nnwdaf_AnalyticsSubscription_Subscribe) Information of previous analytics subscription. When setting up the analytics generation, this information may be used to retrieve analytics context from the previous NWDAF in order to build upon the context that is already related to this subscription as described in clause 6.1B.2.1.

-	[OPTIONAL] Use case context: indicates the context of use of the analytics to select the most relevant ML model.

NOTE 7:	The NWDAF can use the parameter "Use case context" to select the most relevant ML model, when several ML models are available for the requested Analytics ID(s). NWDAF containing AnLF can additionally provide the parameter "Use case context" when requesting an ML model from an NWDAF containing MTLF. The values of this parameter are not standardized. For example, the AMF can use a given value of "Use case context" when requesting UE Mobility analytics for optimizing the definition of a Registration Area, and a different value of "Use case context" when requesting UE Mobility analytics for determines a paging strategy.

-	(Only for Nnwdaf_AnalyticsSubscription_Subscribe) [OPTIONAL] Analytics feedback information: indicates that the consumer NF has taken an action(s) influenced by the previously provided analytics, which may or may not affect the ground truth data corresponding to analytic ID requested at the time which the prediction refers to, and consequently affect the ML Model Accuracy monitoring by the subscription with following parameter(s):

-	Corresponding Analytics ID(s) which has been used for taking an action(s);

-	Indication whether the action will affect on ground truth data (if available).

Editor's note:	It is FFS, whether "Time stamp of the action taken" or "Expected time duration for affecting the action" is needed.

NOTE 8:	The consumer NF cannot include Analytics feedback information in initial subscription request. Analytics feedback information can be included in modification request for the existing analytics subscription.

-	Analytics Accuracy Request information with the following parameters:

-	Analytics accuracy request: indicates NWDAF to provide accuracy information to the analytics consumer.

-	Analytics accuracy information time window: time interval [start. end], which indicates that analytics consumers only consider the accuracy information which is generated within this time interval.

-	[Optional] Analytics accuracy information periodicity: time period, which indicates periodic reporting of accuracy information for the corresponding Analytics ID(s).

-	[Optional] Accuracy deviation threshold(s): a reporting threshold of deviation value, which is indicated that the NWDAF can provide accuracy information to the analytics consumer when deviation value is above this threshold. The deviation value indicates the deviation of the predictions from the actual network data. If parameter "Stop analytics consumption flag" or "Updated Analytics flag" is provided, this parameter may also be used as reporting threshold for "Stop analytics consumption indication" or "Updated Analytics".

Editor's note:	It is FFS to determine whether the deviation refers to a difference between accuracy values or difference of prediction from the actual network data.

-	[OPTIONAL] Minimal number of analytics output occurrences: determines the minimal number of analytics output provided by NWDAF that have to be considered in the determination of the accuracy information.

-	[Optional] Updated Analytics flag: indicates that the NWDAF can provide updated analytics for provided Analytics ID(s), if updated analytics can be generated within analytics accuracy information time window.

-	[Optional] Correction time period: a relative time interval as the gap with respect to analytics is provided, which is indicated the time interval during which the updated analytics can be accepted by the analytics consumer.

-	[Optional] Pause analytics consumption flag: is a flag indicating to NWDAF to stop sending the notifications of analytics outputs for a subscribed analytics ID, without unsubscribing to such analytics ID.

-	[OPTIONAL] Pause Analytics Subscription time window: indicates the period of time that NWDAF should stop sending the notifications of analytics outputs for a subscribed analytics ID.

-	[OPTIONAL] Resume Analytics Subscription request: is a flag indicating to NWDAF to resume the notification of analytics outputs for an existing analytics ID(s) subscription(s) that have been previously paused.

The NWDAF provides to the consumer of the Nnwdaf_AnalyticsSubscription_Notify or Nnwdaf_AnalyticsInfo_Request service operations described in clause 7, the output information listed below:

-	(Only for Nnwdaf_AnalyticsSubscription_Notify) The Notification Correlation Information.

-	For each Analytics ID the analytics information in the requested Analytics target period.

-	Timestamp of analytics generation: allows consumers to decide until when the received information shall be used. For instance, an NF can deem a received notification from NWDAF for a given feedback as invalid based on this timestamp;

-	Validity period: defines the time period for which the analytics information is valid.

-	Confidence: probability assertion:, i.e. confidence in the prediction.

-	[OPTIONAL] For each Analytics ID the Termination Request, which notifies the consumer that the subscription is requested to be cancelled as the NWDAF can no longer serve this subscription, e.g. due to user consent revoked, NWDAF overload, UE moved out of NWDAF serving area, etc.

-	[OPTIONAL] Analytics metadata information: additional information required to aggregate the output analytics for the requested Analytics ID(s). This parameter shall be provided if the "Analytics metadata request" parameter was provided in the corresponding Nnwdaf_AnalyticsSubscription_Subscribe or Nnwdaf_AnalyticsInfo_Request service operation.

-	Number of data samples used for the generation of the output analytics;

-	Data time window of the data samples;

-	Dataset Statistical Properties of the analytics output used for the generation of the analytics;

-	[OPTIONAL] Data source(s) of the data used for the generation of the output analytics;

-	[OPTIONAL] Data Formatting and Processing applied on the data;

-	Output strategy used for the reporting of the analytics.

-	(Only for error response or error notification) Revised waiting time: indicates to the consumer a revised waiting value for "Time when analytics information is needed". Each NWDAF may include this as part of error response or error notification to "Time when analytics information is needed" as described in clause 6.2.5. Revised waiting time is the minimum time interval recommended by NWDAF to use as "Time when analytics information is needed" for similar future analytics requests/subscriptions.

-	Analytics accuracy information generated for each analytics ID, including:

-	Accuracy value for requested Analytics ID(s): a value shall be provided if "Analytics accuracy request" parameter was provided in the corresponding Nnwdaf_AnalyticsSubscription_Subscribe service operation.

Editor's note:	It is FFS to determine if this parameter may be provided if the deviation between predictions and its corresponding ground truth is above specified accuracy deviation threshold.

-	(Optionally) An indication that the determined accuracy value for the analytics ID does not meet the preferred level of accuracy requested for the analytics ID.

Editor's note:	It is FFS to determine how an accuracy value relates to the preferred level of accuracy requested for the analytics ID.

-	[Optional] Updated Analytics: NWDAF provides updated Analytics, which is generated within analytics accuracy information time window, for provided Analytics ID(s), if "Updated Analytics flag" parameter was indicated in the corresponding Nnwdaf_AnalyticsSubscription_Subscribe service operation.

-	[Optional] Stop analytics consumption indication: NWDAF provides to the consumer an indication to stop the consumption of the Analytics ID(s) related to the subscription ID based on NWDAF internal logic or specified accuracy deviation threshold.

-	Resume Analytics Output Consumption indication: NWDAF provides to consumer an indication to resume the consumption of analytics output for existing subscription to the analytics ID(s) that was previously paused.

-	[OPTIONAL] Accuracy Information Termination: NWDAF notifies the consumer that the subscription to the accuracy information for an analytics ID has been cancelled as the NWDAF does not support the accuracy checking capability, e.g. as an indication that a new target serving NWDAF supporting accuracy checking capability could not have been selected during the analytics transfer procedures.

# * * * * Second change * * * *

6.1B.2.1	Analytics context transfer initiated by target NWDAF selected by the NWDAF service consumer

The procedure in Figure 6.1B.2.1-1 is used when an NF decides to select a new NWDAF instance due to internal or external triggers, e.g. the NF starts serving a UE with analytics subscription information received upon UE context transfer procedure as described in TS 23.502 [3], or the NF starts to request NF related analytics, or the NF receives a "Termination Request" for an existing analytics subscription from an NWDAF. The NF sends to the target NWDAF information about the NWDAF previously used for analytics subscription, if available, in Nnwdaf_AnalyticsSubscription_Subscribe service operation. The target NWDAF may initiate the transfer of the analytics context, using the Nnwdaf_AnalyticsInfo_ContextTransfer or Nnf_DataManagement_Subscribe service operation.

The procedure in Figure 6.1B.2.1-1 is also used when an Aggregator NWDAF decides to select a new NWDAF to request output analytics for analytics aggregation. For example, upon receiving a Termination Request from one of the NWDAFs that are collectively serving a request for analytics subscription as specified in clause 6.1A, the Aggregator NWDAF queries the NRF or UDM to select a target NWDAF as specified in clause 6.1A.3 using information e.g. the UE location, the 5GC NFs (identified by their NF Set IDs or NF types) serving the UE or to be contacted for data collection (if Area of Interest is not provisioned for the requested analytics), or the subset of AoI (if Area of Interest is provisioned for the requested analytics). Then, the Aggregator NWDAF sends information about the NWDAF previously used for analytics subscription, if available, in Nnwdaf_AnalyticsSubscription_Subscribe service operation towards the selected target NWDAF.

Figure 6.1B.2.1-1: Analytics context transfer initiated by target NWDAF selected by the NWDAF service consumer

1.	The NWDAF service consumer determines to select an NWDAF instance. The consumer discovers and selects the target NWDAF as specified in clause 5.2.

2.	The consumer sends a request for analytics subscription to the target NWDAF using Nnwdaf_AnalyticsSubscription_Subscribe service operation, including information on the previous analytics subscription (i.e. NWDAF ID, Analytics ID(s), SUPIs, Analytics Filter Information for UE-related Analytics, Subscription Correlation ID, the Analytics Accuracy Request information (as defined in Clause 6.1.3) when the Target NWDAF supports accuracy checking capability) which relates to the requested analytics subscription, if available. If the target NWDAF accepts the analytics subscription request, it sends Nnwdaf_AnalyticsSubscription_Subscribe response with a Subscription Correlation ID.

If the target NWDAF does not receive information of previous analytics subscription in step 2, for UE related Analytics, the target NWDAF may discover previously used NWDAF in UDM as specified in clause 5.2.

NOTE 1:	If the selected target NWDAF instance is the same as the source NWDAF instance (as received from the other consumer in step 0), the target NWDAF will update the existing analytics subscription to the new analytics consumer. Following steps are skipped.

NOTE 2:	The consumer can provide information on the previous analytics subscription when, e.g. the consumer is an AMF and it received information from the old AMF, see clause 5.2.2.2.2 of TS 23.502 [3].

3a.	[Option 1] If the target NWDAF decides to request an analytics context transfer from the previously used NWDAF, it may make use of information sent in step 2 (e.g. the provided Subscription Correlation ID) and use the analytics context transfer procedure as specified in clause 6.1B.3. The target NWDAF may receive an ADRF ID or DCCF ID for collecting the historical data and/or analytics.

If the target NWDAF supports the accuracy checking capability and the request received in Step 2 contains the Analytics Accuracy Request information, the target NWDAF may include in the Nnwdaf_AnalyticsInfo_ContextTransfer request the field Requested Analytics Context Type with value set to Analytics accuracy related information (as defined in Clause 6.1B.4) in order to retrieve the necessary information for starting the analytics accuracy information for the transferred analytics subscription(s).

3b-c.	[Option 2] If the target NWDAF decides to only request historical data and/or analytics, then it may collect the data and/or analytics via Nnf_DataManagement_Subscribe service, where the NFs may be either the ADRF, NWDAF or DCCF, as described in clauses 10.2.6, 7.4.2 and 8.2.2 respectively.

Target NWDAF is now ready to generate analytics information, and if applicable analytics accuracy information taking into account the information received in step 3.

4.	[Optional] Source NWDAF may purge analytics context after completion of step 3a, if performed and if not already done, unsubscribes from the data source(s) and/or model source(s) that are no longer needed for the remaining analytics subscriptions.

5.	[Optional] Target NWDAF may subscribe to relevant data source(s) and/or model source(s), if it is not yet subscribed to the data source(s) and/or model source(s).

# * * * * Third change * * * *

6.1B.2.2	Analytics Subscription Transfer initiated by source NWDAF

The procedure in Figure 6.1B.2.2-1 is used by an NWDAF instance to request the transfer of analytics subscription(s) to another NWDAF instance, using the Nnwdaf_AnalyticsSubscription_Transfer service operation defined in clause 7.2.5.

If the source NWDAF discovers that the analytics consumer may change concurrently to this procedure, the source NWDAF should not perform the procedure. In such a case, the source NWDAF may send a message to indicate to the analytics consumer that it will not serve this subscription anymore.

NOTE 1:	To discover the possible change of analytics consumer, if the Analytics ID is UE related, the source NWDAF takes actions responding to external trigger (such as UE mobility), for example, checking if the Target of Analytics Reporting is still within the serving area of the analytics consumer, if the serving area information is available.

NOTE 2:	Handling of overload situation or preparation for a graceful shutdown are preferably executed inside an NWDAF Set, when available, therefore, not requiring an analytics subscription transfer as described in this clause. The procedure in Figure 6.1B.2.2-1 is applicable for analytics subscription transfer across NF Sets or if the NWDAF is not deployed in a Set.

Figure 6.1B.2.2-1: Analytics subscription transfer initiated by source NWDAF

0.	The analytics consumer subscribes to analytics from source NWDAF. The analytics consumer may send its NF ID or serving area, enabling NWDAF to determine whether the following analytics subscription transfer procedure is applicable. Optionally the source NWDAF subscribes to UE mobility events.

1.	[Optional] Source NWDAF determines, e.g. triggered by a UE mobility event notification, to prepare an analytics subscription transfer to target NWDAF(s), as specified in the procedure illustrated in clause 6.1B.2.3.

2.	Source NWDAF determines, e.g. based on the UE location information received and the analytics consumer's serving area either directly received in step 0 or indirectly received via NRF, to perform an analytics subscription transfer to target NWDAF(s). Therefore, the source NWDAF determines the analytics subscription(s) to be transferred to a target NWDAF.

3.	Source NWDAF performs an NWDAF discovery and selects the target NWDAF. NWDAF discovery may be skipped if the target NWDAF had already been discovered as part of a prepared analytics subscription transfer. In the case of aggregated analytics from multiple NWDAFs, the source NWDAF may use the set of NWDAF identifiers related to aggregated analytics (see clause 6.1.3) to preferably select a target NWDAF that is already serving the consumer. If the analytics subscription to be relocated to a target NWDAF also includes the Analytics Accuracy Request information, the source NWDAF selects, if possible, a target NWDAF also with accuracy checking capability. If the source NWDAF does not discover a target NWDAF with accuracy checking capability, the source NWDAF notifies the analytics consumer with Accuracy Information Termination. Therefore, the consumer based on local policy may decide to unsubscribe the analytics ID or to keep using the analytics ID even without receiving the accuracy information.

4.	Source NWDAF requests, using Nnwdaf_AnalyticsSubscription_Transfer Request service operation, a transfer of the analytics subscription(s) determined in step 2 to the target NWDAF. The request contains a callback URI of the analytics consumer. The request may also contain active data source ID(s) and ML model related information, which are related to the analytics subscriptions requested to be transferred, if not already provided as part of the prepared analytics subscription transfer in the preparation procedure (see step 1). The ML model related information contains the ID(s) of NWDAF(s) containing MTLF that provided the trained models and may contain the file address(es) of the trained ML model(s), where the file address(es) of the trained ML model(s) is included only when the source NWDAF itself provides the trained ML model(s) for the analytics subscription(s) being transferred. The request message may also include "analytics context identifier(s)" indicating the availability of analytics context for particular Analytics ID(s).

5.	Target NWDAF accepts the analytics subscription transfer and takes over the analytics generation, and if applicable the analytics accuracy information generation based on the information received from the source NWDAF.

The target NWDAF may use analytics accuracy request information included in the analytics subscription transfer received in Step 4 to start the process of checking and generating analytics accuracy information for the consumer of the transfer analytics subscription.

Target NWDAF may use the ML model related information, if provided in the Nnwdaf_AnalyticsSubscription_Transfer request. If the ID(s) of NWDAF(s) containing MTLF is provided in the Nnwdaf_AnalyticsSubscription_Transfer request and the NWDAF(s) containing MTLF is part of the locally configured set of NWDAFs containing MTLF, target NWDAF may request or subscribe to the ML model(s) from the indicated NWDAF(s) containing MTLF as specified in clause 6.2A and use the ML model(s) for the transferred analytics subscription. If the file address(es) of the trained ML model(s) is provided and if the NWDAF containing MTLF is part of the locally configured set of NWDAFs containing MTLF, the target NWDAF may retrieve the ML model using the file address of the trained ML model. If the provided ID(s) of NWDAF(s) containing MTLF are not part of the locally configured set of ID(s) of NWDAFs containing MTLF, the target NWDAF discovers the NWDAF(s) supporting MTLF that can provide trained ML model(s) for the Analytics ID(s) as described in clause 5.2.

NOTE 3:	If not yet done during a prepared analytics subscription transfer, the target NWDAF allocates a new Subscription Correlation ID to the received analytics subscriptions.

NOTE 4:	The target NWDAF might already have received information on some/all of the analytics subscriptions as part of the prepared analytics subscription transfer request received in step 1 and, thus, might already have started to prepare for the analytics generation, e.g. by having already subscribed to relevant event notifications.

6.	Target NWDAF informs the analytics consumer about the successful analytics subscription transfer using a Nnwdaf_AnalyticsSubscription_Notify message. A new Subscription Correlation ID, which was assigned by the target NWDAF, is provided in the Subscription Correlation ID and the old Subscription Correlation Id, which was allocated by the source NWDAF, is provided in the Subscription Change Notification Correlation ID parameter of this message as specified in clause 7.2.4.

NOTE 5:	Notification correlation information in the Nnwdaf_AnalyticsSubscription_Notify message allows the analytics consumer to correlate the notifications (of analytics output and if applicable of analytics accuracy information) to the initial subscription request made with the source NWDAF in step 0.

NOTE 6:	The existing Analytics context in the source NWDAF is not deleted directly but will be purged first when it was collected by the target NWDAF.

NOTE 7:	If this subscription is used as input for analytics aggregation by the analytics consumer, the analytics consumer might inform the other NWDAFs instance participating in this analytics aggregation that the Set of NWDAF identifiers of NWDAF instances used by the NWDAF service consumer for this analytics aggregation (see clause 6.1.3) has changed using the Nnwdaf_AnalyticsSubscription_Subscribe service operation.

7.	[Conditional] If "analytics context identifier(s)" had been included in the Nnwdaf_AnalyticsSubscription_Transfer Request received in step 4, the target NWDAF requests the "analytics context". The analytics context transfer procedure is specified in clause 6.1B.3.

If the transfer request received by the target NWDAF also includes the Analytics Accuracy Request information, the target NWDAF will include in the Nnwdaf_AnalyticsInfo_ContextTransfer request the field Requested Analytics Context Type with value set to Analytics accuracy related information (as defined in Clause 6.1B.4) in order to retrieve the necessary information for generating the analytics accuracy information for the transferred analytics subscription(s).

8.	[Optional] Target NWDAF subscribes to relevant data source(s), if it is not yet subscribed to the data source(s) for the data required for the Analytics.

9.	Target NWDAF confirms the analytics subscription transfer to the source NWDAF.

10.	[Optional] Source NWDAF unsubscribes with the data source(s) that are no longer needed for the remaining analytics subscriptions. In addition, Source NWDAF unsubscribes with the NWDAF(s) containing MTLF, if exist, which are no longer needed for the remaining analytics subscriptions.

NOTE 8:	At this point, the analytics subscription transfer is deemed completed, i.e. the source NWDAF can delete all information related to the successfully transferred analytics subscription.

11-12.	Target NWDAF at some point derives new output analytics and analytics accuracy information (if applicable) based on new input data and notifies the analytics consumer about the new analytics and new analytics accuracy information (if applicable) using a Nnwdaf_AnalyticsSubscription_Notify message as specified in clause 6.1.1.

# * * * * Fourth change * * * *

6.1B.2.3	Prepared analytics subscription transfer

The procedure in Figure 6.1B.2.3-1 is used by an NWDAF instance to request another NWDAF instance for a prepared analytics subscription transfer from the source NWDAF instance, using the Nnwdaf_AnalyticsSubscription_Transfer service operation defined in clause 7.2.5.

NOTE 1:	The source NWDAF might determine that it needs to prepare to transfer analytics to another NWDAF instance, e.g. when the source NWDAF estimates for UE related analytics subscription that the UE might enter an area which is not covered by the source NWDAF (e.g. by subscribing to AMF event exposure service for UE mobility event notifications, by performing UE mobility analytics, or by subscribing to another NWDAF providing UE mobility analytics). If the source NWDAF discovers that the analytics consumer may change concurrently to this procedure, the source NWDAF does not perform the procedure. If the procedure makes use of predictions to determine the candidate NWDAFs, care must be taken with regards to load and signalling cost when sending data to an NWDAF that will not eventually start serving the UE.

NOTE 2:	The source NWDAF might also determine that it needs to prepare to transfer analytics subscriptions to another NWDAF instance, as the source NWDAF wants to resolve an internal load situation or prepare for a graceful shutdown.

NOTE 3:	Handling of overload situation or preparation for a graceful shutdown are preferably executed inside an NWDAF Set, when available.

Figure 6.1B.2.3-1: Prepared analytics transfer

0.	Analytics consumer subscribes to the source NWDAF for certain analytics as specified in clause 6.1.1.

0a.	Source NWDAF starts data collection from relevant data source(s) (e.g. NFs or OAM) as specified in clause 6.2. Source NWDAF starts generating requested analytics.

0b.	[Conditional] (Only if the source NWDAF does not serve the whole PLMN and the requested analytics involves UE related data) The source NWDAF subscribes, using Namf_EventExposure_Subscribe service operation, to receive notifications on UE mobility events from AMF.

1.	The source NWDAF determines that it needs to prepare to transfer analytics to another NWDAF instance.

2.	The source NWDAF discovers candidate target NWDAF instances (e.g. NWDAFy and NWDAFz) supporting the requested analytics information for the predicted target area(s). NWDAF discovery and selection is specified in clause 6.3.13 of TS 23.501 [2]. In the case of aggregated analytics from multiple NWDAFs, the source NWDAF may use the set of NWDAF identifiers related to aggregated analytics (see clause 6.1.3) to preferably select a target NWDAF that is already serving the consumer. In case of the analytics subscription to be transferred also include an Analytics Accuracy Request information as defined in clause 6.1.3, the source NWDAF takes into consideration in the selection process whether target NWDAF have the accuracy checking capability.

NOTE 4:	In this procedure, NWDAFy and NWDAFz are examples for target NWDAF instances that are candidates to take over those analytic subscriptions.

3-4.	In the case of a prepared analytics subscription transfer, the source NWDAF requests, using Nnwdaf_AnalyticsSubscription_Transfer Request, to the candidate target NWDAFs (e.g. NWDAFy and NWDAFz) to prepare for an analytics subscription transfer by including a "prepared analytics subscription transfer indication" in the request message. The request message includes information on the analytics subscriptions to be transferred. The request message may also include "analytics context identifier(s)" indicating the availability of analytics context for particular Analytics ID(s).

The candidate target NWDAFs (e.g. NWDAFy and NWDAFz) respond to the request from the source NWDAF using a Nnwdaf_AnalyticsSubscription_Transfer Response message.

5-6.	[Conditional] If "analytics context identifier(s)" had been included in the Nnwdaf_AnalyticsSubscription_Transfer Request received in step 4, the determined target NWDAFs (e.g. NWDAFy and NWDAFz) may request the "analytics context" from the source NWDAF by invoking the "Nnwdaf_AnalyticsInfo_ContextTransferservice" operation. The analytics context transfer procedure is specified in clause 6.1B.3.

If the target NWDAF supports the accuracy checking capability and the request received in Step 3-4 contains the Analytics Accuracy Request information, the target NWDAF includes in the Nnwdaf_AnalyticsInfo_ContextTransfer request the field Requested Analytics Context Type with value set to Analytics accuracy related information (as defined in Clause 6.1B.4)  in order to retrieve the necessary information for starting the analytics accuracy information for the transferred analytics subscription(s).

NOTE 5:	The target NWDAFs (e.g. NWDAFy and NWDAFz) can allocate a new Subscription Correlation ID to the received analytics subscriptions.

7.	[Optional] Based on the information received from the source NWDAF, the target NWDAFs (e.g. NWDAFy and NWDAFz) start data collection from NFs or OAM (as specified in clause 6.2), analytics generation for the indicated analytics subscriptions, and if applicable the analytics accuracy checking and generation (as specified in Clause 6.2D).

NOTE 6:	After step 7, the source NWDAF initiates the analytics subscription transfer to the target NWDAF as specified in steps 4 to 12 of the analytics subscription transfer procedure illustrated in Figure 6.1B.2.2-1.

8.	The source NWDAF cancels the prepared analytics subscription transfer to a candidate target NWDAF (e.g. NWDAFz), using Nnwdaf_AnalyticsSubscription_Transfer Request include an "analytics subscription transfer cancel indication". The target NWDAF (e.g. NWDAFz) confirms the cancelation to the source NWDAF and, if applicable, deletes any analytics data that is no longer needed. If the target NWDAF (e.g. NWDAFz), as part of the analytics subscription preparation, had already subscribed to entities to collect data or acquire ML model, it unsubscribes to those entities if the subscriptions are not needed for other active analytics subscriptions with the target NWDAF. If the candidate NWDAF (e.g. NWDAFz), as part of the analytics subscription preparation, had already started the analytics accuracy generation (as specified Clause 6.2D), it cancels the generation and if any extra data collection has been started, this data collection is also stopped, if not used by any other process in the target NWDAF.

Step 8 may take place any time after step 4 if the NWDAF determines that the candidate target NWDAF (e.g. NWDAFz) does no longer need to prepare for the analytics subscription transfer. In particular, the source NWDAF shall cancel the prepared analytics subscription transfer to all remaining candidate target NWDAFs after one target NWDAF has accepted the analytics subscription transfer (see NOTE 6).

If the source NWDAF is not able to discover a target NWDAF with accuracy checking capability, the source NWDAF notifies the analytics consumer with Accuracy Information Termination. Therefore, the consumer may decide to unsubscribe the analytics ID or to keep using the analytics ID even without receiving the accuracy information.

# * * * * Fifth change * * * *

6.1B.3	Analytics Context Transfer

The procedure depicted in Figure 6.1B.3-1 is used by an NWDAF instance to request analytics context from another NWDAF instance, using the Nnwdaf_AnalyticsInfo_ContextTransfer service operation as defined in clause 7.3.3. This procedure, for example, can be invoked in the procedures described in clause 6.1B.2 to request the transfer of relevant analytics context.

Figure 6.1B.3-1: Analytics Context Transfer

The procedure of analytics context information transfer comprises the following steps:

1.	The consumer NWDAF requests analytics context by invoking Nnwdaf_AnalyticsInfo_ContextTransfer request service operation. The parameters that can be provided in the request are listed in clause 6.1B.4.

2.	The provider NWDAF responds with analytics context to the consumer NWDAF. The analytics context that can be provided in the response is listed in clause 6.1B.4.

If the provider NWDAF stores analytics context (i.e. Historical output Analytics and/or Data related to Analytics) in ADRF, the provider NWDAF may include in the response the ADRF ID together with an indication of the Analytics Context Type stored in the ADRF (i.e. Historical output Analytics and/or Data related to Analytics).

Upon receiving the analytics context, the consumer NWDAF may:

-	provide the pending output analytics or historical analytics information to the analytics consumer per the subscription/request;

-	use the historical data and analytics metadata in the analytics context to generate analytics;

-	use the analytics accuracy related information in the analytics context to activate the checking of analytics accuracy information for the transferred analytics ID, generate and provide the analytics accuracy information for the consumer; the consumer NWDAF may analyse the timestamps of the historical data included in the analytics context in order to obtain the inference configuration used at the source NWDAF for data collection and may decide to use the same inference configuration for the analytics accuracy generation.

-	subscribe to data collected for analytics with the data sources indicated in the analytics context;

-	if the ID(s) of the NWDAF(s) containing MTLF indicated in the analytics context is part of the locally configured (set of) IDs of NWDAFs containing MTLF, retrieve trained ML model(s) from the indicated NWDAF(s) containing MTLF or based on the file address(es) of the trained ML model(s) and use for analytics; and/or

-	subscribe to output analytics from the indicated NWDAFs that collectively serve the transferred analytics subscription and perform analytics aggregation on the output analytics using the analytics metadata information, based on the analytics subscription aggregation information.

# * * * * Sixth change * * * *

6.1B.4	Contents of Analytics Context

The Nnwdaf_AnalyticsInfo_ContextTransfer service operation is used to transfer analytics context from a source NWDAF instance to a target NWDAF instance, whereby the target NWDAF consumes the Nnwdaf_AnalyticsInfo_ContextTransfer service operation produced by the source NWDAF instance to request the analytics context as depicted in Figure 6.1B.3-1.

The consumers of the Nnwdaf_AnalyticsInfo_ContextTransfer service operation (as specified in clause 7.3.3) provide the following input parameters:

-	A list of analytics context identifier(s): identify a set of analytics context that are available at the NWDAF instance providing this service and that are requested to be transferred to the consumer NWDAF instance. The analytics context identifier is provided as the following:

-	Subscription Correlation ID: identifies the analytics subscription for which the related analytics context is requested; or

-	A set of SUPI and associated Analytics ID for UE related Analytics; or

-	An Analytics ID for NF related Analytics.

-	[OPTIONAL] Requested Analytics Context Type per analytics context identifier: indicates which part of the analytics context the consumer wishes to receive. Following values are specified:

-	Pending output Analytics;

-	Historical output Analytics;

-	Analytics subscription aggregation information;

-	Data related to Analytics;

-	Aggregation related information;

-	ML Model related information;

-	Analytics accuracy related information.

NOTE:	A list of "analytics context identifier(s)" can be provided by the source NWDAF to the target NWDAF in an analytics subscriptions transfer request as described in clause 6.1B.2.2. Information allowing to identify an analytics context can also be provided by the NWDAF consumer to the target NWDAF in the Nnwdaf_AnalyticsSubscription_Subscribe request and based on this information the target NWDAF derives the "analytics context identifier", as defined in clause 6.1B.2.1.

The producer NWDAF provides to the consumer of the Nnwdaf_AnalyticsInfo_ContextTransfer service operation (as specified in clause 7.3.3), the output information listed below:

-	(Set of) Analytics context matching the input parameters of the Nnwdaf_AnalyticsInfo_ContextTransfer request. If no Requested Analytics Context type parameters are available in the request, all available analytics context types are sent. Analytics context includes the following information parts, if available:

-	Analytics related:

-	Pending output analytics (i.e. not yet notified to the consumer).

-	Historical output analytics information. The content of the output analytics is specified in clause 6.1.3 as output information of the Nnwdaf_AnalyticsSubscription_Notify or Nnwdaf_AnalyticsInfo_Request service operations.

-	Timestamp(s) of the last output analytics provided to the analytics consumer(s). Value is set to 0 if no output analytics had been sent yet.

-	Analytics subscription aggregation information (only provided when analytics context is related to analytics aggregation): information about the analytics subscriptions that the source NWDAF has with the NWDAFs that collectively serve the transferred analytics subscription, which includes IDs and analytics metadata information of these NWDAFs for specific Analytics ID(s) and optionally input parameters of analytics exposure as defined in clause 6.1.3.

-	Data related to Analytics:

-	Historical data that is available at the source NWDAF and that is related to the analytics to be handed over to the target NWDAF. If available, the time period of the collected data, NF ID(s) of the data source(s) and information (e.g. filter and event reporting parameters) on the subscriptions with those data sources which were used to generate this historical data.

-	Aggregation related information: Related to analytic consumers that aggregate analytics from multiple NWDAF subscriptions:

-	(Set of) NWDAF identifiers of NWDAF instances used by the NWDAF service consumer when aggregating multiple analytic subscriptions.

-	ML Model related information:

-	ID(s) of NWDAF(s) containing MTLF: Instance/Set ID(s) of the NWDAF(s) containing MTLF from which the source NWDAF currently subscribes to the ML Model Information used for the analytics.

-	Optionally, file address(es) of the trained ML model(s), which is included only when the source NWDAF itself provides the trained ML model(s) for the analytics subscription(s) for which the related analytics context is requested.

-	Analytics Accuracy related information: The information is related to the parameters of the Analytics Accuracy Request Information and status of the analytics subscription due to NF consumer and NWDAF interactions pausing or resuming the analytics output. It includes:

-	Timestamp(s) of the last analytics accuracy information provided to the analytics consumer(s). Value is set to 0 if no analytics accuracy information had been sent yet;

-	Indication whether analytics subscription is paused;

-	Remaining time window of paused analytics subscription;

-	Ground truth information: data types and data sources of the ground truth per analytics used for the accuracy information computation.

# * * * * Seventh change * * * *

7.2.4	Nnwdaf_AnalyticsSubscription_Notify service operation

Service operation name: Nnwdaf_AnalyticsSubscription_Notify.

Description: NWDAF notifies the consumer instance of the analytics that has subscribed to the specific NWDAF service. See also clause 6.1.3 for contents of the Analytics Exposure.

Inputs, Required: Notification Correlation Information: this parameter indicates the Notification Correlation Id that has been assigned by the consumer during analytics subscription.

Inputs, Optional:

-	Set of the tuple (Analytics ID, Analytics specific parameters): this parameter shall be present if output analytics are reported.

-	Timestamp of analytics generation (required when ADRF is deployed).

-	Validity period.

-	Confidence

-	Revised waiting time.

-	Analytics Metadata Information.

-	Termination Request: this parameter indicates that NWDAF requests to terminate the analytics subscription, i.e. NWDAF will not provide further notifications related to this subscription, with cause value (e.g. user consent revoked, NWDAF overload, UE moved out of NWDAF serving area, etc.).

-	Accuracy Information Termination.

NOTE:	Validity period can also be provided as part of Analytics specific parameters for some NWDAF output analytics.

-	Subscription Change Notification Correlation ID: this parameter shall be present if the notification is for informing the assignment of a new Subscription Correlation Id by the NWDAF. It is set to the old Subscription Correlation ID.

-	Subscription Correlation ID: this parameter shall be present if the notification is for informing the assignment of a new Subscription Correlation Id by the NWDAF. It is set to a new Subscription Correlation ID assigned by the NWDAF.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

# * * * * Eighth change * * * *

7.2.5	Nnwdaf_AnalyticsSubscription_Transfer service operation

Service operation name: Nnwdaf_AnalyticsSubscription_Transfer.

Description: Requests to NWDAF for transferring analytics subscription(s) from the consumer NWDAF.

Inputs, Required:

-	Transfer type: indicates the type of the transfer request. The following values are supported:

-	Analytics subscription transfer preparation: requests the NWDAF to prepare for taking over the analytics subscription(s) and/or prepare for collecting Analytics Context(s).

-	Analytics subscription transfer: requests the NWDAF to take over the analytics subscription(s).

-	Analytics subscription transfer cancel: cancels a prepared analytics subscription request.

Inputs, Optional:

-	If this service operation is for "analytics subscription transfer preparation", the following parameter shall be provided:

-	(Set of) analytics subscription information with the following parameters:

-	All input parameters for the analytics exposure as specified in clause 6.1.3.

-	[OPTIONAL] Active data source ID(s): Instance ID or Set ID of the active data source(s) the consumer NWDAF is currently using for the analytics of this analytics subscription.

-	[OPTIONAL] ML Model related information, i.e. information related to the ML model(s) that the NWDAF is currently using for the analytics:

-	ID(s) of NWDAF(s) containing MTLF: Instance ID(s) of the NWDAF(s) containing MTLF from which the consumer NWDAF currently subscribes to the ML model information used for the analytics.

-	[OPTIONAL] File address of the ML model(s).

-	ID of the analytics consumer, e.g. NF, AF or OAM, that is subscribed to receive analytics.

-	[OPTIONAL] (Set of) analytics context identifier(s): identifies analytics context available at the consumer NWDAF as defined in clause 6.1B.4.

-	If this service operation is to request analytics subscriptions transfer and if analytics subscription transfer preparation is not performed, the same parameters as those for transfer type "Analytics subscription transfer preparation" shall be provided.

-	If this service operation is to request analytics subscriptions transfer and if analytics subscription transfer preparation is performed, the same parameters as those for transfer type "Analytics subscription transfer preparation" can be provided with updated parameter values.

-	If this service operation is to request analytics subscriptions transfer cancel, the following parameter shall be provided:

-	Subscription Correlation ID.

Outputs Required: Operation execution result indication.

Outputs, Optional: None.

# * * * * End of changes * * * *