3GPP TSG-WG SA2 Meeting #157	S2-2307091

May 22 – 26, 2023, Berlin, DE	(revision of S2-2304440)

Title:	[DRAFT] ReplyLS on API enhancement for GMEC services and AIMLSys services

Response to:	LS (S2-2306275/C3-230783) on API enhancement for GMEC services and AIMLSys services from CT3

Release:	Rel-18

Work Item:	GMEC, AIMLSys

Source:	[Huawei to be] SA2

To:	CT3

Cc:

Contact Person:

Name:	Qianghua Zhu

Tel. Number:

E-mail Address:	zhuqianghua AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

SA2 would like to thank 5G-ACIA for their LS (S2-2306275/C3-230783) and would like to provide the following reply:

CT3 Questions:

- Whether SA2 is planning to merge these two APIs into a single one?

- What is the common part and/or difference between the Nnef_AFRequestForQoS service API and the Nnef_MultiMemberAFsessionWithQoS service API?

SA2 reply:

SA2 has discussed whether to merge AFRequestForQoS service API and MultiMemberAFsessionWithQoS service API, as well as whether to merge with other NEF services, e.g. AFsesseionWithQoS, ParameterProvision, ServiceParameter, and TrafficInfluence, and SA2 agrees to keep GMEC AFRequestForQoS service API and AIMLsys MultiMemberAFsessionWithQoS service API separate given the comparison summarized in the below table. SA2 would also like to ask CT3 to provide the identified technical concerns to specify separate APIs, if any.

The table below shows the summary of comparison:

|  | Separate NEF Service |  | Common NEF service |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | AFRequestForQoS | MultiMemberAFsessionWithQoS | AFSessionWithQoS | ParameterProvision | ServiceParameter | TrafficInfluence |
| Target UE | Group ID or GPSI | list of UE IP addresses | UE address | UE address, GPSI, Group ID | UE address, GPSI, Group ID | UE address, GPSI, Group ID, any UE |
| Support Future PDU Session | Yes | No | No | Yes | Yes | Yes |
| Multiple memberships? | Yes (one per PDU Session) | Yes (multiple per PDU Session) | No | Yes | Yes | Yes |
| Area/scope | Regional area/factory | Whole PLMN | Whole PLMN | Regional area/factory/Whole PLMN | Regional area/factory/Whole PLMN | Regional area/factory |
| Procedure dependency | TSCTSF required | None | TSCTSF required, depends | Yes, depends | UDR required | UDR required |
| Group control | TSCTSF or PCF | NEF | None | UDM | UDR | UDR |
| QoS type | Both GBR and non-GBR | Individual GBR QoS | Both GBR and non-GBR | None | None | None |
| Flow description | Communication between UEs or between UE and DN, 3 tuples | Communication only between UE and DN, 5 tuples | Communication only between UE and DN, 5 tuples | None | None | Communication only between UE and DN, 5 tuples |
| Update frequency | rare | Frequent (wrt. current members, QoS, monitoring) | Frequent (wrt. QoS, monitoring) | rare | rare | Frequent (wrt. Traffic routing) |
| commonalities | Certain similarity in functionality and commonality of some parameters (i.e. for an individual UE‘s PDU Session QoS and QoS monitoring is controlled) |  |  |  |  |  |
| Specifics | Traffic characteristics, Temporal invalidity, Group Data Rate | Consolidated bitrate monitoring | XRM support, TSC parameters etc. | UE related parameters | Service specific parameters | Traffic routing parameters |
| Summary | Straightforward for different area application developers, simple and clear specification |  | Integrated NEF functionalities applicable to all different applications, NEF needs validation of the correctness for parameter combos for a given application and invoke different backend procedures accordingly, every developer has to learn all the parameters or procedures even none of their interest, complex specification mixed of different features and use cases and this becomes worse in future evolution of the API when even more scenario specific functionality and parameters have to be supported |  |  |  |

2. Actions:

To CT3 group.

ACTION: 	SA2 asks CT3 group to take the above responses into account

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #158		August 21 – 25, 2023			Goteborg, SE

TSG-SA2 Meeting #159		October 09 – 13, 2023			Xiamen, CN