3GPP TSG-WG SA2 Meeting #157	S2-2307134

# Berlin, Germany, May 22 – 26, 2023		(revision of S2-2306065)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1284 | rev | 04 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Supporting UE centric AR Telephony Communication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | [Ericsson,] Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC |  |  |  |  |  | Date: |  |  | 2023-05-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | WID NG_RTC has objective to support AR telephony communication in IMS network, which has been concluded in TR 23.700-87. This CR reflects the TR conclusions. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new Annex for both the UE centric AR Architecture and network centric AR architecture based on the DC architecture. This CR includes the network centric architecture. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete Specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex AC.X(new), AC.X.1, AC.X.2, AC.X.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Change for SA2 157 meeting: - Adding procedure for network rendering. - Clarify what is AR media in the old text. - Use a unified architecture for both UE centric and network centric scenario. - Some editorial correction. |  |  |  |  |  |  |  |  |

# * * * * First change * * *

# AC.X Support of AR Communication

# AC.X.1 General

This annex describes the enhancements to the IMS architecture supporting data channel services to additionally support AR services. Two modes of AR services are supported; UE centric and network centric.

## AC.X.2 Architecture

Figure AC.X.2.1-1: Architecture to support AR communication

AR communication architecture is enhanced based on data channel architecture, including the following network functions enhancement:

MF/MRF:

-	The MF/MRF receives and stores AR service media handling logic from the DC AS, and provides AR media rendering function based on the AR metadata and audio/video media streams received from UE.

IMS AS:

-	The IMS AS receives the media control instructions from the DCSF and accordingly interacts with the UE for connecting the UE’s audio/video media termination to the MF.

# AC.X.3 Procedures

## AC.X.3.1 UE Centric Procedure

Figure AC.X.3.1-1: Establishment of UE Centric AR communication session

Figure AC.X.3.1-1 depicts a typical call flow procedure to establish a UE centric AR IMS session from UE-A perspective. The main steps in the call flow are as follows:

1.	UE-A initiates an IMS communication with UE-B, including establishment of bootstrap data channel.

2.	The user of UE-A upgrades the IMS session with AR experience. 
UE-A initiates a re-INVITE adding media descriptors required by the AR application to be established E2E, e.g. RTP and/or application data channels. Application data channel may be anchored in the MF/MRF.

In this scenario, the UE performs AR media rendering based on the AR metadata from its local application.

3.	UE-A captures AR metadata, performs AR media rendering locally and then encodes to audio/video media stream.

4.	UE-A sends the audio/video media stream to the peer through the established media connection(s).

5.	UE-A receives AR metadata from the peer through established media connection(s).

6.	UE-A decodes the AR metadata and performs AR media rendering locally and displays on its screen.

Figure AC.X.3.1-1: Establishment of UE Centric AR communication session

## AC.X.3.2 Network Centric Procedure

Figure AC.X.3.2-1 depicts a typical call flow procedure to establish a network centric AR IMS session from UE-A perspective. The main steps in the call flow are as follows:

Figure AC.X.3.2-1: Establishment of Network Centric AR communication session

1.	The UE-A initiates a session and establishes audio and video session connections with the UE-B. The bootstrap data channel(s) are established at the same time for both the UE-A and UE-B.

AR Media Rendering Negotiation Procedure:

2.	The UE-A decides to request network media rendering based on its status such as power, signal, computing power, internal storage, etc.

3.	The UE-A finishes AR media rendering negotiation with the DC AS.

NOTE: The media negotiation for split rendering between the UE and the network will be specified by SA WG4.

4.	If the negotiation result is successful in step 3, the UE-A initiates new P2A application data channels, which are used for AR metadata transmission between UE-A and MF.

5(Optional). IMS initiates a Media re-Negotiation request with UE-A, to connect UE-A’s audio/video media stream to MF.

6. 	IMS initiates a Media re-Negotiation request with UE-B, to connect UE-B’s audio/video media stream to MF.

The UE-A can request to change the network rendering content if the status changed in the UE-A, such as low power, low computing power etc.

Networking Centric Procedure:

7.	The UE-A starts AR media rendering.

8.	The UE-A sends AR metadata through the dedicated application data channel(s) which are established in step 4 to the MF.

9.	Based on the AR metadata received from the UE-A, the MF performs the parts of AR media rendering according to the negotiation result in step 3.

10(Optional).	The UE-A performs the parts of the AR media rendering itself according to the negotiation result in step3.

11-12.	The UE-A sends the rendered audio/video media over RTP to the MF through the IMS-AGW. If the UE-A does not perform the media rendering, the UE-A sends the original audio/video media stream over RTP to the MF.

13-14.	The MF combines the media rendered by the MF and the audio/video media stream received from the UE-A, and sends the audio/video media stream over RTP to the UE-B. If the UE-A does not perform the rendering itself, step 13 can be omitted, and the MF directly sends the rendered audio/video media stream to the UE-B.

15(Optional).	The MF sends the rendered audio/video media stream to the UE-A when UE-A needs overlay video through the video back channel.

# * * * * End of changes * * * *