3GPP TSG-SA WG2 Meeting #157	S2-2307180

# Berlin, Germany, 22 – 26 May, 2023		(revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0154 | rev | - | Current version: | 18.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | V-EASDF handling during UE mobility |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGE_Ph2 |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This contribution suggests to clarify for each intra/inter PLMN mobility scenario with respect to V-EASDF and DNS context handling and HR-SBO re-authorization. The following Editor's note to be resolved: “Editor’s Note: whether the above EASDF removal applies to intra PLMN mobility is FFS.” |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify whether and how the V-EASDF and DNS context is handled for intra/inter PLMN mobility scenario with V-SMF insertion/change/removal cases. - Clarify how HR-SBO authorization is performed for both intra/inter PLMN mobility scenario. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not clear how for each UE mobility scenario the V-EASDF with DNS context is handled and HR-SBO is authorized to support EAS Re-discovery procedure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS 23.502 CR 4060 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

6.7.3.2	EAS Re-discovery

Figure 6.7.3.2-1 shows the procedure of EAS re-discovery in VPLMN for HR roaming.

Figure 6.7.3.2-1: EAS rediscovery procedure

1.	For UE mobility triggered EAS rediscovery, step 4 is skipped.

The procedure described in clause 4.23.7 (N2 based handover) or 4.23.11 (Xn based handover) of TS 23.502 [3] is performed by replacing I-SMF with V-SMF and SMF with H-SMF.

In the case of V-SMF insertion or V-SMF change, when the AMF sends Nsmf_PDUSession_CreateSMContext Request to the target V-SMF, the AMF includes HR-SBO allowed indication if received from the UDM.

1a.	(V-SMF insertion case) The (target) V-SMF retrieves SM context from H-SMF using Nsmf_PDUSession_Context Request/Response.

1b.	(V-SMF change case) The target V-SMF retrieves SM context from the source V-SMF using Nsmf_PDUSession_Context Request/Response.

In the case of intra PLMN mobility, the SM context includes Authorization Result for HR-SBO and VPLMN Specific Offloading Policy in step 1b.

In the case of inter PLMN mobility, the target V-SMF shall request HR-SBO to be authorized for the target PLMN to the H-SMF in step 3 including the Serving Network (serving PLMN ID) if HR-SBO allowed indication was received from the AMF.

The SM context also includes EAS information to be refreshed for EAS re-discovery. The EAS information corresponds to the old target DNAI if it has been inserted by the H-SMF (V-SMF insertion case) or the source V-SMF (V-SMF change case).

In the case of V-SMF insertion or V-SMF change, when the AMF sends Nsmf_PDUSession_CreateSMContext Request to the target V-SMF, the AMF includes HR-SBO allowed indication if received from the UDM.

Step 1 is skipped if UE moves without V-SMF change.

Steps 1-6 are skipped if UE moves with V-SMF removal.

After the handover procedure, the (target) V-SMF may perform UL CL/BP and local PSA insertion/change/removal as described in clause 4.23.9 of TS 23.502 [3] by replacing I-SMF with V-SMF and SMF and H-SMF. Differences are shown as steps 2 to 5.

NOTE 1:	When there are other V-UPF(s) between UL CL/BP and H-UPF, the (target) V-SMF sets up user plane between this ULCL/BP and the existing V-UPF.

2.	The (target) V-SMF/H-SMF (re-)selects a new (V-)EASDF, e.g. based on the DNAI corresponding to the target UE location. The (target) V-SMF/H-SMF obtains the new (V-)EASDF address using Neasdf_DNSContextCreate service or based on local configuration.

In the case of inter PLMN mobility, the target V-SMF selects a new V-EASDF and configure DNS handling rules for V-SMF change or the H-SMF selects a new EASDF in the HPLMN for V-SMF removal case.

In the case of intra PLMN mobility, the (target) V-SMF may select a new V-EASDF with/without V-SMF change and configure DNS handling rules.

NOTE 2:	Re-selecting the old V-EASDF to reuse the existing DNS context in the case of intra PLMN mobility is not supported in this release of the specification.

3.	The (target) V-SMF invokes Nsmf_PDUSession_Update Request (EAS rediscovery indication, EAS information to be refreshed for EAS re-discovery (such as FQDN range and IP range corresponding to the old local part of DN) if available, the optional new V-EASDF address, HR-SBO request indication, serving PLMN ID in the case of inter PLMN mobility) to the H-SMF.

In the case of inter PLMN mobility, the H-SMF authorizes the serving PLMN for HR-SBO based on SM subscription data if HR-SBO request indication and serving PLMN ID is received from the source V-SMF.

For the case of local PSA removal, the request message is sent to the H-SMF without including any DNAI.

The EAS information to be refreshed for EAS re-discovery may correspond to:

-	the new target DNAI in case of local PSA insertion.

-	the new target DNAI and old target DNAI in case of local PSA change.

-	the old target DNAI in case of local PSA removal or local PSA is unchanged.

The DNS context in the old (V-)EASDF is removed by the (source) V-SMF/H-SMF using Neasdf_DNSContext_Delete service when the resource and/or the SM context is released during UE mobility.

In the case of inter PLMN mobility, the following apples:

-	(V-SMF insertion case) The H-SMF removes the DNS context in the EASDF in the HPLMN after the H-SMF receives the Nsmf_PDUSession_Update request to release the resource.

-	(V-SMF change or removal case) The (source) V-SMF removes the DNS context in the old V-EASDF along with the SM context after the (Source) V-SMF receives the Nsmf_PDUSession_ReleaseSMContext request.

In the case of intra PLMN mobility with/without V-SMF change, the (source) V-SMF removes the DNS context in the old V-EASDF if a new V-EASDF is selected in step 2.

4a.	For AF triggered EAS rediscovery via interacting with HPLMN, the AF may indicate the EAS rediscovery for the impacted applications, which are identified by Application Identifier(s), to the H-SMF via the H-PCF using the AF influence on traffic routing procedure as described in clause 4.3.6 of TS 23.502 [3].

4b.	For AF triggered EAS rediscovery via interacting with VPLMN, the AF may indicate the EAS rediscovery for the impacted applications via the V-NEF using the procedure described in clause 4.3.6 of TS 23.502 [3].

5.	(V-SMF insertion or change case) The H-SMF initiates Nsmf_PDUSession_Update Request towards the (target) V-SMF including the PCO.

The PCO includes EAS rediscovery indication (optional), impact field (optional) and V-EASDF address (optional).

6.	(V-SMF insertion or change case) The V-SMF initiates PDU Session Modification command including the PCO to the UE.

7.	(V-SMF removal case) The (H-)SMF selects an EASDF in HPLMN and creates a context for the PDU session in that EASDF and then initiates PDU Session Modification command including PCO to the UE.

The PCO includes EAS rediscovery indication (optional) and the impact field (optional).

Editor's note:	It is FFS whether H-SMF needs to contact UDM to retrieve UE subscription during inter-PLMN handover.

* * * * End of Changes * * * *