SA WG2 Meeting #157	S2-2307212

22 - 26 May, 2023, Berlin, Germany

# Source:	ZTE

# Title:	New SID on QoS Monitoring Enhancement

# Document for:	Information

# Agenda Item:	10.5

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	Study on QoS Monitoring Enhancement

Acronym:	FS_eQME

Unique identifier:	TBD

Potential target Release:	Rel-19

1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  | x | x |  |
| No |  |  |  |  |  |
| Don't know | x |  |  |  | x |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a …

| x | Study |
| --- | --- |
|  | Normative – Stage 1 |
|  | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| MeasureData | SA1 | 980108 | Measurement Data Collection |

2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 980108 | Measurement Data Collection | TS 22.261: Stage 1 Requirements |

Dependency on non-3GPP (draft) specification:

3	Justification

In the 23.501, the QoS monitoring can be performed per QoS Flow per UE level, or per GTP-U path level. In the SA1 spec 22.261, there is new requirement on the QoS monitoring granularity.

- Based on operator request, for direct network connection scenarios in non-public networks, the 5G system shall be able to activate/deactivate efficient QoS monitoring with a finer granularity (e.g. per data packet) in a specific QoS flow (e.g. supporting URLLC services) to report on data packets not meeting the required QoS level.

The new requirement is to better support vertical industries which have stringent deterministic communication service performance requirement in latency, such as motion control and high speed current differential protection.

Currently the per packet measurement for N3 and N9 latency has been supported since the UPF can add the time stamp on each downlink packet and the RAN sends UL packet with time stamp immediately. However the RAN cannot report the latency over Uu per packet. How to support the per packet report of Uu latency needs to be further studied and coordination with RAN2/RAN3 may be needed.

In the existing QoS monitoring defined in the 23.501, there are some restrictions on the QoS monitoring:

- the 5GC does not know the RAN capability on the QoS monitoring. RAN does not provide any feedback if it does not support the QoS monitoring request.

The SMF may need to know if the RAN has accepted the QoS monitoring request and if not the SMF should notify the PCF and the PCF can notify to AF to allow the AF takes actions.

- For the UL only Qos flow, the existing QoS monitoring cannot be applied. In the 23.501 clause 5.33.3.2

For both time synchronised and not time synchronised between NG-RAN and PSA UPF, the PSA UPF creates and sends the monitoring packets to the RAN in a measurement frequency.

For the UL only QoS flow, it is unclear how the PSA UPF can trigger the monitoring packets since there is no DL traffic.

4	Objective

The objective of this study is to investigate the feasibility of further enhancement on network resilience.

The following objectives are expected to be studied:

1.	How to support stage 1 requirements on enhanced QoS monitoring granularity (e.g. per data packet):

2.	How to determine the RAN capability on the QoS monitoring by SMF

3.	How to support QoS monitoring for the UL only QoS flow

TU estimates and dependencies

| Work Task ID | TU Estimate (Study) | TU Estimate (Normative) | RAN Dependency (Yes/No/Maybe) | Inter Work Tasks Dependency Editor’s Note: This column should highlight if WT#x is self-contained, or it depends on the completion of other WTs |
| --- | --- | --- | --- | --- |
| WT#1 |  |  | Yes | SC |
| WT#2 |  |  | Yes | SC |
| WT#3 |  |  | No | SC |

Total TU estimates for the study phase:

Total TU estimates for the normative phase:

Total TU estimates:

5	Expected Output and Time scale

| New specifications |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| Internal TR | 23.xxx | Study on enhancement on QoS Monitoring |  |  |  |

6	Work item Rapporteur(s)

7	Work item leadership

SA2

8	Aspects that involve other WGs

The following aspects may arise related to this study:

-	Charging, OAM aspects; and

-	RAN aspects.

9	Supporting Individual Members

Supporting IM name