3GPP TSG-WG SA2 Meeting #156e	S2-2307284

# Berlin, Germany, 22 – 26 May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0159 | rev | - | Current version: | 18.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | HR-SBO re-authorization procedure due to VPLMN change |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGE_Ph2 |  |  |  |  |  | Date: |  |  | 2023-05-13 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | H-SMF authorizes HR-SBO per Serving PLMN. During VPLMN change, the V-SMF is required to request the HR-SBO request and the H-SMF checks the HR-SBO authorization for the Target VPLMN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to add a PDU Session Update Procedure for HR-SBO triggered by PLMN handover. During this procedure, the Target V-SMF sends the HR-SBO request indication for the established HR-SBO session and H-SMF authorizes the HR-SBO session for Target PLMN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | HR-SBO authorization per VPLMN does not work |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.2.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st Changes (all-new-text) * * * *

6.7.2.X	PDU Session Update Procedure for HR-SBO re-authorization

Figure 6.7.2.X-1: PDU Session Update Procedure for HR-SBO re-authorization

- The UE established a HR-SBO PDU Session described in the clause 6.7.2.2.

- Inter-PLMN handover procedure is performed.

- After the step 2, the UE performs the registration procedure to the Target AMF. During the registration procedure triggered by PLMN change, the AMF receives the SMF selection data including HR-SBO allowed indication per DNN/S-NSNAI of the Target PLMN from the UDM in the step 14b of the procedure in the clause 4.2.2.2.2 of TS 23.502 [3].

- If the Target AMF detects the inter-PLMN handover, it sends the Service Network identifier (i.e. Target VPLMN ID) and HR-SBO allowed indication for the established Session for DNN, S-NSSAI in the PDUSession_SMContext Create/Update Request to the Target V-SMF (for V-SMF insertion or change with PLMN change).

- If the Target V-SMF detects the inter-PLMN handover, it sends Serving Network identifier (i.e. Target PLMN ID) and HR-SBO request indication in the PDUSession_Update Request to the H-SMF.

- The H-SMF receives the PDUSession Update Request from the V-SMF. If the H-SMF identities the inter-PLMN handover, it may retrieve SM subscription data for the Target PLMN from the UDM in order to check whether the DNN/S-NSSAI of the established PDU Session is allowed in Target PLMN.

- If the HR-SBO is authorised for Target PLMN, the H-SMF requests the VPLMN ID and HR-SBO indication to retrieve the VPLMN Specific Offloading Policy for Target PLMN from the H-PCF.

- The H-SMF sends the VPLMN Specific Authorization Information in the Nsmf_PDUSession_Update Response message to the V-SMF.

# * * * * End Changes * * * *