3GPP TSG-WG SA2 Meeting #157	S2-2307293

# May 22-26, 2023, Berlin, Germany		Revision of S2-2306239

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4527 | rev | 3 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update TS23.501 for PDU Set and PDU Handling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | [Nokia, Nokia Shanghai Bell], Xiaomi, |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM |  |  |  |  |  | Date: |  |  | 2022-5-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Application flows are mapped by the UPF to QoS Flows based on a Packet Detection Information in the PDR. Some of the PDUs in the QoS Flow will be associated with media components (e.g. I-Frame and P-Frame), and will be classified by the UPF as belonging to PDU Sets and handled accordingly. However it is not clear how PDUs that do not contain recognizable user media components will be handled. For example, an application service flow may contain a video stream for which the UPF performs PDU Set detection and determines the GTP-U header markings. However, the same application service flow may also contain other PDUs such as RTCP PDUs (i.e., to control the RTP flow(s)), or PDUs that represent additional media components such as an audio stream for which the UPF determines PDU Set classification is not appropriate or not needed. In general, the application service flow may contain a variety of PDUs that do not conform to a template, algorithm or other methodology used by the UPF to determine that a PDU belongs to a PDU Set. All these PDUs may be multiplexed into one application flow, for instance using the various IETF protocol multiplexing options that cover RTP with multiple media streams, RTCP, STUN, DTLS-SRTP and WebRTC data channel. There are a number of alternatives for handling the PDUs of an application flow that do not naturally belong to a PDU Set. - Within a QoS flow, perform PDU Set Processing (using PSER, PSDB, PSIHI) for PDUs that belong to PDU Sets, and non-PDU Set processing (using PER, PDB) for PDUs that do not belong to PDU Sets. A major disadvantage of this is that it adds complexity for the RAN, which would need to dynamically switch between PDU Set and non-PDU Set processing / profiles on a per-PDU basis on a single DRB (for the normal case that a QoS flow is mapped to a DRB). - Have the UPF assign all PDUs in a QoS Flow to a PDU Set. If necessary, the PDU Set may comprising a single PDU. As a result, all PDUs in the QoS flow would belong to a PDU Set and hence the RAN would then only perform PDU Set based QoS handling. An advantage of this option is its simplicity. A disadvantage is that PDU Set QoS Parameters (PSER, PSDB) intended for user media components (e.g frames, slices) would now also apply to single PDUs for which they are not intended. This can especially affect PSER in that small and hence easier to deliver single PDU PDU Sets are mixed with other larger (e.g. I-Frame) PDU Sets when determining PSER. - Setup a separate QoS Flow for PDUs that do not belong to PDU Sets and PDUs that do belong to PDU Sets. - PDU Set based QoS handling using the PSDB, PSER, PSIHI PDU Set QoS parameters are used in a QoS flow for PDUs that match the PDR “Packet Detection Information” and are identified by the UPF as belonging to a PDU Set. - QoS (Rel. 17) using PDB and PER are used in a QoS flow for PDUs that match the PDR “Packet Detection Information” but are not identified by the UPF as belonging to a PDU Set. Existing QoS is used for these QoS flows. The approach with two QoS Flows may use two PCC rules with different precedence as shown in the figure below. Alternative 3 is slightly more complex for the 5GC compared to alternative 2, but it preserves the appropriate processing for PDUs that belong to PDU Sets and PDUs that do not belong to PDU Sets without impact to the RAN. Proposal: Options 2 is supported. The UPF may map one or more PDU(s) unmarked the HE to a PDU set. How the UPF maps one or more PDU(s) to a PDU set is up to the implementation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updates are made as proposed above so one QoS flow is used to support QoS for XR. The QoS flow only performs PDU Set based QoS handling using {PSER, PSDB and PSIHI} and all PDUs must belong to a PDU Set. Update the proposal to clarify the mapping for the PDU(s) unmarked the HE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification for handling PDUs that are not determined by the UPF to belong to a PDU Set is not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.37.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 4th change * * * *

5.37.5	PDU Set based QoS Handling

5.37.5.1	General

A PDU Set is comprised of one or more PDUs carrying an application layer payload such as, e.g. a video frame or video slice. The PDU Set based QoS handling by the NG-RAN is determined by PDU Set QoS parameters specified in clause 5.7.7 and PDU Set information provided by the PSA UPF as described in clause 5.37.5.2.

Editor's note:	The applicability and details of PDU Set handling in uplink direction is pending RAN WG's progress.

In addition to the PDU related service information, the AF may provide PDU Set related assistance information for dynamic PCC control. One or more of the following PDU Set related assistance information may be provided to the NEF/PCF using the AF session with required QoS procedures in clauses 4.15.6.6 and 4.15.6.6a of TS 23.502 [3].

-	PDU Set QoS parameters as described in clause 5.7.7

-	Protocol Description: Indicates protocol and payload type used by the service data flow.

AF provided PDU Set QoS Parameters and Protocol Description may be used in determining PCC Rules by the PCF and the Protocol Description may be used for identifying the PDU Set information by the PSA UPF.

Editor's note:	Whether a standardized S-NSSAI (SST) is defined for XRM or whether non-standardized S-NSSAI is used is FFS.

When the PCF determines that PDU Set based QoS Handling is to be performed for an application service flow, the PCF generates a PCC rule containing the PDU Set QoS parameters (PSER, PSDB and PSIHI) and the SMF determines a QoS Profile for the QoS Flow. PSA UPF identifies PDUs that belong to PDU Sets. If the UPF receives a or several PDU(s) that do not belong to a PDU Set based on Protocol Description for PDU Set identification, then the UPF still maps PDU(s)  to a PDU Set (e.g. it could be a PDU Set with one or several PDU(s)).

NOTE:	How the UPF maps one or more PDU(s) to a PDU set is up to the implementation, e.g. considering the arrival time period between them, the RTP head and the payload of the PDU.

Alternatively, the SMF may be configured to support PDU Set QoS without receiving PCC rules from a PCF.

Editor’s note: How PDU Set Importance is applied to the PDU(s) that does not belong to a PDU Set based on Protocol Description is FFS.

# * * * * End of change * * * *