SA WG2 Meeting #S2-157	S2-2307297

22 - 26 May, 2023, Berlin, Germany	(revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4234 | rev | - | Current version: | 18.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on the AMF and NEF subscribe and notification |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TRS_URLLC |  |  |  |  |  | Date: |  |  | 2023-05-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the last meeting, a new clause (4.15.9.X) is added to 23.502 in the CR#4130(S2-2306224) on Procedures for subscription and management of network timing synchronization status monitoring. The TSCTSF provides Clock Quality Reporting Control Information to AMF and subscribe the time status from AMF. It is needed to reflect this to AMF service operation In the new clause, the Nnef_TimeSynchronization_ConfigUpdateNotify is not only for configuration change, but also for the time status change notification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modify the Namf_Communication_NonUeN2 related service operation to support timing sync status report. Adding timing sync status change for Nnef_TimeSynchronization_ConfigUpdateNotify |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The papameter in the Namf_Communication_NonUeN2 related service operation is missing for timing sync status report. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.6.25.5, 5.2.2.2.17, 5.2.2.2.18, 5.2.2.2.19 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

Next CHANGE

##### 5.2.6.25.5	Nnef_TimeSynchronization_ConfigUpdateNotify operation

Service operation name: Nnef_TimeSynchronization_ConfigUpdateNotify

Description: Forward the notification for the time synchronization configuration or PTP time distribution status change. When the NEF receives a notification of a change corresponding to a time synchronization configuration or PTP time distribution status from the TSCTSF, it forwards the notification by invoking a Nnef_TimeSynchronization_ConfigUpdateNotify service operation to the NF consumer(s) that has subscribed for the event. The event parameters are described in Table 5.2.27.2.5-1.

Known NF Service Consumers: AF.

Inputs, Required: As specified in clause 5.2.27.5.2.

Inputs, Optional: As specified in clause 5.2.27.5.2.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

Next CHANGE

##### 5.2.2.2.17	Namf_Communication_NonUeN2MessageTransfer service operation

Service operation name: Namf_Communication_NonUeN2MessageTransfer

Description: NF Service Consumer requests to transfer a non-UE specific message to NG-RAN node(s) via N2.

Input, Required: N2 Message Container.

Input, Optional: TAI List, RAT Selector (ng-eNB or gNB), Global RAN Node List, Send Write-Replace-Warning-Indication, Send Stop-Warning-Indication, Clock Quality Reporting Control Information (CQRCI) as specified in clause 5.27.1.12 in TS 23.501 [2].

Output, Required: N2 Information Transfer Result.

Output, Optional: PWS Result Data.

PWS Result Data is included when parts of a PWS received message have not been comprehended or were missing, or if the message contained logical errors. It may contain the Unknown Tracking Area List which may be present in the associated PWS response message.

Next CHANGE

##### 5.2.2.2.18	Namf_Communication_NonUeN2InfoSubscribe service operation

Service operation name: Namf_Communication_NonUeN2InfoSubscribe

Description: The NF Service Consumer invokes this service operation to subscribe to the delivery of non-UE specific information from the NG-RAN node sent via N2 to the AMF.

Input, Required: NF Service Consumer ID, N2 information type to be subscribed.

Input, Optional: Timing synchronization status report.

Output, Required: None.

Output, Optional: None.

Next CHANGE

##### 5.2.2.2.20	Namf_Communication_NonUeN2InfoNotify service operation

Service operation name: Namf_Communication_NonUeN2InfoNotify

Description: The AMF uses this service operation to notify a particular event towards the NF Service Consumer that has subscribed for the specific information. The AMF receives messages for such events from NG-RAN via N2.

Input, Required: N2 information.

Input, Optional: Global RAN Node ID.

Output, Required: None.

Output, Optional: None.

END OF CHANGES