3GPP TSG-WG SA2 Meeting #157 	S2-2307462

Berlin, Germany, May 22 – 26, 2023	(revision of S2-2306779)

Source:	Huawei, HiSilicon

Title:	Update on Network based SL Positioning for UE without NAS connection

Document for:	Approval

Agenda Item:	9.5.2

Work Item / Release:	Ranging_SL / Rel-18

Abstract: This paper proposes to update Network based SL Positioning for UE without NAS connection.

1. Introduction/Discussion

This paper proposes some updates on Network based SL Positioning for UE without NAS connection and remove two ENs.

1. Remove the EN on enhanced 5GC-MO-LR message, and clarify the Located UE initiate the 5GC-MO-LR procedure with a new type of location service (Network based SL positioning).

Editor's note:	It is FFS if an enhanced 5GC-MO-LR message is sent by the Located UE.

2. Add a NOTE to clarify the functionality of routing identifier is same with the one used in Deferred 5GC-MT-LR Procedure (clause 6.3.1 of TS 23.273). And the routing identifier can be configured in Ranging/SL Positioning policy. Each Located UE gets the routing identifier from the Target UE.

3. To clarify that LMF may interact with GMLC to get the location of Located UE.

4. It was agreed that 5GC-MT-LR is not applicable in the case Network based SL positioning for UE without NAS connection. So the following EN can be removed.

Editor's note:	It is FFS when AMF does not maintain target UE's context or GMLC cannot get target UE AMF address because of deregistration, whether and how location report from Located UE on behalf of Target UE is supported.

5. To clarify that LMF provide the resulting location via one of the Located UE(s) to the Target UE, not via all Located UE(s).

Rev:

Also include the proposal from S2-2307110 that related to the result exposure to LCS client and AF.

2. Text Proposal

It is proposed to capture the following changes vs. TS 23.586.

# * * * * First change * * * *

### 5.5.3	Network based SL positioning for UE without NAS connection

When Target UE cannot establish the NAS connection with AMF due to the Target UE being out of coverage and at least one of the discovered Located UEs can establish NAS connection for 5GC-MO-LR, the following principles are applied:

NOTE 1:	5GC-MT-LR is not applicable in this release of the specification.

-	The Target UE performs the Located UE's discovery and selection.

-	The Target UE and Located UE(s) perform ranging/SL positioning.

-	The Target UE selects at least one of the Located UEs that can use SL-MO-LR service for SL Positioning.

-	The Target UE may transmit its ranging measurements data/ results to the Located UE(s).

-	Each Located UE that can establish the NAS connection may report the ranging/SL positioning measurement data or result and the routing identifier from Target UE to the LMF via the serving AMF of the Located UE. This may include ranging measurements data/results received from the Target UE. The endpoints for LPP messages are the LMF and the Located UE(s).

NOTE 2:	The routing identifier indicates an LMF that manages the Target UE positioning, and also used to assist the LMF in identifying the Target UE positioning, as defined in clause 6.3.1 of TS 23.273 [8].

-	The LMF may use the received information (i.e. ranging/SL positioning measurement or estimation result with the same routing identifier) and the location of Located UE(s) to calculate the location of the Target UE, and provide the resulting location via the Located UE to the Target UE.

-	If a LMF is aware of multiple Located UEs associated with the same Target UE based on the same routing identifier, it may perform the calculation using all information from these Located UEs.

-	The LMF may use the received information (i.e., ranging/SL positioning measurement data/result and the same routing identifier) and the location of Located UE(s) to calculate the location of the Target UE, and provide the resulting location via the Located UE that initiates the 5GC-MO-LR procedure to the Target UE. In case the LMF has the Target UE context, the LMF may provide the resulting location via the GMLC to the LCS Client or AF as specified in TS 23.273 [8] clause 6.3. In case the UE context was deleted, then this functionality cannot be performed.

Editor's note:	Security aspects require confirmation from SA WG3.

# * * * * End of changes * * * *