3GPP TSG-WG SA2 Meeting #157 	S2-2307517

May 22 – 26, 2023, Berlin	(revision of S2-2307461 was S2-2307119)

Source:	Sony

Title:	Update MO-LR and MT-LR Procedures for UE with NAS connection using SL positioning.

Document for:	Approval

Agenda Item:	9.5.2

Work Item / Release:	Ranging_SL / Rel-18

Abstract: There is an overlap of steps described in this clause and the referred clause in TS 23.273. This paper proposes to remove the overlapping steps and add enhance the delta description instead. Aligning headers.

# 1. Introduction/Discussion

There is an overlap of steps described in this clause and the referred clause in TS 23.273. This paper proposes to remove the overlapping steps and add enhance the delta description instead.

Also

Aligning headers of the sub-clauses.

# 2. Text Proposal

It is proposed to capture the following changes in TS 23.586.

# * * * * First change* * * *

### 6.5.1	Procedures for Network based SL Positioning for UE with NAS connection

#### 6.5.1.1	MO-LR Procedure for Network based SL Positioning for UE with NAS connection

UE can trigger 5GC-MO-LR and the LMF determines that SL positioning will be used to determine the absolute location of the UE as specified in 5GC-MO-LR procedure using SL positioning in clause 6.x.2 of TS 23.273[8] or the UE can trigger SL-MO-LR procedure defined in clause 6.x.1 of TS 23.273 [8] is performed with the following adaptations:

-	UE1 is the Target UE, and UE2/…/UEn is the Located UE.

-	It is indicated in step 8 that location calculation assistance by the network is needed and absolute location is requested.

# * * * * Next changes* * * *

#### 6.5.1.2	MT-LR Procedure for Network based SL Positioning for UE with NAS connection

5GC-MT-LR using SL positioning procedure as defined in clause 6.x.5 of TS 23.273 [8] is performed with the following adaptations:

-	The LMF performs the absolute location estimation.

# * * * * End of changes* * * *