3GPP TSG SA WG2 Meeting #157	S2-2307530

# Berlin, Germany, May 22-26, 2023 												(revision S2-2307506)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1320 | rev | 2 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Usage of Mr/Cr with IMS DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC |  |  |  |  |  | Date: |  |  | 2023-05-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Two alternative media configurations for IMS DC is supported by the current specification, MF or MRF. Detailed Nmf SBI specification is available when using MF but no information exist for the Mr/Cr interface when using an MRF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add information of Mr/Cr procedures when using MRF for IMS DC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AC.7.X (new Clause) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR… CR… |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

## AC.7.X	Mr’/Cr Procedure

To enable the use of MRF as media resource function for IMS DC, there is a need to map the MF MRM SBI services, operations, and attributes to Mr’/Cr requests and responses.  Table AC.7.X-1 defines a high level mapping between  MF MRM SBI services, operation and  Mr’/Cr Requests:

Table AC.7.X-1: MF MRM SBI service operations mapping to Mr’/Cr

| Nmf_MediaResourceManagement (MRM) service operation name | Source | Destination | Mr’/Cr Request |
| --- | --- | --- | --- |
| Nmf_MRM_Create (create one media context including one media termination) | IMS AS | MRFC | INVITE <media context service information> |
| Nmf_MRM_Update (add new media termination to existing media context with one or multiple media streams) | IMS AS | MRFC | INVITE <media context service information> |
| Nmf_MRM_Update (modifying the ‘DC Media Specification’ data of one or multiple media streams of a media termination) | IMS AS | MRFC | INFO |
| Nmf_MRM_Update (add/update/delete media stream of an existing media termination) | IMS AS | MRFC | reINVITE |
| Nnf_MRM_Delete (media context delete) | IMS AS | MRFC | BYE |

**** End of Changes ****