3GPP TSG-SA2 Meeting #157	S2-2307660

May 22-26, 2023, Berlin, DE							                                      (revision of S2-2306393)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4569 | Rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for Asynchronous Type Communication for CN based MT communication handling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RedCap_Ph2 |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - The current text in clause 5.31.7.2.4 is not clear about the AMF behaviour when the UE is not reachable in case the CN handles mobile terminated (MT) communication for the UE configured with eDRX for RRC_INACTIVE state |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clause 5.31.7.2.4 (RRC_INACTIVE): clarified that in case UE is unreachable, the AMF stores the relevant information and later forwards the N2 DL Data Notification message when the AMF determines that the UE has become reachable again. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear AMF behaviour when the UE is not reachable in case of CN based MT communication handling for UE in RRC_INACTIVE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.31.7.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

##### 5.31.7.2.4	Paging for extended DRX for RRC_INACTIVE in NR connected to 5GC

For NR, the NG-RAN may request the CN to handle mobile terminated (MT) communication for the UE configured with eDRX for RRC_INACTIVE state by means of the Connection Inactive procedure with CN based MT communication handling Procedure (see clause 4.8.1.1a of TS 23.502 [3]). This allows the CN to apply high latency communication functions as specified in clause 5.31.8. The NG-RAN provides the eDRX cycle value for RRC_INACTIVE to AMF (i.e. >10.24s). Based on the request from NG-RAN, the AMF responds to NG-RAN and informs other NFs (e.g. SMF and UPF) involved in downlink data or signalling handling and trigger the data buffering as specified in clause 4.8.1.1a of TS 23.502 [3].

When MT data or signalling arrives for a UE in RRC_INACTIVE state, the other NFs communicate with the AMF for delivery of MT data or signalling. The AMF calculates the UE reachability based on the eDRX cycle value for RRC_INACTIVE state provided by NG-RAN and triggers NG-RAN paging via an N2 DL Data Notification message if the UE is considered reachable as specified in clause 4.8.2.2b of TS 23.502 [3]. Otherwise, the AMF stores the information received in the NF request and replies to the requesting NF to apply high latency communication functions as specified in clause 5.31.8 based on eDRX cycle for RRC_INACTIVE (e.g. an Estimated Maximum Wait Time is calculated based on eDRX cycle value for RRC_INACTIVE). When the AMF determines that the UE has become reachable for paging, the AMF uses the stored information to send an N2 DL Data Notification message.

When the UE enters RRC_CONNECTED state if the NG-RAN had sent the indication for the CN to handle mobile terminated (MT) communication, NG-RAN informs the AMF that the UE is now in RRC_CONNECTED state as specified in clause 4.8.2.2 of TS 23.502 [3]. The AMF then informs other NFs that the UE is now reachable using the high latency communication functions as specified in clause 5.31.8 and MT data and signalling can be delivered to the UE.

END CHANGE