3GPP TSG-WG SA2 Meeting #157	S2-2307127

# Berlin, May 22 – 26, 2023		(revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0391 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications on UPF in Modification of User Plane Connection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi, Inspor |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eLCS_Ph3 |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To support establishing user plane connection between UE and LMF, it is described in the precondition: SMF should select a PSA UPF (located in central site or local site) connecting with the LMF for this PDU session, based on S-NSSAI, DNN and UE location information, etc. In the Modification procedure of User Plane Connection between UE and LMF, AMF may perform LMF reselection: 1b. [Conditional] AMF based on LMF indications need to reselect an LMF used for user plane positioning. If LMF reselection has occurred, the PSA UPF connecting with source LMF may not be appropriate for the target LMF. To support establishing a user plane connection between UE and target LMF, SMF shall perform UPF relocation for target LMF. If UPF relocation has occurred, the user plane connection establishment shall be performed between AMF, UE, relocated UPF, and Target LMF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that LMF reselection can trigger UPF relocation, and user plane connection establishment can be performed between AMF, UE, relocated UPF, and Target LMF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing possible impact of LMF reselection on SMF and UPF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.18.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

6.18.3	Modification of User Plane Connection between UE and LMF

The figure 6.18.3-1 shows how a secure user plane connection between UE and LMF is modified. The flow describes change of LMF but applies also when source and target LMF is the same. The procedure can also be used to terminate the user plane connection to Source LMF not selecting any Target LMF.

Figure 6.18.3-1: Connection modification between UE and LMFs

1a.	[Conditional] The LMF discovers a need to change LMF or re-establish the user plane connection between UE and LMF. The LMF sends an Nlmf_Location_UPNotify message that includes user plane information and indicates the reason for modification. The address of the AMF was provided to LMF as a "Notification Target Address" in latest Nlmf_Location_UPConfig message or Nlmf_Location_UPNotify Subscribe message.

NOTE:	The LMF change procedure is independent from the SSC mode of the PDU Session with dedicated DNN used for positioning. For SSC mode 2/3, PSA UPF connection with the LMF can be relocated with UE movement and LMF can discover the need to change LMF to reduce the user plane path latency.

1b.	[Conditional] AMF based on LMF indications need to reselect an LMF used for user plane positioing. LMF needs to be capable to establish a user plane session for positioning with the UE. Further at AMF relocation the target AMF needs to inform the LMF of the AMF change.

2.	[Conditional] If LMF reselection has occurred, AMF sends an Nsmf_PDUSession_Update_SMContext Request to inform SMF of the change of LMF. SMF determines whether UPF relocation needs to be performed based on the information of target LMF and the preconfigured information, and send an Nsmf_PDUSession_Update_SMContext Response to AMF.

3.	[Conditional] If AMF reallocation has occurred, this step is skipped. Otherwise, steps 3-8 of Figure 6.18.2-1 are performed between AMF, UE, UPF, and Target LMF with addition that UE also terminate connection to Source LMF. If UPF relocation has occurred in step 2, the relocated UPF shall be used.

4.	The AMF sends an Nlmf_Location_UPConfig Request towards the source LMF. The message may include a request for the Source LMF to terminate a specific user plane connection to the UE and the Target LMF. Alternatively, it may include information about AMF reallocation.

5-6.[Conditional] The source LMF may invoke an Nlmf_Location_LocationContextTransfer Request service operation towards the target LMF to provide the current location context of the UE, if there is periodic and triggered UE location events report context. The target LMF informs source LMF of the location context transfer operation results.

7.	[Conditional] If the user plane connection to source LMF is still active, the source LMF terminates the connection to the UE.

8.	The LMF sends Nlmf_Location_UPConfig Response message to AMF to confirm connection termination or acknowledge change of AMF.

* * * End of Changes * * * *