3GPP TSG-WG SA2 Meeting #157	S2-2307698

# Berlin, Germany, May 22 – 26, 2023		(revision of S2-2306935)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0255 | rev | 2 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of multi-path communication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2023-05-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In clause 5.17.2, the multi-path transmission via direct Uu path and via the UE-to-Network Relay is decided by the application layer mechanisms. However, the URSP and not the application layer decides the connection establishment for the multi-path transmission, and the upper layer application layer makes use of the multi-path transmission. If the PC5 connection for relaying is not available, it is clear what Remote UE does. But if the PC5 connection for relaying is available, what Remote UE does is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - To clarify that multi-path communication may be apply to the application traffic. - To clarify if the PC5 connection with Layer-3 Relay is available, this may trigger Layer-2 link modification to associate the corresponding ProSe service into the PC5 connection. - To clarify whether the PC5 connection with Layer-3 Relay is available or not, relates to the desired RSC, since it is possible that PC5 connection with Layer-3 Relay is available for RSC#1, but the desired RSC is RSC#2 based on the ProSe policy, so the exsiting PC5 connection can not be used. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not clear what is done by 3GPP specification and upper layers for multi-path communication, and not clear what Remote UE does if the PC5 connection for relaying is available. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.17.2, 6.5.4, 6.9.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 5.17.2	Multi-path transmission via direct Uu path and via 5G ProSe Layer-3 UE-to-Network Relay

A 5G ProSe Layer-3 Remote UE may access the network via a 5G ProSe Layer-3 UE-to-Network Relay with or without N3IWF.

When a 5G ProSe Layer-3 Remote UE accesses the network via a 5G ProSe Layer-3 UE-to-Network Relay without N3IWF, the multi-path transmission via direct Uu path and via the UE-to-Network Relay may be  applied to the application traffic .

When a 5G ProSe Layer-3 Remote UE accesses the network via a 5G ProSe Layer-3 UE-to-Network Relay with N3IWF, the Layer-3 Remote UE connection via Layer-3 UE-to-Network Relay with N3IWF is considered as "untrusted non-3GPP access to 5GC via N3IWF", therefore the multi-path transmission can be achieved using MA PDU Session in ATSSS features specified in clause 5.32 of TS 23.501 [4].

NOTE:	MA PDU Session requires ATSSS Information in SM subscription data as specified in TS 23.502 [5].

# * * * * Second change * * * *

### 6.5.4	5G ProSe Remote UE traffic handling for 5G ProSe UE-to-Network Relay support

For the 5G ProSe Remote UE to access the service via 5G ProSe UE-to-Network Relay, the following apply:

-	The application traffic on the 5G ProSe Remote UE is managed by URSP rules (with consideration of local configurations), following the procedure defined in clauses 6.1.2.2.1 and 6.6.2.3 of TS 23.503 [9]. The URSP rules defined in clause 6.6.2.1 of TS 23.503 [9] applies for the 5G ProSe Remote UE, with RSD enhanced to include:

-	a "ProSe Layer-3 UE-to-Network Relay Offload indication"; or

-	a "ProSe Multi-path Preference".

-	If an application or application traffic matches a URSP rule, corresponding RSDs shall be used to evaluate the existing PDU sessions, or establish a new PDU session, or determine to offload outside of a PDU session, or multipath transmission via 5G ProSe Layer-3 UE-to-Network Relay outside of a PDU session and a PDU Session over Uu reference point.

-	If the selected RSD contains "ProSe Layer-3 UE-to-Network Relay Offload indication":

-	The 5G ProSe Remote UE will route the traffic to the 5G ProSe Layer-3 UE-to-Network Relay connection without establishing a PDU session, when such connection is available.

This may trigger the 5G ProSe Remote UE to start 5G ProSe UE-to-Network Relay discovery if it is not yet started. The discovery and establishment of the connection with the 5G ProSe Layer-3 UE-to-Network Relay is controlled by the ProSe Policy (pre-) configured on the 5G ProSe Remote UE.

-	If the matched URSP rule contains both a RSD with ''Non-Seamless Offload indication" and a RSD with "ProSe Layer-3 UE-to-Network Relay Offload indication", whether to offload the traffic to non-3GPP access or the 5G ProSe Layer-3 UE-to-Network Relay connection depends on the priority of the RSDs, and the availability of the connections, as specified in the clause 6.6.2.3 of TS 23.503 [9].

-	If the selected RSD does not contain the "ProSe Layer-3 UE-to-Network Relay Offload indication" or, "Non-Seamless Offload indication", the 5G ProSe Remote UE shall use a PDU session to route the corresponding application traffic.

If configured in the ProSe Policy, the 5G ProSe Remote UE may attempt the discovery of a Relay Service Code corresponding to a 5G ProSe Layer-3 UE-to-Network Relay with N3IWF support in the discovery procedure, when the non-3GPP access type is preferred in the selected RSD. The 5G ProSe Remote UE may attempt the discovery of a Relay Service Code corresponding to a 5G ProSe Layer-2 UE-to-Network Relay in the discovery procedure, when the 3GPP access type is preferred in the selected RSD.

-	If the selected RSD contains "ProSe Multi-path Preference":

-	The 5G ProSe Remote UE is preferred to route the traffic over a PC5 connection with a 5G ProSe Layer-3 UE-to-Network Relay and a PDU Session matched to other components (e.g., Network Slice Selection) in the selected RSD. If either connection is not available then the traffic may be routed via a PC5 connection with a 5G ProSe Layer-3 UE-to-Network Relay or a PDU Session matched to other components.

-	If the PC5 connection with 5G ProSe Layer-3 UE-to-Network Relay associated with the desired RSC is not available, this may trigger the 5G ProSe Remote UE to start 5G ProSe UE-to-Network Relay discovery and connection establishment, controlled by the ProSe Policy configured on the 5G ProSe Remote UE. If the PC5 connection with 5G ProSe Layer-3 UE-to-Network Relay associated with the desired RSC is available, this may trigger Layer-2 link modification to associate the corresponding ProSe service into the PC5 connection. Determination on the desired RSC is based on the Policy/Parameters specified in clause 5.1.4.1.

-	If the PDU Session matched to other components in the selected RSD is not available, this may trigger the establishment of a new PDU Session over Uu reference point using the values specified by the selected RSD.

-	If the 5G ProSe Remote UE has an indirect connection via a 5G ProSe Layer-2 UE-to-Network Relay connection available, it will be treated as the "3GPP" access type. If the 5G ProSe Remote UE has an indirect connection via a 5G ProSe Layer-3 UE-to-Network Relay with N3IWF support available, it will be treated as the "non-3GPP" access type. The URSP handling as defined in TS 23.503 [9] applies.

For the 5G ProSe Layer-3 UE-to-Network Relay and 5G ProSe Layer-2 UE-to-Network Relay, the URSP handling does not apply to the relayed traffic from the 5G ProSe Remote UE.

For the 5G ProSe Layer-3 UE-to-Network Relay, the PDU session established for relaying the 5G ProSe Remote UE's traffic is controlled by the ProSe Policy.

# * * * * Third change * * * *

### 6.9.2	Multi-path transmission via direct Uu path and via 5G ProSe Layer-3 UE-to-Network Relay

When a 5G ProSe Layer-3 Remote UE accesses the network via a 5G ProSe Layer-3 UE-to-Network Relay without N3IWF, if the UE decides to establish multi-path via direct Uu path and via the UE-to-Network Relay based on "ProSe Multi-path Preference" in the selected RSD of the matched URSP rule for the application traffic as specified in clause 6.5.4, the following is performed:

-	The UE may establish a new PDU Session as specified in clause 4.3.2 of TS 23.502 [5] or modify an existing PDU Session as specified in clause 4.3.3 of TS 23.502 [5] for the direct Uu path.

-	The Remote UE may establishe 5G ProSe Communication via 5G ProSe Layer-3 UE-to-Network Relay without N3IWF as specified in clause 6.5.1.1 or may modify an existing PC5 connection with a 5G ProSe Layer-3 UE-to-Network Relay as specified in clause 6.4.3.6.

NOTE 1:	How to make use of the multi-path communication via direct Uu path and via a 5G ProSe Layer-3 UE-to-Network Relay for redundant or split traffic delivery is out of scope of this specifications.

When a 5G ProSe Layer-3 Remote UE accesses the network via a 5G ProSe Layer-3 UE-to-Network Relay with N3IWF, if the UE decides to establish multi-path via direct Uu path and via the UE-to-Network Relay, the UE establishes a MA PDU Session as specified in clause 4.22 of TS 23.502 [5].

# * * * * End of changes * * * *