3GPP TSG-WG SA2 Meeting #157	                                                                S2-2307871

# Berlin, Germany, May 22 – 26, 2023		(revision of S2-2306913)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3764 | rev | 6 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Extension of NWDAF registration information to reflect new accuracy checking capability |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-05-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The conclusions of KI#1 in TR 23.700-81 define that NWDAF can have the accuracy checking capability and consumers of NWDAF can request the accuracy information to such NWDAFs with such capability. When multiple NWDAFs are deployed in the network, consumers of NWDAF services requiring the accuracy information need to select the proper NWDAF with the accuracy checking capability. Such definitions need to be included in the description of NWDAF capabilities as well as in the description of NWDAF discovery and selection. Rev5: Such capability also needs to be included in the NRF profile of NWDAF capability. The Nnwdaf_MLModelMonitor service is defined in TS23.288 for ML model accuracy information monitored by an NWDAF. Such service needs to be included in the NWDAF serivice in TS23.501. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding the description of NWDAF providing accuracy information about analytics IDs and/or ML models to consumers. Adding the accuracy checking capability as a parameter for NWDAF discover and selection. Adding the accuracy checking capability in the NRF profile of NWDAF ap. Adding the Nnwdaf_MLModelMonitor service in NWDAF service. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | R18 agreements on KI#1 will not be properly captured in R18 NWDAF description. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.18, 6.3.13, 6.2.6.2, 7.2.12 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.2.18	Network Data Analytics Function (NWDAF)

The Network Data Analytics Function (NWDAF) includes one or more of the following functionalities:

-	Support data collection from NFs and AFs;

-	Support data collection from OAM;

-	NWDAF service registration and metadata exposure to NFs and AFs;-	Support analytics information provisioning to NFs and AFs;

-	Support Machine Learning (ML) model training and provisioning to NWDAFs (containing Analytics logical function).

-	Support accuracy information about Analtyics IDs provisioning for NFs;

-	Support accuracy information about ML model provisioning for NFs;

The details of the NWDAF functionality are defined in TS 23.288 [86].

NOTE 1:	Some or all of the NWDAF functionalities can be supported in a single instance of an NWDAF.

NOTE 2:	NWDAF functionality beyond its support for Nnwdaf is out of scope of 3GPP.

# * * * * Second change * * * *

6.3.13	NWDAF discovery and selection

Multiple instances of NWDAF may be deployed in a network.

The NF consumers shall utilize the NRF to discover NWDAF instance(s) unless NWDAF information is available by other means, e.g. locally configured on NF consumers. NF consumers may make an additional query to UDM, when supported, as detailed below. The NWDAF selection function in NF consumers selects an NWDAF instance based on the available NWDAF instances.

The NRF may return one or more candidate NWDAF instance(s) and each candidate NWDAF instance (based on its registered profile) supports the Analytics ID with a time that is less than or equal to the Supported Analytics Delay.

The following factors may be considered by the NF consumer for NWDAF selection:

-	S-NSSAI.

-	Analytics ID(s).

-	Supported service(s), possibly with their associated Analytics IDs.

-	NWDAF Serving Area information, i.e. list of TAIs for which the NWDAF can provide analytics, trained ML models and/or data.

NOTE 1:	If all services provided by one NWDAF do not support the same Analytics ID, the NWDAF registers the Analytics IDs of the services at the service level.

NOTE 2:	Analytics ID(s) at service level take precedence over Analytics ID(s) at NF level.

NOTE 3:	For discovery of NWDAF supporting Nnwdaf_AnalyticsSubscription or Nnwdaf_AnalyticsInfo services, the Analytics IDs at the NWDAF NF profile are used.

-	(only when DCCF is hosted by NWDAF):

-	NF type of the data source.

-	NF Set ID of the data source.

NOTE 4:	Can be used when the NWDAF determines that it needs to discover another NWDAF which is responsible for co-ordinating the collection of required data. The NWDAF does a new discovery for a target NWDAF via NRF using NF Set ID of the data source.

NOTE 5:	For discovery of NWDAF supporting Nnwdaf_DataManagement service, at least the NWDAF Serving Area information from the NWDAF profile are used.

NOTE 6:	The presence of NF type of data source or NF set ID of the data source denotes that the NWDAF can collect data from such NF Sets or NF Types.

-	Supported Analytics Delay of the requested Analytics ID(s) (see clause 6.2.6.2).

In the case of multiple instances of NWDAFs deployment, following factors may also be considered:

-	NWDAF Capabilities:

-	Analytics aggregation capability.

-	Analytics metadata provisioning capability.

-	Accuracy checking capability.

Applicable when NF consumer cannot determine a suitable NWDAF instance based on NRF discovery response, and when NWDAF registration in UDM is supported, as defined in clause 5.2 of TS 23.288 [86]: NF consumers may query UDM (Nudm_UECM_Get service operation) for determining the ID of the NWDAF serving the UE. The following factors may be considered by NF consumers to select an NWDAF instance already serving a UE for an Analytics ID:

-	SUPI.

-	Analytics ID(s).

When selecting an NWDAF for ML model provisioning, the following additional factors may be considered by the NWDAF:

-	The ML model Filter information parameters S-NSSAI(s) and Area(s) of Interest (see clause 5.2, TS 23.288 [86]) for the trained ML model(s) per Analytics ID(s) and ML Model Interoperability indicator per Analytics ID, if available.

When selecting an NWDAF that supports Federated Learning, the following additional factors may be considered by the NWDAF:

-	Time Period of Interest: time interval [start…end], during which the Federated Learning will be performed.

-	when selecting FL client:

-	FL capability type as FL client per Analytics ID.

-	Data available by the FL client.

-	when selecting FL server:

-	FL capability type as FL server per Analytics ID.

-	The ML model Filter information parameters S-NSSAI(s) and Area(s) of Interest (see clause 5.2 of TS 23.288 [86]) for the trained ML model(s) per Analytics ID(s), if available.

Editor's note:	Clarification for "Data available by the FL client" is FFS.

# * * * * Third change * * * *

6.2.6.2	NF profile

NF profile of NF instance maintained in an NRF includes the following information:

-	NF instance ID.

-	NF type.

-	PLMN ID in the case of PLMN, PLMN ID + NID in the case of SNPN.

-	Network Slice related Identifier(s) e.g. S-NSSAI, NSI ID.

-	FQDN or IP address of NF.

-	NF capacity information.

-	NF priority information.

NOTE 1:	This parameter is used for AMF selection, if applicable, as specified in clause 6.3.5. See clause 6.1.6.2.2 of TS 29.510 [58] for its detailed use.

-	NF Set ID.

-	NF Service Set ID of the NF service instance.

-	NF Specific Service authorization information.

-	if applicable, Names of supported services.

-	Endpoint Address(es) of instance(s) of each supported service.

-	Identification of stored data/information.

NOTE 2:	This is only applicable for a UDR profile. See applicable input parameters for Nnrf_NFManagement_NFRegister service operation in clause 5.2.7.2.2 of TS 23.502 [3]. This information applicability to other NF profiles is implementation specific.

-	Other service parameter, e.g. DNN or DNN list, notification endpoint for each type of notification that the NF service is interested in receiving.

-	Location information for the NF instance.

NOTE 3:	This information is operator specific. Examples of such information can be geographical location, data centre.

-	TAI(s).

-	NF load information.

-	Routing Indicator, Home Network Public Key identifier, for UDM and AUSF.

-	For UDM, AUSF and NSSAAF in the case of access to an SNPN using credentials owned by a Credentials Holder with AAA Server, identification of Credentials Holder (i.e. the realm of the Network Specific Identifier based SUPI).

-	For UDM and AUSF, and if UDM/AUSF is used for access to an SNPN using credentials owned by a Credentials Holder, identification of Credentials Holder (i.e. the realm if Network Specific Identifier based SUPI is used or the MCC and MNC if IMSI based SUPI is used); see clause 5.30.2.1.

-	For AUSF and NSSAAF in the case of SNPN Onboarding using a DCS with AAA server, identification of DCS (i.e. the realm of the Network Specific Identifier based SUPI).

-	For UDM and AUSF, and if UDM/AUSF is used as DCS in the case of SNPN Onboarding, identification of DCS (i.e. the realm if Network Specific Identifier based SUPI, or the MCC and MNC if IMSI based SUPI).

-	One or more GUAMI(s), in the case of AMF.

-	For the UPF, see clause 5.2.7.2.2 of TS 23.502 [3].

-	UDM Group ID, range(s) of SUPIs, range(s) of GPSIs, range(s) of internal group identifiers, range(s) of external group identifiers for UDM.

-	UDR Group ID, range(s) of SUPIs, range(s) of GPSIs, range(s) of external group identifiers for UDR.

-	AUSF Group ID, range(s) of SUPIs for AUSF.

-	PCF Group ID, range(s) of SUPIs for PCF.

-	HSS Group ID, set(s) of IMPIs, set(s) of IMPU, set(s) of IMSIs, set(s) of PSIs, set(s) of MSISDN for HSS.

-	For NWDAF: Supported Analytics ID(s), possibly per service, NWDAF Serving Area information (i.e. list of TAIs for which the NWDAF can provide services and/or data), Supported Analytics Delay per Analytics ID (if available), NF types of the NF data sources, NF Set IDs of the NF data sources, if available, Analytics aggregation capability (if available), Analytics metadata provisioning capability (if available), ML model Filter information parameters S-NSSAI(s) and Area(s) of Interest for the trained ML model(s) per Analytics ID(s) (if available), FL capability type (i.e. FL server or FL client, if available), Time interval supporting FL (if available), Accuracy checking capability(if available).

Editor's note:	Whether to register Time interval supporting FL to the NRF is FFS.

NOTE 4:	The NWDAF's Serving Area information is common to all its supported Analytics IDs.

NOTE 5:	The Analytics IDs supported by the NWDAF may be associated with a Supported Analytics Delay i.e. the Analytics report can be generated with a time (including data collection delay and inference delay) in less than or equal to the Supported Analytics Delay.

NOTE 6:	The determination of Supported Analytics Delay, and how the NWDAF avoid updating its Supported Analytics Delay in NRF frequently is NWDAF implementation specific.

-	Event ID(s) supported by AFs, in the case of NEF.

-	Event Exposure service supported event ID(s) by UPF.

-	Application Identifier(s) supported by AFs, in the case of NEF.

-	Range(s) of External Identifiers, or range(s) of External Group Identifiers, or the domain names served by the NEF, in the case of NEF.

NOTE 7:	This is applicable when NEF exposes AF information for analytics purpose as detailed in TS 23.288 [86].

NOTE 8:	It is expected service authorization information is usually provided by OA&M system, and it can also be included in the NF profile in the case that e.g. an NF instance has an exceptional service authorization information.

NOTE 9:	The NRF may store a mapping between UDM Group ID and SUPI(s), UDR Group ID and SUPI(s), AUSF Group ID and SUPI(s) and PCF Group ID and SUPI(s), to enable discovery of UDM, UDR, AUSF and PCF using SUPI, SUPI ranges as specified in clause 6.3 or interact with UDR to resolve the UDM Group ID/UDR Group ID/AUSF Group ID/PCF Group ID based on UE identity, e.g. SUPI (see clause 6.3.1 for details).

-	IP domain list as described in clause 6.1.6.2.21 of TS 29.510 [58], Range(s) of (UE) IPv4 addresses or Range(s) of (UE) IPv6 prefixes, Range(s) of SUPIs or Range(s) of GPSIs or a BSF Group ID, in the case of BSF.

-	SCP Domain the NF belongs to.

-	DCCF Serving Area information, NF types of the data sources, NF Set IDs of the data sources, if available, in the case of DCCF.

-	Supported DNAI list, in the case of SMF.

-	For SNPN, capability to support SNPN Onboarding in the case of AMF and capability to support User Plane Remote Provisioning in the case of SMF.

-	IP address range, DNAI for UPF.

-	Additional V2X related NF profile parameters are defined in TS 23.287 [121].

-	Additional ProSe related NF profile parameters are defined in TS 23.304 [128].

-	Additional MBS related NF profile parameters are defined in TS 23.247 [129].

-	Additional UAS related NF profile parameters are defined in TS 23.256 [136].

# * * * * Fourth change * * * *

7.2.12	NWDAF Services

The following NF services are specified for NWDAF:

Table 7.2.12-1: NF Services provided by NWDAF

| Service Name | Description | Reference in TS 23.288 [86] |
| --- | --- | --- |
| Nnwdaf_AnalyticsSubscription | This service enables the NF service consumers to subscribe/unsubscribe for different type of analytics from NWDAF. | 7.2 |
| Nnwdaf_AnalyticsInfo | This service enables the NF service consumers to request and get different type of analytics information from NWDAF or enables NWDAF to request transfer of analytics context from another NWDAF. | 7.3 |
| Nnwdaf_DataManagement | This service enables the NF service consumer to subscribe/unsubscribe and fetch data from NWDAF. | 7.4 |
| Nnwdaf_MLModelProvision | This service enables the consumer to receive a notification when an ML model matching the subscription parameters becomes available in NWDAF containing MTLF. | 7.5 |
| Nnwdaf_MLModelInfo | This service enables the consumer to request and get ML Model Information from NWDAF containing MTLF. | 7.6 |
| Nnwdaf_MLModelMonitor | This service enables the consumer to subscribe/unsubscribe for ML model accuracy information monitored by an NWDAF (i.e. NWDAF containing AnLF). The service also enables the NWDAF containing AnLF registers the use and monitoring capability for an ML model into the model provider NWDAF, i.e. NWDAF containing MTLF. | 7.9 |

# * * * * End of changes * * * *