SA WG2 Meeting #157	S2-2307916

# Berlin, Germany, May 22nd – 26th, 2023           	  			  			       (revision of S2-2306510)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4592 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Definition of TL Containers |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TRS_URLLC |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The Editor’s Notes in Annex M.1 were resolved with the approved CRs defined in S2-2306230 and S2-2306231. Based on these approvals this CR provides a proposal for the TL Container content with a complete list of parameters and respective information on references and notes. Learning of TL support based on the Feature flag. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Definition of TL container content. Usage of Feature Flag to learn the capability of support of TL at SMF/CUC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage 3 work is postponed due to missing information on TL-Container handling. Stage 3 has the capability to use Feature Flag to learn the support of TL, which would be discrepancy if not defined in SA2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.28a.2, Annex M.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Updates to the TL container and placing the TL container under Annex M |  |  |  |  |  |  |  |  |

FIRST CHANGE

### 5.28a.2	Transfer of TL-Container between SMF/CUC and AN-TL and CN-TL

When NG-RAN and UPF has signalled the support of AN-TL and CN-TL, the SMF/CUC may use the TL-Container to send a:

1)	get-request.

2)	set-request: submits the following elements of the Status group to the AN-TL or CN-TL:

-	list of InterfaceConfiguration(s) (one InterfaceConfiguration is associated with each QFI in the N3 tunnel)

submits the following elements of Talker group to the AN-TL or CN-TL, as derived by the SMF/CUC:

-	Interval.

-	EarliestTransmitOffset.

-	LatestTransmitOffset.

The AN-TL or CN-TL may use the TL-Container to send a:

1)	get-response: indicates the following elements of the Talker or Listener group from the AN-TL or CN-TL:

-	EndStationInterfaces: list of InterfaceIDs, one InterfaceID is associated with each Local F-TEID.

-	InterfaceCapabilities.

-	Buffer capability: maximum possible buffer duration supported by the Talker.

2)	set-response: acknowledges the reception of the Status group to the SMF/CUC.

Details on the TL-Container information are provided in table [M.2.X], Annex M.

The SMF may request the UPF to report AN-TL or CN-TL information by including a TL-Container with a get-request to the AN-TL or CN-TN, respectively. The get-request is sent to AN-TL in the N2 SM information and to CN-TL in the N4 Session Establishment or Modification request as described in clause 4.3.2.2 of TS 23.502 [3].

If the NG-RAN/UPF supports AN-TL/CN-TL, the NG-RAN/AN-TL or UPF/CN-TL responds with a TL-Container including the elements defined for the get-response. The elements are associated with the corresponding N3 Tunnel Info (Local F-TEID) in the NG-RAN or UPF.

The SMF may submit TL-Container including a set-request the elements defined for the set-request to NG-RAN/AN-TL or UPF/CN-TL. The set-request is sent to AN-TL in the N2 SM information and to CN-TL in the N4 Session Establishment or Modification request as described in clause 4.3.2.2 of TS 23.502 [3]. The InterfaceConfiguration is associated with the corresponding QFI in the N3 tunnel in the NG-RAN or UPF, respectively. The AN-TL or CN-TL uses the provided configuration for the traffic in the QoS Flow of the given QFI as described in Annex M.

# * * * * Second change * * * *

Annex M (normative):
Interworking with TSN deployed in the Transport Network

# M.2	TL-Container Information

Table [M.2.X]: TL-Container Information

| TL-Container information | Supported Operations - AN-TL/CN-TL (see NOTE 1) |  | Reference |
| --- | --- | --- | --- |
|  | Talker | Listener |  |
| End Station Parameters of AN-TL/CN-TL |  |  |  |
| List of InterfaceID group(s) |  |  |  |
| >Mac Address | Get | Get | IEEE Std 802.1Q [98], Table 46-3 |
| >InterfaceName (see NOTE 2) | Get | Get | IEEE Std 802.1Q [98], Table 46-3 |
| InterfaceCapabilities (see NOTE 3) |  |  |  |
| >VlanTagCapable (see NOTE 4) | Get | Get | IEEE Std 802.1Q [98], Table 46-3 |
| >BufferCapability (see NOTE 5) | Get | - | Annex M.1 |
| TN Stream Parameters |  |  |  |
| TN Stream Identification Information for mask-and-match (see NOTE 7, NOTE 14) |  |  |  |
| >tsnCpeMmIdMsduMaskLength | Set | Set | IEEE Std 802.1CBdb [X], clause 9.1.6.5 |
| >tsnCpeMmIdMsduMask | Set | Set | IEEE Std 802.1CBdb [X], clause 9.1.6.6 |
| >tsnCpeMmIdMsduMatch | Set | Set | IEEE Std 802.1CBdb [X], clause 9.1.6.7 |
| TN Stream Identification Information for DataFrameSpecification (See NOTE 13, NOTE 14) |  |  |  |
| >DataFrameSpecification | Set | Set | IEEE Std 802.1Q [98], Clause 46.2.3.4 |
| >>IEEE802-MacAddresses | Set | Set | IEEE Std 802.1Q [98], Clause 46.2.3.4.1 |
| >>IEEE802-VlanTag | Set | Set | IEEE Std 802.1Q [98], Clause 46.2.3.4.2 |
| >>IPv4-tuple | Set | Set | IEEE Std 802.1Q [98], Clause 46.2.3.4.3 |
| >>IPv6-tuple | Set | Set | IEEE Std 802.1Q [98], Clause 46.2.3.4.4 |
| InterfaceConfiguration of End Station |  |  |  |
| List of Interface(s)/Port(s) |  |  |  |
| >InterfaceName (see NOTE 8) | Set | Set | IEEE Std 802.1Q [98],Table 46-3 |
| IEEE802-MacAddresses |  |  |  |
| >>DestinationMacAddress (see NOTE 9) | Set | Set | IEEE Std 802.1Q [98],Table 46-4 |
| IEEE802-VlanTag |  |  |  |
| >>PriorityCodePoint (see NOTE 9) | Set | Set | IEEE Std 802.1Q [98], Table 46-5 |
| >>VlanId (see NOTE 10) | Set | Set | IEEE Std 802.1Q [98], Table 46-5 |
| Parameters to Calculate Gate Control Information (See NOTE 6) |  |  |  |
| List of TN Streams at Interface/Port |  |  |  |
| >Interval | Set | - | IEEE Std 802.1Q [98], Table 46-8 |
| >MaxFrameSize (see NOTE 11) | Set | - | IEEE Std 802.1Q [98], Table 46-8 |
| >TimeAwareOffset (see NOTE 12) | Set | - | IEEE Std 802.1Q [98], clause 46.2.5.3.5 |
| NOTE 1: Get = Get Response; Set = Set Request; - = not supported. NOTE 2: This parameter is optional and only required if End Station has more than one Interface/Port. It identifies the Interface/Port at the End Station. NOTE 3 Interface Capabilities are identical for all Interfaces/Ports of the End Station. NOTE 4: Possible values are TRUE or FALSE. NOTE 5: Maximum possible buffer duration for a packet with the maximum size of an Ethernet packet (1522 Bytes). NOTE 6: SMF/CUC enables the TL to calculate Gate Control Information. Port(s) of the Talker TL may create jitter due to implementation of the TL. How the jitter behaviour is identified and the correction is carried out is up to implementation. NOTE 7: The parameters are provided if mask-and-match stream identification is supported. NOTE 8: The parameter is provided if End Station supports multiple Interfaces/Ports. SMF/CUC guarantees that the End Station MAC address + InterfaceName, which is provided from TN-CNC as InterfaceConfiguration is translated with respect to the LLDP information sent via each interface at the user plane. NOTE 9: The parameter is provided if Stream Transformation is performed at AN-TL/CN-TL and if Active Destination MAC and VLAN stream identification is signalled from SMF/CUC to TN CNC and if TN CNC provides it in the status group for the Interface/Port. NOTE 10: The parameter is provided if Stream Transformation is performed at AN-TL/CN-TL and if Active Destination MAC and VLAN stream identification is signalled from SMF/CUC to TN CNC and if TN CNC provides it in the status group for the Interface/Port and End Station VlanTagCapable is TRUE. NOTE 11: Message Length of the Stream including the GTP-U header size. NOTE 12: The parameter is provided if TN Stream is scheduled. NOTE 13: Information provided in the DataFrameSpecification shall match to the GTP-U information received outside of the TL-Container. NOTE 14: Mask-and-match and DataFrameSpecification shall not be supported at the same time. |  |  |  |

END OF CHANGES