3GPP TSG-WG SA2 Meeting #157	S2-2308024

# Berlin Germany, May 22–26, 2023	              (Revision of S2-2307376, 6201. Merge of 7021, 6616)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4322 | rev | 6 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | EN resolving for KI#8 Except TSCAC and clarifications on AF inputs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Tencent, Tencent Cloud, Huawei, HiSilicon, InterDigital Inc., Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM |  |  |  |  |  | Date: |  |  | 2023-05-24 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In clause 5.37.8 of TS 23.501, there is one statement saying AF provide peridicty if it is trusted. However, being trusted or not is not the direct reason regarding to whether AF provide periodicity or not. This should be clarified. Also, there is one EN about UPF to SMF reporting as follows which can be removed by adding a N4 session level report procedure in TS 23.502. Editor's note: The method used to report the Periodicity information from the UPF to the SMF is FFS. Another EN about new RTP header as follows which should not be an EN and should be resovled Editor’s note: Support of new RTP header extension for End of Data Burst indication by the application server depends on progress in SA WG4 5G_RTP WI. In addition, some editorial changes on N6 jitter are needed. Reason for changes on top of Rev 6: There is a reference to N6 jitter measurements that should be changed to Traffic Parameter measurements. Section 5.38.8.2 refers to a procedure called AF Session Establishment with QoS, but there is no AF “establishment” of a session with QoS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Removing an EN about UPF reporting proceudre to SMF and make corresponding changes in 5.8.5 and 5.37.8 - Resolve the EN and add reference - Some editorial changes to N6 jitter/jitter, NG RAN In Rev 6, changed N6 jitter measurements to Traffic Parameter measurements. Updated the text in 5.38.8.2 to say “procedures for setting up or updating an AF session”. Removed the update to the references section since 26.522 was already added to the references section in agreed CR S2-2306194. Simplified NOTE 1 and 2 to clarify how the UPF derives the N6 jitter and periodicity when not provided by AF is implementation dependent. Made clarifications on what N6 jitter measures. Add End of Data Burst indication to the QER and remove it from the FAR. The request to perform End of Data Burst marking is included in the QER, since that’s what contains instructions related to the QoS enforcement of the traffic, instead of forwarding actions in the FAR and the End of Data Bursts are to be detected at QoS Flow level. See C4-231525 for reference. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | EN not resolved and some conceptual confusions remains in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.8.5.4, 5.8.5.11, 5.8.5.12, 5.37.8.1, 5.37.8.2, 5.37.8.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change* * * *

#### 5.8.5.4	QoS Enforcement Rule

The following table describes the QoS Enforcement Rule (QER) that defines how a packet shall be treated in terms of bit rate limitation and packet marking for QoS purposes. All Packet Detection Rules that refer to the same QER share the same QoS resources, e.g. MFBR.

Table 5.8.5.4-1: Attributes within QoS Enforcement Rule

| Attribute | Description | Comment |
| --- | --- | --- |
| N4 Session ID | Identifies the N4 session associated to this QER |  |
| Rule ID | Unique identifier to identify this information. |  |
| QoS Enforcement Rule correlation ID (NOTE 1) | An identity allowing the UP function to correlate multiple Sessions for the same UE and APN. | Is used to correlate QoS Enforcement Rules for APN-AMBR enforcement. |
| Gate status UL/DL | Instructs the UP function to let the flow pass or to block the flow. | Values are: open, close, close after measurement report (for termination action "discard"). |
| Maximum bitrate | The uplink/downlink maximum bitrate to be enforced for the packets. | This field may e.g. contain any one of: - APN-AMBR (for a QER that is referenced by all relevant Packet Detection Rules of all PDN Connections to an APN) (NOTE 1). - Session-AMBR (for a QER that is referenced by all relevant Packet Detection Rules of the PDU Session) - QoS Flow MBR (for a QER that is referenced by all Packet Detection Rules of a QoS Flow) - SDF MBR (for a QER that is referenced by the uplink/downlink Packet Detection Rule of a SDF) - Bearer MBR (for a QER that is referenced by all relevant Packet Detection Rules of a bearer) (NOTE 1). |
| Guaranteed bitrate | The uplink/downlink guaranteed bitrate authorized for the packets. | This field contains: - QoS Flow GBR (for a QER that is referenced by all Packet Detection Rules of a QoS Flow) - Bearer GBR (for a QER that is referenced by all relevant Packet Detection Rules of a bearer) (NOTE 1). |
| Averaging window | The time duration over which the Maximum and Guaranteed bitrate shall be calculated. | This is for counting the packets received during the time duration. |
| Down-link flow level marking | Flow level packet marking in the downlink. | For UPF, this is for controlling the setting of the RQI in the encapsulation header as described in clause 5.7.5.3. |
| QoS Flow ID | QoS Flow ID to be inserted by the UPF. | The UPF inserts the QFI value in the tunnel header of outgoing packets. |
| Paging Policy Indicator | Indicates the PPI value the UPF is required to insert in outgoing packets (see clause 5.4.3.2). | PPI applies only for DL traffic. The UPF inserts the PPI in the outer header of outgoing PDU. |
| Packet rate (NOTE 1) | Number of packets per time interval to be enforced. | This field contains any one of: - downlink packet rate for Serving PLMN Rate Control (the QER is referenced by all PDRs of the UE belonging to PDN connections using CIoT EPS Optimisations as described in TS 23.401 [26]). - uplink/downlink packet rate for APN Rate Control (the QER is referenced by all PDRs of the UE belonging to PDN connections to the same APN using CIoT EPS Optimisations as described in TS 23.401 [26]). |
| End of Data Burst Indication | Indicates to the UPF to provide an End of Data Burst indication of the last PDU of a Data burst to the NG-RAN over GTP-U | NG-RAN can configure UE power management schemes like connected mode DRX when UPF provides an indication of the End of Data Burst, see clause 5.37.8.3. |
| NOTE 1: This parameter is only used for interworking with EPC. |  |  |

# * * * * Next change* * * *

#### 5.8.5.6	Forwarding Action Rule

The following table describes the Forwarding Action Rule (FAR) that defines how a packet shall be buffered, dropped or forwarded, including packet encapsulation/decapsulation and forwarding destination.

Table 5.8.5.6-1: Attributes within Forwarding Action Rule

| Attribute | Description | Comment |
| --- | --- | --- |
| N4 Session ID | Identifies the N4 session associated to this FAR. | NOTE 9. |
| Rule ID | Unique identifier to identify this information. |  |
| Action | Identifies the action to apply to the packet | Indicates whether the packet is to be forwarded, duplicated, dropped or buffered. When action indicates forwarding or duplicating, a number of additional attributes are included in the FAR. For buffering action, a Buffer Action Rule is also included and the action can also indicate that a notification of the first buffered and/or a notification of first discarded packet is requested (see clause 5.8.3.2). For drop action, a notification of the discarded packet may be requested (see clause 5.8.3.2). |
| Network instance (NOTE 2) | Identifies the Network instance associated with the outgoing packet (NOTE 1). | NOTE 8. |
| Destination interface (NOTE 3) (NOTE 7) | Contains the values "access side", "core side", "SMF", "N6-LAN", "5G VN internal". | Identifies the interface for outgoing packets towards the access side (i.e. down-link), the core side (i.e. up-link), the SMF, the N6-LAN (i.e. the DN), or to 5G VN internal (i.e. local switch). |
| Outer header creation (NOTE 3) | Instructs the UP function to add an outer header (e.g. IP+UDP+GTP, VLAN tag), IP + possibly UDP to the outgoing packet. | Contains the CN tunnel info, N6 tunnel info or AN tunnel info of peer entity (e.g. NG-RAN, another UPF, SMF, local access to a DN represented by a DNAI) (NOTE 8). Any extension header stored for this packet shall be added. The time stamps should be added in the GTP-U header if QoS Monitoring for packet delay is enabled for the traffic corresponding to the PDR(s). PDU Set information may be added to the GTP-U header according to clause 5.37.5.2. |
| Send end marker packet(s) (NOTE 2) | Instructs the UPF to construct end marker packet(s) and send them out as described in clause 5.8.1. | This parameter should be sent together with the "outer header creation" parameter of the new CN tunnel info. |
| Transport level marking (NOTE 3) | Transport level packet marking in the uplink and downlink, e.g. setting the DiffServ Code Point. | NOTE 8. |
| Forwarding policy (NOTE 3) | Reference to a preconfigured traffic steering policy or http redirection (NOTE 4). | Contains a policy identified by the TSP ID(s) in the PCC rule. This Forwarding policy refers to a preconfigured forwarding behaviour in UPF, which may be related to: - N6-LAN steering to steer the subscriber's traffic to the appropriate N6 service functions deployed by the operator; - local N6 steering to enable traffic steering in the local access to the DN according to the routing information provided by an AF as described in clause 5.6.7; - a Redirect Destination and values for the forwarding behaviour (always, after measurement report (for termination action "redirect")). |
| Metadata (NOTE 10) | Metadata the UPF needs to add to traffic sent over a SFC. | The metadata information is associated with a TSP ID related to an N6-LAN steering policy. |
| Request for Proxying in UPF | Indicates that the UPF shall perform ARP proxying and / or IPv6 Neighbour Solicitation Proxying as specified in clause 5.6.10.2. | Applies to the Ethernet PDU Session type. |
| Container for header enrichment (NOTE 2) | Contains information to be used by the UPF for header enrichment. | Only relevant for the uplink direction. |
| Buffering Action Rule (NOTE 5) | Reference to a Buffering Action Rule ID defining the buffering instructions to be applied by the UPF (NOTE 6) |  |
| NOTE 1: Needed e.g. if: - UPF supports multiple DNN with overlapping IP addresses; - UPF is connected to other UPF or NG-RAN node in different IP domains; - UPF "local switch" and N19 forwarding is used for different 5G LAN groups. NOTE 2: These attributes are required for FAR action set to forwarding. NOTE 3: These attributes are required for FAR action set to forwarding or duplicating. NOTE 4: The TSP ID is preconfigured in the SMF and used to determine the Forwarding Policy included in the FAR according to the description in clause 5.6.7 and clause 6.1.3.14 of TS 23.503 [45] for local N6 steering and in clause 5.6.16 and clause 6.1.3.14 of TS 23.503 [45] for N6-LAN steering. The Forwarding Policy action is enforced before the Outer header creation actions. NOTE 5: This attribute is present for FAR action set to buffering. NOTE 6: The buffering action rule is created by the SMF and associated with the FAR in order to apply a specific buffering behaviour for UL/DL packets requested to be buffered, as described in clause 5.8.3 and clause 5.2.4 of TS 29.244 [65]. NOTE 7: The use of "5G VN internal" instructs the UPF to send the packet back for another round of ingress processing using the active PDRs pertaining to another N4 session of the same 5G VN group. NOTE 8: When in architectures defined in clause 5.34, a FAR is sent over N16a from SMF to I-SMF, the FAR sent by the SMF may indicate that the I-SMF is to locally determine the value of this attribute in order to build the N4 FAR rule sent to the actual UPF controlled by the I-SMF. This is further defined in clause 5.34.6. NOTE 9: In the architecture defined in clause 5.34, the rules exchanged between I-SMF and SMF are not associated with a N4 Session ID but are associated with a N16a association. NOTE 10: The use of Metadata is described in clause 5.6.16. How the UPF transforms the metadata into actual information sent with the traffic (e.g. in the encapsulation header) is based on local policies related with the Forwarding Policy and not specified. |  |  |

# * * * * Next change * * * *

#### 5.8.5.11	Session Reporting Rule

The following table describes the Session Reporting Rule (SRR) that defines the detection and reporting events that the UPF shall report, that are not related to specific PDRs of the PDU Session, as follows:

-	Per QoS Flow per UE QoS Monitoring Report, as specified in clause 5.33.3.2.

-	Change of 3GPP or non-3GPP access availability, for an MA PDU session.

-	Per QoS Flow N6 Traffic Parameter Measurement Report.

Table 5.8.5.11-1: Attributes within Session Reporting Rule

| Attribute | Description | Comment |
| --- | --- | --- |
| N4 Session ID | Identifies the N4 session associated to this SRR. |  |
| Rule ID | Unique identifier to identify this information. | Used by UPF when reporting. |
| QoS Monitoring per QoS Flow Control Information | Indicates the UPF to apply perform the QoS Monitoring report for one or more QoS Flows. | The IE is defined in clause 7.5.2.9 of the TS 29.244 [65]. See NOTE 1. |
| Access Availability Control Information | Indicates the UPF to report when an access type becomes available or unavailable for an MA PDU Session. | The IE is defined in clause 7.5.2.9 of TS 29.244 [65]. |
| N6 Traffic Parameter Measurement Control Information | Indicates the UPF to report N6 Traffic parameter measurements for one QoS Flow, e.g.., N6 jitter for a DL Periodicity and, the UL/DL periodicity. | The IE is defined in clause 7.5.2.9 of TS 29.244 [65]. See NOTE 2. |
| NOTE 1: The QoS Monitoring per QoS Flow Control Information may contain an Indication of local direct event notification and. The Indication of local event notification includes a Notification Target Address (the details are described in clause 5.8.2.18) that identifies the recipient of the information being notified by the UPF (Local NEF/AF). The Indication of local direct event notification also indicates that the UPF reports the information to the NF indicated by the Target of reporting via Nupf_EventExposure_Notify service operation. NOTE 2: The DL Periodicity is provided by the SMF in the N6 Traffic Parameters Measurement Control Information when the DL Periodicity is received from the PCF. |  |  |

# * * * * Next change * * * *

#### 5.8.5.12	Session reporting generated by UPF

The UPF sends the session report to inform the SMF the detected events for a PDU Session that are related to an SRR. The UPF may support notification to the AF possibly via local NEF as described in clause 6.4 of TS 23.548 [130].

Table 5.8.5.12-1: Attributes within Session Reporting

| Attribute | Description | Comment |
| --- | --- | --- |
| N4 Session ID | Identifies the N4 session associated to the SRR which triggered the report. |  |
| Rule ID | Unique identifier to identify the Session Reporting Rule within a session which triggered the report. | Used by UPF when reporting. |
| QoS Monitoring Report | Indicates the QoS Monitoring result for one or more QoS Flows. | The IE is defined in clause 7.5.8.6 of TS 29.244 [65]. |
| Access Availability Report | Indicates the change of 3GPP or non-3GPP access availability, for an MA PDU session. | The IE is defined in clause 7.5.8.6 of TS 29.244 [65]. |
| N6 Traffic Parameter Measurement Report | Indicates the N6 Traffic Parameter measurement result for one QoS Flow, e,g., N6 jitter for a DL Periodicity and, the UL/DL periodicity. | The IE is defined in clause 7.5.8.6 of TS 29.244 [65]. |

# * * * * Next change * * * *

#### 5.37.8.1	General

The following traffic assistance information may be provided by the CN to NG-RAN in order to configure UE power saving management scheme for connected mode DRX:

-	UL and/or DL Periodicity;

-	N6 Jitter Information associated with the DL Periodicity;

-	Indication of End of Data Burst.

#### 5.37.8.2	Periodicity and N6 Jitter Information associated with Periodicity

In the procedures for setting up or updating an AF session with QoS, the 5G System may be provided with Periodicity information for NG-RAN to configure UE power management for connected mode DRX. The Periodicity information is provided by the AF to the PCF via NEF or directly to the PCF when the AF is trusted.

Editor's note:	The method used to provide the Periodicity information to SMF is FFS.

If Periodicity information is available at the PCF, the PCF sends the Periodicity information received from the AF/NEF to the SMF and, in accordance with the operator's local policies, include an indication for SMF to request the UPF to perform N6 Traffic Parameter(s) measurement (i.e. DL periodicity associated N6 jitter range and, if not provided by the AF, UL/DL periodicity) within the PCC Rules.

Upon reception of a PCC rule with Periodicity information, the SMF determines the TSCAI and forwards it to the NG-RAN. If the PCC rule indicates to perform N6 Traffic Parameter measurements, the SMF shall request the UPF to monitor and periodically report the N6 Traffic Parameters (i.e. the N6 jitter range associated with the DL Periodicity and, if not provided by the AF, UL/DL periodicity) using the N4 Session Modification procedure, see clause 5.8.5.11. If the DL Periodicity is available at the SMF, the SMF also sends the DL Periodicity to the UPF. The UPF reports the measured N6 Traffic Parameters to SMF via N4 interface.

NOTE 1:	How the UPF derives the N6 jitter and

periodicity (i.e., when periodicity is not provided by the AF) is implementation dependent.

At reception of  measured N6 Traffic Parameter(s) from the UPF in the N4 Session Level Report, the SMF includes the N6 jitter range together with the associated DL periodicity and the UL periodicity if not provided by the AF in the TSCAI and forwards it to the NG-RAN in an NGAP message, see clause 5.27.2.

NOTE 3:	In order to prevent frequent updates from the UPF, the UPF sends the N6 Jitter Measurement Report periodically or only when the N6 jitter is larger than a threshold.

The N6 jitter indicates the maximum positive and negative deviation of the arrival of the DL packets compared to the ideal interval arrival time.

#### 5.37.8.3	End of Data Burst Indication

An indication of End of Data Burst may be provided to the NG-RAN by the UPF, e.g. to configure UE power management schemes like connected mode DRX.

The PCF may provision the Protocol Description within PCC Rules based on the information provided by the AF and/or the local operator policies.

SMF should request the UPF to detect the last PDU of the data burst and mark the End of Data burst in the GTP-U header of the last PDU in downlink, according to the PCC rule and/or the local operator policies.

According to the request and information from the SMF, the UPF identifies the last PDU of a Data burst in the DL traffic based on the End indication according to the protocol header as defined in Protocol Description (see TS 26.522 [TS26522]) or UPF implementation and provides an End of Data Burst indication to the NG-RAN over GTP-U of the last PDU of a Data burst.

NOTE:	There can be some packets from the Data Burst received by NG-RAN after the PDU with End of Data Burst Indication if packets are received out of sequence.

# * * * * End of changes * * * *