3GPP TSG-WG SA2 Meeting #157	S2-2308034

# Berlin, Germany, May 22 – 26, 2023		(revision of S2-2306901)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4427 | rev | 3 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications on the UE member selection assistance functionality |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIMLsys |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on Table 4.15.13.2-1 of TS 23.502, the UE trajectory and expected application operation time duration do not belong to the UE filtering criteria, and the “UE transmission latency performance” should be changed to “end-to-end data volume transfer time performance”. It should be clarified that the candidate UE(s) provided by the NEF may not be the final member UE(s) used in the application operation, and the final member UE(s) can be determined by AF internal logic. According to Table 4.15.13.2-1 of TS 23.502, the “number of UEs” and “S-NSSAI” is removed, and the UE location is changed to UE current location and UE historical location. Besides, since the UE current location and UE historical location can already indicate the area of interest, so the “Area of Interest” is removed from the UE filtering criteria. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the UE filtering criteria. Clarified that the final member UE(s) used in application operation can be determined by AF internal logic. Change the UE location to UE current location and UE historical location, and remove “number of UEs”, “S-NSSAI” and “Area of Interest” from the UE filtering criteria. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE filtering criteria are not aligned with Table 4.15.13.2-1 of TS 23.502. It’s unclear what is the difference between the candidate UE(s) provided by the NEF and the final member UE(s) used in application operation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.46.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.46.2	UE member selection assistance functionality for application operation

5G system may support UE member selection assistance functionality to assist the AF in selecting member UE(s) that can be used in application operations such as AI/ML based applications (e.g. Federated Learning). The UE member selection assistance functionality shall be hosted by NEF, and the features of the UE member selection assistance functionality hosted by NEF include (see clause 4.15.13 of TS 23.502 [3] for details of UE member selection procedures):

-	Receiving a request from the AF that shall include a list of target member UE(s), optionally (a) time window(s), and one or more filtering criteria as specified in Table 4.15.13.2-1 of TS 23.502 [3] (e.g. UE current location, UE historical location, UE direction, QoS requirements, DNN, preferred access/RAT type, Desired end-to-end data volume transfer time performance).

-	Referring to the filtering criteria provided by the AF, NEF interacts with 5GC NFs using existing services to collect the corresponding data from relevant 5GC NFs (e.g. PCF, NWDAF and AMF) to derive the list(s) of candidate UE(s) (i.e. UE(s) among the list of target member UE(s) provided by the AF).

-	Providing the AF with the UE member selection assistance information, including one or more lists of candidate UE(s), and optionally other additional information (e.g. one or more recommended time window(s) for performing the application operation, QoS of each target UE, UE(s) location, Access/RAT type).

The UE member selection assistance information provided by the NEF can be used by the AF to select member UE(s) used in application operations. (See clause 5.2.6.31 of TS 23.502 [3] for details of parameters)

NOTE:	AF can choose whether to use the UE member selection assistance functionality provided by NEF.

AF in either trusted or untrusted domain can select the UE members e.g. participating in federating learning operation, by collecting the corresponding data using network exposure information as described in clause 4.15.13 of TS 23.502 [3], e.g. UE location reporting from the AMF, user plane information from the UPF and data analytics from NWDAF.

Editor's note:	The term "UE member selection" is FFS.

# * * * * End of changes * * * *