3GPP TSG-SA WG2 Meeting #157	S2-2308090

# Berlin, Germany, May 22 – 26, 2023                                              (revision of S2-23xxxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0728 | rev | 5 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#2- Removing Editor´s Note |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eUEPO |  |  |  |  |  | Date: |  |  | 05/12/2023 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The Conclusions of KI#2 states that a new Analytics ID will be discussed to be supported to monitor traffic in one or multiple PDU Session for the purpose to monitor if the UE enforces the URSP Rules as instructed by the PCF. However, the current NWDAF role is to provide analytics without knowing how the consumer will use them, as such each Analytics ID can be used in multiple procedures. The proposal is to generalize this Analytics ID to report the UE(s) run traffic as indicated by the consumer, and then the consumer knows how to use the Analytics ID. The following Editor´s Note are to be resolved: Editor's note: It is FFS how the SMF or UPF can collect URL list and Domain Name list. Proposed resolution: The NWDAF uses the existing Event on UsageDataMeasure that reports both URL/s and/or Domain name as defined in 23.502 for this Event. Editor's note: It is FFS how the Domain Name list is collected when the EASDF is deployed. Proposed resolution: The Domain name list is provided by UPF according to existing Event reporting. No discussion on whether EASDF plays a role, and what this role is. This is proposed to be removed, and take this discussion in Rel-19 if needed. Editor's Note: Whether a Rel 18 UPF supports reporting traffic not matching a SDF template on Nupf_EventExposure is FFS. Proposed resolution: There is no support for the UPF to report traffic not matching a SDF template. It is possible to report traffic matching a “match all” SDF template, it is proposed to remove this Editor´s Note and clarify that the UPF can report traffic matching a “match all” rule (or a default rule). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove Editor´s Note on KI#2 to allow that the consumer can provide a Traffic Descriptor including an Application ID and a Domain Descriptor. Refer to the Traffic Descriptor provided by the consumer, since the NWDAF does not know if a URSP Rule was sent to the UE or not. The NWDAF provides analytics on traffic detected on the PDU Session, with further categorization whether it matches the Traffic Descriptor, and then the consumer knows what this Traffic Descriptor was used for. Extend the input data to list the information that Nupf_EventExposure provides when subscription to event “UserDataUsageMeasurest” is requested on PDU Session level, and request to report per Flow information. Editor´s note described above are removed with the proposed resolution. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conclusions of KI#2 are not specified. Editor´s Note remains |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.20, 6.20.1, 6.20.2, 6.20.3, 6.20.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * *First change * * * *

## 6.20	PDU Session traffic analytics

### 6.20.1	General

This clause specifies the procedure for an NWDAF to provide statistics on whether traffic of UEs via one or multiple PDU sessions is according to the information provide by the service consumer.

The NWDAF collects traffic flow information of UE traffic via PDU session(s) established for a specific S-NSSAI and/or DNN and provides statistics of UEs that route traffic according to the information provided by the service consumer (e.g., Traffic Descriptor, S-NSSAI, DNN)  and UEs that route traffic which is not expected according to the information provided by the service consumer (e.g., Traffic Descriptor, S-NSSAI, DNN)

The assumption is that Traffic Descriptors are known (e.g. known flow description), so that there are associated Packet Detection Rule(s) for the known traffic configured in the UPF.

NOTE 1:	How traffic from different applications over the same S-NSSAI/DNN and/or PDU session is discerned is implementation-specific.

The service consumer may be a PCF.

The consumer of these analytics includes in the request:

-	Analytics ID = "PDU Session traffic".

-	Target of Analytics Reporting: a SUPI or a list of SUPIs or an Internal-Group-Idor any UE.

-	Traffic Descriptor: Application Identifier, IP Descriptions or Domain Descriptors are applicable.

NOTE 2:	 The Application Id provided by the consumer is known by the network, i.e., corresponds to an Application Id known at the UPF.

-	Analytics Filter Information containing:

-	Area of Interest (i.e. the location of UEs where PDU Session traffic is monitored);

-	S-NSSAI;

-	DNN;

-	An Analytics target period indicates the time period over which the analytics are requested.

-	Optionally, maximum number of objects.

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

### 6.20.2	Input Data

NWDAF collects input data from the SMF and/or UPF. The detailed data are described in Table 6.20.2-1.

Table 6.20.2-1: Collected PDU Session User Plane Traffic Information

| Information | Source | Description |
| --- | --- | --- |
| SUPI | SMF, UPF | UE ID for the UE that established PDU sessions to an S-NSSAI/DNN. |
| S-NSSAI | SMF, UPF | The S-NSSAI for which the PDU session is established. |
| DNN | SMF, UPF | The DNN for which the PDU session is established. |
| PDU Session Information |  | The detected Service Data Flow, volume measured and Application related information in a PDU Session. |
| > Packet Filter Set | SMF, UPF | Packet Filter Set for the detected Service Data Flow as defined in clause 5.8.2.4.2 in TS 23.502 [3]. |
| > Application Identifier | SMF, UPF | If available, the Application Identifier is reported. |
| > URL list | SMF, UPF | If available, list of URLs extracted from the inspected user plane packets in the Service Data Flow. |
| > Domain Name list | SMF, UPF | If available, list of domain names extracted from the inspected user plane packets in the Service Data Flow. |
| > UL Data volume | SMF, UPF | Measured UL data traffic volume for the Service Data Flow for the duration of the PDU Session. |
| > DL Data volume | SMF, UPF | Measured DL data traffic volume for the Service Data Flow for the duration of the PDU Session. |

NOTE:	Care needs to be taken with regards to load and major signalling caused when requesting Any UE. This could be achieved via utilization of some event filters (e.g., Area of Interest), a specific DNN, S-NSSAI or sampling ratio as part of Event Reporting Information.

The NWDAF collects input data from the UPF either indirectly via the SMF, or directly from the UPF using the "UserDataUsageMeasures" event exposure event as described in clause 4.15.4.5 of TS 23.502 [3]. Further details about input parameters are described in Table 4.15.4.5.1 of TS 23.502 [3].

### 6.20.3	Output Analytics

The NWDAF collects input data from the UPF, via SMF when the request is for a UE or a group of UEs or directly from the UPF when possible (see TS 23.501[2] clause 5.8.2.17) if the request applies for any UE and NWDAF provides analytics of UEs about the traffic they route over a PDU session of specific S-NSSAI/DNN according to the Traffic Descriptor provided by the consumer and traffic they route over a PDU Session that is not according to the Traffic Descriptor provided by the consumer, and information about that traffic.

The output analytics is shown in Table 6.20.3-1.

Table 6.20.3-1: PDU Session traffic statistics

| Information | Description |
| --- | --- |
| List of SUPIs or SUPI, | Identifies a SUPI, or a list of SUPIs for which analytics are provided. |
| S-NSSAI | Identifies the Network Slice for which analytic information is provided. |
| DNN | Identifies the data network name (e.g. internet) for which analytics information is provided. |
| Traffic matching the Traffic Descriptor | Identifies traffic that matches Traffic Descriptor provided by the consumer in those PDU Sessions identified by the S-NSSAI and DNN above and the volume. |
| > Traffic Descriptor | IP Flow descriptor containing 3-tuple, server side (destination address, port, and protocol) or Application ID or Domain descriptor. |
| > Volume | measures of data volume exchanged (UL, DL and/or overall) and/or number of packets exchanged (UL, DL and/or overall) for the Traffic Descriptor within the PDU Session. |
| Traffic which does not match Traffic Descriptor | Identifies traffic that does not match Traffic Descriptor provided by the consumer in those PDU Sessions identified by the S-NSSAI and DNN above and the volume. |
| > Traffic Descriptor | IP Flow descriptor containing 3-tuple, server side (destination address, port, and protocol) or Application ID or Domain descriptor. |
| > Volume | measures of data volume exchanged (UL, DL and/or overall) and/or number of packets exchanged (UL, DL and/or overall). |

NOTE:	Predictions are not provided

### 6.20.4	Procedures

The procedure for deriving PDU Session traffic  is shown below.

Figure 6.20.4-1: NWDAF providing PDU Session traffic analytics

1.	The Consumer NF (e.g. the PCF) requests or subscribes to the NWDAF to request PDU Session Traffic analytics. The consumer includes within analytics filter the expected traffic via a PDU session according to the Traffic Descriptors  and the SUPI or list of SUPIs, the S-NSSAI and DNN of the PDU session. It is assumed that there are associated Packet Detection Rule(s) for the expected traffic listed in the analytics request, i.e. the expected traffic is known.

2.	The NWDAF determines to collect data, either directly from the UPF or indirectly via the SMF and identifies the SMF(s) and/or UPF(s) to retrieve input data according to the S-NSSAI/DNN and SUPI or list of SUPIs  in the analytics request.

3.	The NWDAF collects input information using the procedure for subscription to UPF event exposure for certain UEs via SMF as described in clause 4.15.4.5.2 of TS 23.502 [3]; the Target of Event Reporting is set to each of the tuple (SUPI, DNN and S-NSSAI) that has a PDU Session in the same SMF, the Target Subscription Information is set to “UserDataUsageMeasures”, the Target service data flows is absent to indicate that the UPF reports any detected service data flow in the PDU Session.

The UPF reports volume measurements for the PDU Session with per application granularity, the report includes volume measurements, and provide the application related information, i.e. URLs and Domain names.

4.	The NWDAF derives analytics indicating a list of UEs and the traffic they route according to the provided information provided by the consumer (i.e. Traffic Descriptor, S-NSSAI and DNN), including its volume and the traffic they route that it is not expected according to the information provided by the consumer (i.e. Traffic Descriptor, S-NSSAI and DNN) including its volume.

5.	The NWDAF notifies URSP enforcement analytics.

# * * * * End of changes * * * *