SA WG2 Meeting #157	S2-2308172

# Berlin, Germany, 2023-05-22 – 2023-05-26 											was S2-2307939

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0112 | rev | 3 | Current version: | 18.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Home Routed-Session Breakout (HR-SBO) - offload policy structure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGE_Ph2 |  |  |  |  |  | Date: |  |  | 2023-05-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | It was agreed to support Home Routed Session Breakout (HR-SBO) in Rel-18 to enable local traffic routing in VPLMN for Home Routed PDU Session. The VPLMN-specific offload policy may have different composition, depending on which MNO the Service Provider (SP) is in contract with. If there is an agreement between the SP and the VPLMN, then EC Application information (e.g., EDIs) is available in VPLMN. In this case, the HPLMN may, as a minimum, provide only the HR-SBO authorization indication to the VPLMN. However, it could be that HPLMN provides other services for the given UE and PDU Session, for other, non-EC applications. If that is the case, then traffic for these applications should not be allowed for SBO in VPLMN. Proposal 1: the VPLMN-specific offload policy should be possible to include traffic descriptors (e.g., destination IP ranges, ports, FQDNs) that are not allowed for HR-SBO. If there is an agreement between the SP and the HPLMN, then HPLMN needs to provide the offload policy necessary for HR-SBO to VPLMN. Proposal 2: the VPLMN-specific offload policy should be possible to include traffic descriptors (FQDNs and IP address ranges) that are allowed HR-SBO. Note: The traffic offload policy for the allowed and not allowed traffic should be mutually exclusive, i.e., either a list of allowed or a list of not allowed traffic descriptors should be sent, but not both |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding information of how to convey VPLMN specific offloading policy |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Structured offload policy for HR-SBO is not supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## FIRST CHANGE

#### 6.7.2.2	PDU Session for supporting HR-SBO in VPLMN

Figure 6.7.2.2-1: Procedure for PDU Session supporting HR-SBO in VPLMN

1.	During the registration procedure, the AMF receives the HR-SBO allowed indication per DNN/S-NSNAI from the UDM in the step 14b of the procedure in the clause 4.2.2.2.2 of TS 23.502 [3].

2.	During the PDU Session establishment procedure for Home-routed roaming as in clause 4.3.2.2.2 of TS 23.502 [3], if the AMF had received in SMF selection data from UDM the HR-SBO allowed indication for the DNN/S-NSSAI in the step 1, the AMF selects a V-SMF supporting HR-SBO and sends an HR-SBO allowed indication to the V-SMF in the step 2 and the step3a of the procedure in figure 4.3.2.2.2-1 in clause 4.3.2.2.2 of TS 23.502 [3].

Editor's note:	It is FFS how to route the DNS traffic between the UE and the V-EASDF where multiple DNN networks with the same IP address range are deployed in different HPLMNs or in the same HPLMN.

If the V-SMF supporting the HR-SBO receives the HR-SBO allowed indication from AMF, the V-SMF may:

-	select UL-CL and PSA functionalities based on UE location information and this indication in the step 4 of the Figure 4.3.2.2.2-1 of TS 23.502 [3].

NOTE 1:	The UL-CL UPF and PSA UPF can be co-located in the single V-UPF.

-	select a V-EASDF,

-	obtain the V-EASDF IP address based on local configuration, or invoke Neasdf_DNSContext_Create Request including the DNN, S-NSSAI, HPLMN ID and the UE IP address set to unspecified address as specified in clause 7.1.2.2 to obtain the V-EASDF IP address, and

-	send to the H-SMF the request for the establishment of the PDU Session supporting HR-SBO in VPLMN and the V-EASDF IP address to in the Nsmf_PDUSession_Create Request in the step 6 of the procedure in figure 4.3.2.2.2-1 in clause 4.3.2.2.2 of TS 23.502 [3].

The H-SMF authorizes the request for HR-SBO based on SM subscription data (i.e. HR-SBO authorization indication) in the step 7 of the procedure in the clause 4.3.2.2.2-1 of TS 23.502 [3].

Once the HR-SBO is authorized, the H-SMF requests and retrieves the optional VPLMN Specific Offloading Policy from H-PCF.

If HR-SBO is authorized for the PDU session, the H-SMF provides in the Nsmf_PDUSession_Create Response in the step 13 of the procedure in figure 4.3.2.2.2-1 in clause 4.3.2.2.2 of TS 23.502 [3] with the following information:

-	optional VPLMN Specific Offloading Information that may include FQDN range, IP range, session AMBR for the local part of DN and charging policy. The VPLMN specific offloading policy may refer to either allowed or not allowed traffic for HR-SBO (the latter is being used when the HPLMN would like to ensure that certain traffic are not allowed for HR-SBO). The VPLMN specific offloading policy for the allowed and not allowed traffic should be mutually exclusive, i.e., either a list of allowed or a list of not allowed traffic descriptors should be sent, but not both. The VPLMN specific offloading policy can be configured in the H-SMF per EC application and tagged with Offload Identifiers, and which VPLMN specific offloading policy to be sent to V-SMF can be indicated by these Offload Identifiers received from UDM/H-PCF. Similarly, H-SMF also sends the Offload Identifiers as labels to the list of HR-SBO related information to be sent to V-SMF for each application. If the given V-SMF has already received the VPLMN specific offloading policy for certain Offload Identifiers, this could be indicated to the H-SMF in any subsequent request to another HR-PDU Session from the same V-SMF, and the H-SMF will in this case send only the Offload Identifier as a response, if the policies for the given Offload Identifier have not changed. (if available in HPLMN based on the SLA between HPLMN and VPLMN),

-	the V-EASDF IP address (corresponding to clause 6.7.2.3) or DNS server IP address of HPLMN (corresponding to clause 6.7.2.5) as DNS server address to be sent to the UE via PCO, and

-	the DNS server address of HPLMN (to be used for DNS requests related with traffic not to be subject to HR-SBO),

-	the HR-SBO authorization result (i.e. whether HR-SBO request is authorized or not).

The H-SMF may indicate to the UE either that for the PDU Session the use of the EDC functionality is allowed or that for the PDU Session the use of the EDC functionality is required.

If the request for HR-SBO is not authorized and DNS context has been created, the V-SMF delete the DNS context from the selected V-EASDF, and the subsequent steps related to the EASDF in this procedure are skipped.

The detailed information of VPLMN Specific Offloading Policy is described in clause 6.4 of TS 23.503 [4].

NOTE:	The VPLMN Specific Offloading Policy can be prior configured in HPLMN based on the service level agreement between the VPLMN and HPLMN.

3.	The V-SMF configures the V-EASDF with the DNS handling rules using the received VPLMN specific offloading policy and may configure the V-EASDF with the DNS server address of HPLMN as default DNS server, after the step 13 of the procedure in figure 4.3.2.2.2-1 in clause 4.3.2.2.2 of TS 23.502 [3] if they are received from H-SMF in the step 2.

If the V-SMF has interacted with the V-EASDF in step 2, then the V-SMF invokes Neasdf_DNSContext_Update Request including UE IP address to complete the configuration of the context in the V-EASDF.

The V-SMF configures the V-UPF selected in the step 2 to forward DNS messages to V-EASDF.

4A.	EAS Discovery procedure with V-EASDF is performed as described in clause 6.7.2.3.

4B.	EAS Discovery procedure with Local DNS Server/Resolver is performed as described in clause 6.7.2.4.

4C.	EAS discovery procedure with V-EASDF using IP replacement mechanism as described in clause 6.7.2.5.

Editor's note:	It is FFS if and how DNS with security (i.e. DNSSEC, DoT and DoH) can be supported when using IP replacement (step 4C and step 1b of clause 6.7.2.3).

## END of CHANGES