SA WG2 Meeting #157	S2-2308173

# Berlin, Germany, May 22nd – 26th, 2023           	  			  	  (revision of S2-2307941

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0155 | rev | 2 | Current version: | 18.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | EAS Re-discovery in HR-SBO context |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell,[ China Unicom, Huawei, Hsilicon, CATT, LGE] |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGE_Ph2 |  |  |  |  |  | Date: |  |  | 2023-05-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | inter V-SMF intra VPLMN and inter PLMN Hand-over in HR-SBO case is not fully specified EAS Re-discovery in HR-SBO context is not fully specified |  |  |  |  |  |  |  |  |
| Summary of change: |  | Fully Specify inter V-SMF intra VPLMN and inter PLMN Hand-over in HR-SBO case EAS Re-discovery in HR-SBO context is not fully specified |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete EDGE_Ph2 KI1 related specifications |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.3.2 ; 6.7.2.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

fIrst  CHANGE

#### 6.7.3.2	Network triggered EAS change in HR-SBO context

Figure 6.7.3.2-1 shows the procedure of EAS re-discovery when HR-SBO is supported and allowed in the target serving PLMN.

Editor’s Note: the case where HR-SBO is not possible for the target serving PLMN is FFS

Figure 6.7.3.2-1: Network triggered EAS rediscovery HR-SBO context procedure

1	The procedure described in clause 6.7.2.X is performed: inter V-SMF intra VPLMN or inter PLMN N2 Hand-over in HR-SBO case

As part of this procedure, if the AF had subscribed to the corresponding event and a serving PLMN change occurred towards a PLMN where local traffic offload is possible for the PDU Session, the H-SMF notifies the AF, indicating the new serving PLMN ID. This may take place as soon as the H-SMF has received an indication of Handover Complete (see clause 6.7.2.X).

NOTE:	via this mechanism the AF is aware of the PLMN to contact to issue traffic influence requests. The AF is assumed to check whether it has an SLA with the new serving PLMN. If the AF has no SLA with the new serving VPLMN the AF interacts with H-NEF to have EAS used that are reached via HPLMN UPF(s) .

This may trigger the AF triggered EAS relocation / rediscovery as defined in step 1b of Figure 6.2.3.3-1.

2a.	For AF triggered EAS rediscovery via interacting with HPLMN  , the AF may indicate the EAS rediscovery for the impacted applications, which are identified by Application Identifier(s), to the H-SMF via the H-PCF using the AF influence on traffic routing procedure as described in clause 4.3.6 of TS 23.502 [3]. The AF may also provide IP replacement information and target DNAI together with an indication of the PLMN assopciated with this target DNAI i.e. the serving PLMN ID.

2b.	For AF triggered EAS rediscovery via interacting with target VPLMN, the AF may indicate the EAS rediscovery for the impacted applications via the V-NEF using the procedure described in clause 4.3.6 of TS 23.502 [3]. This may trigger step 2 of of Figure 6.2.3.3-1 where the SMF that initiates PDU Session modification is the V-SMF that initiates Nsmf_PDUSession_Update request with the requested PCO; The AF may also provide IP replacement information and target DNAI

3a	(For AF triggered EAS rediscovery via interacting with HPLMN case) The AF request relayed via a PCC rule to H-SMF may trigger step 2 of of Figure 6.2.3.3-1 where the SMF that initiates the PDU Session modification is the H-SMF: the H-SMF issues a Nsmf_PDUSession_Update request. This Nsmf_PDUSession_Update request may contain IP replacement information and target DNAI provided by AF in step 2.

Editor’s Note: whether this may trigger an intra VPLMN V-SMF Change (induced by target DNAI) and if yes how, is FFS

3b.	(For AF triggered EAS rediscovery via interacting with VPLMN case) The H-SMF initiates Nsmf_PDUSession_Update Response towards the (target) V-SMF including the PCO information to be sent to the UE. In intra PLMN V-SMF change the target V-SMF may use the source and target DNAI to determine the Imapct field to be sent to the UE. In inter PLMN mobility the target V-SMF provides EAS rediscovery information without an Impact field.

4.	The V-SMF initiates PDU Session Modification command including the PCO to the UE.

The PCO may includes EAS rediscovery indication (optional) and the impact field (optional).

5 	The V-SMF may configure the V-UPF (UL CL) IP replacement information provided by AF in step 2

NEXT CHANGE All text is NEWchanges indicate changes in the new revision

#### 6.7.2.X	inter V-SMF (intra VPLMN and inter PLMN) N2 Hand-over in HR-SBO case

This clause defines the procedure for inter V-SMF intra VPLMN and inter PLMN N2 based Hand-over for HR-SBO PDU sessions. This procedure is based on Inter NG-RAN node N2 based handover with I-SMF insertion/change/removal defined in TS 23.502 [3] clause 4.23.7.3.

The flow assumes the UE has an HR-SBO PDU session established using a source serving PLMN (e.g. involving a source V-SMF) and there is a HO to a target serving PLMN (e.g. involving a target V-SMF). The flow is also meant to apply when source or target serving network (before or after an inter PLMN HO) is the HPLMN;

Editor’s Note: the case where HR-SBO is not possible for the target serving PLMN is FFS

Figure 6.7.3.2-1: inter V-SMF intra VPLMN and inter PLMN N2 based Hand-over in HR-SBO case

NOTE: 	the flow above does not show all steps in clause 4.23.7.3 of TS 23.502 [3] but only those that are impacted by the support of HR-SBO

The procedure described in clauses 4.23.17 and 4.23.7.3 (N2 based handover) of TS 23.502 [3] is performed by replacing I-SMF with V-SMF and SMF with H-SMF with following modifications.

0.	The AF may subscribe onto H-SMF for the event related with a mobility of the PDU Session towards a serving PLMN where local traffic offload is possible, i.e. mobility of the PDU Session either towards HPLMN or towards a VPLMN where HR-SBO is possible i.e. supported and allowed

- Step 1 of TS 23.502 [3] Figure 4.23.7.3.2-1 (initiation of the N2 based handover). For an intra PLMN HO, the source AMF provides the target AMF with whether HR-SBO is allowed for each PDU session of the UE

Steps 3 to 5 are skipped if the UE moves with V-SMF removal

For an inter PLMN HO, the target T-AMF uses local policies related to the SLA with the HPLMN of the UE to determine whether HR-SBO allowed indication is sent to the V-SMF(s) for the PDU Sessions of the UE,

NOTE 1:	This means that if HR-SBO is allowed only for some users of the HPLMN and is not allowed for the UE being handed over, this will only be detected at a later step of the procedure i.e. at step 8.

2.	2a (V-SMF insertion or V-SMF change) Step 2 and 3 of TS 23.502 [3] Figure 4.23.7.3.2-1 take place. If the T-AMF consider that HR SBO is possible, the T-AMF selects a V-SMF supporting HR-SBO and sends an HR-SBO allowed indication to the target V-SMF in Nsmf_PDUSession_CreateSMContext Request as part of the step 3 of the procedure in figure 4.23.7.3.2 of TS 23.502 [3].

2b (V-SMF removal) Step 9 to 12b of TS 23.502 [3] Figure 4.23.7.3.2-1 take place: the target AMF sends to H-SMF a Nsmf_PDUSession_CreateSMContext request  as in Step 9 of TS 23.502 [3] Figure 4.23.7.3.2-1. The H-SMF selects an EASDF in HPLMN and configures the DNS context in this EASDF (Neasdf_DNSContext_Create) and may select and configure UPF(s) in HPLMN;

3.	steps 4 or 5 of TS 23.502 [3] Figure 4.23.7.3.2-1.

3a.	(V-SMF insertion case) The (target) V-SMF retrieves SM context from H-SMF using Nsmf_PDUSession_Context Request/Response, followed by the Nsmf_PDUSession_Context Request/Response.

3b.	(V-SMF change case) The target V-SMF retrieves SM context from the source V-SMF using Nsmf_PDUSession_Context Request/Response indicating HR-SBO support in the Request in the case of an intra PLMN V-SMF case.

In step 3b, if source and target V-SMFs belong to same VPLMN, the SM context includes, Authorization Result for HR-, VPLMN Specific Offloading Policy,  and (if previously received by source V-SMF from H-SMF) the HPLMN address information, and the DNS Server address provided by the HPLMN.

4.	The (target) V-SMF may (re-)select a (new) V-EASDF e.g., based on the target UE location. The (target) V-SMF may invoke Neasdf_DNSContext_Create including the DNN, S-NSSAI and HPLMN ID to obtain the new V-EASDF address. In case of V-SMF insertion, the UE IP address is set to the unspecified address in Neasdf_DNSContext_Create as specified in clause 7.1.2.2.

In the case of inter PLMN mobility or in the case of intra PLMN mobility with V-SMF change, the target V-SMF shall select a new V-EASDF if it is configured to use an EASDF for the PDU Session.

In the case of intra PLMN mobility without V-SMF change, the V-SMF may select a new V-EASDF or reuse the existing V-EASDF.

To support EAS discovery with local DNS server as described in clause 6.7.2.4, the (target) V-SMF (re-)selects a local DNS server, e.g. based on the target DNAI corresponding to the target UE location.

5	The (target) V-SMF may perform UL CL/BP and local PSA insertion/change/removal as described in step 2 of Figure 6.7.2.2-1.

NOTE 2:	When there are other V-UPF(s) between UL CL/BP and H-UPF, the (target) V-SMF sets up user plane between this ULCL/BP and the V-UPF.

6	The H-SMF (V-SMF removal) or the target V-SMF (V-SMF insertion or V-SMF change) answers to the target AMF with a Nsmf_PDUSession_CreateSMContext Response as in Step 8 (V-SMF insertion or V-SMF change)  or step 13 (V-SMF removal) of TS 23.502 [3] Figure 4.23.7.3.2-1

The hand-over further continues as defined in steps 15 to last of Figure 4.23.7.3.2-1 of TS 23.502 [3] and then per clause 4.23.7.3.3 of TS 23.502 [3], with clause 4.23.7.3.3 of TS 23.502 [3] modified as follows,

7	(V-SMF insertion or V-SMF change) when the T-AMF sends to the Target V-SMF an indication of Handover Complete within Nsmf_PDUSession_UpdateSMContext Request (at step 2 of TS 23.502 [3] Figure 4.23.7.3.3-1) the target V-SMF invokes Nsmf_PDUSession_Update Request to the H-SMF. The V-SMF indicates HR-SBO support (even in intra PLMN V-SMF change). If a V-EASDF or a local DNS server has been selected in step 4, the selected V-EASDF or local DNS server address is also provided in the request (the V-SMF may not provide the selected V-EASDF or local DNS server address if V-SMF is configured to use EAS discovery with V-EASDF using IP replacement mechanism corresponding to clause 6.7.2.5); if this is an inter PLMN change, and the UE indicated support of the feature, the V-SMF may provide an EAS rediscovery indication, to the H-SMF. This Nsmf_PDUSession_UpdateSMContext Request triggers steps 8 to 11

8.	(V-SMF insertion or V-SMF change) If the UE has changed of serving VPLMN or has moved from HPLMN to a VPLMN the H-SMF

-	invokes Nudm_SDM_Get service to get subscription data from UDM by providing serving PLMN ID to get subscription data specific to the Serving PLMN.

-	authorizes the request for HR-SBO based on the SM subscription data (i.e., HR-SBO authorization indication) and

-	If the HR-SBO is authorized, retrieves the VPLMN Specific Offloading Policy from H-PCF.

9.	(V-SMF insertion or V-SMF change) As defined in step 8 of clause 4.23.7.3.3 of TS 23.502 [3], the H-SMF invokes Nsmf_PDUSession_Update Response, providing the same HR-SBO related information as sent by the H-SMF in step 2 of Figure 6.7.2.2-1. The H-SMF furthermore provides V-SMF with EAS rediscovery indication to be sent to the UE if received at step7.  The target V-SMF stores the DNS information for the UE until it receives an indication of HO completion. This information is also provided for intra VPLMN V-SMF change.

If the request for HR-SBO is not authorized and a V-EASDF context had been created, the V-SMF deletes the DNS context from the selected V-EASDF, and the subsequent steps related to the HR-SBO in this procedure are skipped.

10	(V-SMF insertion or V-SMF change) The (target) V-SMF configures the V-EASDF with the DNS handling rules using the received VPLMN specific offloading policy as defined in step 3 of Figure 6.7.2.2-1.

If the (target) V-SMF has interacted with the V-EASDF in step 4, the V-SMF invokes Neasdf_DNSContext_Update Request including UE IP address to complete the configuration of the context in the V-EASDF.

The (target) V-SMF may configure the V-UPF selected in the step 5 to forward DNS messages to V-EASDF.

When the V-SMF is configured to use EAS discovery with V-EASDF using IP replacement mechanism as described in clause 6.7.2.5, the (target) V-SMF configures the (target) UPF in the VPLMN with the IP replacement information (i.e. IP address of the DNS server provided by the HPLMN, the selected V-EASDF IP address and port number and DNS traffic filtering rules related with when IP replacement applies).

11	(V-SMF insertion or V-SMF change) the target V-SMF sends the DNS information for the UE (received at step 9) and EAS rediscovery indication via PCO in a PDU session Modification Command sent to the UE

(In the case of V-SMF removal) At step 10 of Figure 4.23.7.3.3-1 of TS 23.502 [3], when receiving Nsmf_PDUSession_UpdateSMContext Request (Handover Complete indication), the H-SMF sends the DNS information for the UE via PCO in a PDU session Modification Command sent to the UE

12 In case of V-SMF change or in case of V-SMF removal, when the S-AMF invokes Nsmf_PDUSession_ReleaseSMContext Request to inform the Source V-SMF to release the SM context of the PDU Session, e.g. at step3a or 11a of TS 23.502 [3] Figure 4.23.7.3.3-1, the DNS context in the old (V-)EASDF is removed by the (source) V-SMF using Neasdf_DNSContext_Delete service.

In case of V-SMF insertion, when the target V-SMF sends Nsmf_PDUSession_Update Request with a Handover Complete Indication at step 6 of TS 23.502 [3] Figure 4.23.7.3.3-1, the DNS context in the old EASDF is removed by the H-SMF using Neasdf_DNSContext_Delete service

If the serving PLMN and the V-SMF has not changed the V-SMF is responsible of determining whether or not and when to remove the context in the old V-EASDF.

NOTE3 :	Re-selecting the old V-EASDF to reuse the existing DNS context in the case of intra PLMN inter V-VSMF mobility is not supported in this release of the specification.

13.	if the AF had subscribed in step 0 and a serving PLMN change occurred towards a PLMN where local traffic offload is possible for the PDU Session, the H-SMF notifies the AF indicating the target serving PLMN ID. This may take place as soon as the H-SMF has received an indication of Handover Complete

14.	EAS discovery procedure by UE may take place: EAS Discovery procedure with V-EASDF for HR-SBO (§ 6.7.2.3) / with Local DNS for HR-SBO (§6.7.2.4) / with V-EASDF using IP replacement mechanism (§6.7.2.5)

NEXT CHANGE

#### 6.7.2.Y	inter V-SMF (intra VPLMN and inter PLMN) IDLE mode mobility in HR-SBO case

This clause defines the procedure for inter V-SMF intra VPLMN and inter PLMN IDLE mode mobility for HR-SBO PDU sessions. This procedure is based on IDLE mode mobility with I-SMF insertion/change/removal defined in TS 23.502 [3] clause 4.23.

- The UE establishes a HR-SBO PDU Session described in clause 6.7.2.2.

- The UE initiates Mobility Registration Update procedure to the Target AMF as described in TS 23.502 [3] clause 4.2.2.2.2. During the registration procedure, the target AMF (in roaming case) receives the SMF selection data including HR-SBO allowed indication per DNN/S-NSNAI from the UDM in the step 14b of the procedure in the clause 4.2.2.2.2 of TS 23.502 [3].

- (V-SMF insertion or V-SMF change) the Target AMF selects a V-SMF and sends the received HR-SBO allowed indication for the established Session for DNN, S-NSSAI in the PDUSession_SMContext Create Request to the Target V-SMF.

- As in step 4 of Figure 6.7.3.2-1

- As in step 5 of Figure 6.7.3.2-1

- The target V-SMF invokes Nsmf_PDUSession_Update Request to the H-SMF as in step 7 of Figure 6.7.3.2-1

- As in step 8 of Figure 6.7.3.2-1

- As in step 9 of Figure 6.7.3.2-1, The H-SMF sends the VPLMN Specific Authorization Information in the Nsmf_PDUSession_Update Response message to the V-SMF.

- As in step 9 of Figure 6.7.3.2-1, the V-SMf may configure the V-EASDF, V-UPF based on the received VPLMN Specific Authorization Information

- The target V-SMF answers to the target AMf and the registration procedure continues as defined in TS 23.502 [3] clause 4.2.2.2.2.

Editor’s Note: this procedure needs a figure and more precise description needs to be added

END OF CHANGES