3GPP TSG-SA WG2 Meeting #157	S2-2308245

# 22 – 26 May Berlin, Germany 2023		     						   		       (revision of S2-2308217)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4031 | rev | 5 | Current version: | 18.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Service Experience filtering criteria in assistance to UE member selection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital, Europe, Ltd., MediaTek Inc., Vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIMLsys |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to TS 23.501, clause 5.46.2, “The 5G system may support UE member selection assistance functionality to assist the AF in selecting member UE(s) that can be used in application operations such as AI/ML based applications”. The functionality entails an AF request that includes a list of target UE members, an optional time window(s) and one or more filtering criteria. Furthemore, as per current Editor’s note in clause 4.15.13.2, “The list of UE Member filtering criteria and their corresponding actions are FFS”. In addtion, as part of the conclusions from 23.700-80, clause 8.7.2, whethere Service Experience analytics can be used as filtering criteria to assistt in UE member selection can be discussed during normative phase. Therefore this CR discusses how Service Experience analytics can be used as filtering criteria to assistt in UE member selection. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce new Filtering Criteria considering Service Experience, enbling AF to request UE member selection assistance based on Service Experience for a particular Service Experience Type and with the possibility to define thresholds consdiering contribution weights |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE member selection functionality won’t be able to use Service Experince analytics, useful to detect e.g., unperforming UEs, as filtering criteria |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.13.2, 4.15.13.X, 4.15.13.X.1, 4.15.13.X.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | CR ... TS 23.501, CR3893 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This is a revision of S2-2305443 addressing some issues in the steps 4-8 of the figure and step 9 of the procedure in clause 4.15.13.X.2. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

#### 4.15.13.2	UE Member Filtering Criteria for 5GS assistance to UE member selection

Table 4.15.13.2-1 provides a summary of the UE member filtering criteria that the AF may request.

Table 4.15.13.2-1: Description of UE Member filtering criteria

| UE Member filtering criteria | Description of filtering criterion | UE filtering information | Detailed description clause |
| --- | --- | --- | --- |
| QoS | The Quality of Service of the selected UEs match or exceed the QoS of the filtering criteria |  | 4.15.13.3 |
| Access Type and/or RAT Type of the PDU Session | Indicate the Access Type and/or RAT Type of the selected UE for the PDU Session used by the application (e.g. 3GPP/NR, Non-3GPP/WLAN, additional Access Type and RAT Type for MA PDU session) | Service operation: Nsmf_EventExposure Filter: a list of GPSI(s) or SUPI(s), DNN/S-NSSAI Event ID: Change of Access Type and/or Change of RAT Type | This can be provided by existing service operations, events, and procedures. |
| End-to-end data volume transfer time | Indicate the end-to-end data volume transfer time that refers to a time for completing the transmission of a specific data volume between UE to AF | Service operation: Nnwdaf_AnalyticsSubscription/ Nnwdaf_AnalyticsInfo Filter: Analytics ID = End-to-end data volume transfer time | Clause 6.18 of TS 23.288 [50]. |
| UE current location | Indicate the certain area that the selected UE currently located in. | Service operation: Namf_EventExposure Filter: a list of GPSI(s) or SUPI(s) | 4.15.13.3 |
| UE historical location | Indicate the certain area that the selected UE appeared in a historical period of time | Service operation: Nnwdaf_AnalyticsSubscription Filter: Analytics ID = UE mobility, Filters include "Visited AoI = Target AOI" and "target period = historical nomadic period | 4.15.13.3 |
| UE direction | Indicate the selected UEs should include different moving direction | Service operation: Nnwdaf_AnalyticsSubscription Filter: Analytics ID= UE Mobility, Direction | 4.15.13.3 |
| Service Experience | Indicates selected UEs fulfilling ceratin Service Experience criteria e.g., MOS value | Service operation: Nnwdaf_AnalyticsInfo_Request/Subscribe Analytics ID=Service Experience Filter: S-NSSAI, DNN, Application ID, DNAI, AoI, Service Experience Contribution weight reporting threshold (NOTE 1), Service Experience Type (NOTE 2) | 4.15.13.X |
| NOTE 1: The Service Experience Contribution Weights signal the relative importance of each UE’s Service Experience value (i.e., MOS), as defined in TS 23.288 [50]. For example, it might be that the service experience of a UE in relation to other UEs may not be as important e.g., because the data provided by such UE is not as critical to the service. NOTE 2: Indicates the type of service experience analytics, e.g., AIML traffic where a customized MoS apply. |  |  |  |

Editor's note:	The list of UE Member filtering criteria and their corresponding actions are FFS.

Editor's note:	Each Filter Criteria may add a new clause where the specific procedures for such filter criteria are described. For example:"4.15.13.3	Specific procedure for criterial foo".

* * * Next Change * * *

#### 4.15.13.X	Specific procedure for Service Experience UE member filtering criteria

##### 4.15.13.X.1	General

An AF may invoke Nnef_UEMemberSelectionAssistance_Subscribe service operation with a Service Experience filtering criteria for receiving a list of UEs that match or exceed such criteria.

In addition to the mandatory parameters, the AF also includes in the request:

- Service Experience filtering criteria.

- Optionally, an Area of Interest: location area of the candidate UEs.

- Optional time windows for selecting the candidate UEs: start time and stop time.

- Optional Service Experience contribution weights

- Optionaly Service Experience Type

The AF provides a Service Experience filtering criteria, including contribution weights associated to location, e.g., AoI or DNAI, time window, Application ID and Service Experience type. E.g., contribution weight may be provided to favor Service Experience type relative to AIML traffic in a particular location.

##### 4.15.13.X.2	UE Member Assistance with Service Experience filtering criteria

Figure 4.15.X.3-2: 5GC assistance to UE member selection based on Service Experience

- AF subscribes the member selection assistance functionality by sending Nnef_ UEMemberSelectionAssistance_subscribe request including the Application Identity, AoI, DNAI(s), Service Experiece Type and contribution weights associated to location, time window, Application ID and Service Experience type,

2.	NEF verifies the authorization of the AF Request and identifies which information needs to be collected and executed based on the Service Experience filtering criteria provided by the AF.

3.	If an Application ID is present in the request from the AF, the NEF derives the S-NSSAI, DNN and DNAI(s) to which this Application ID have access. Due to the absence of a time window in the request from the AF, NEF discovers querying UDM and NRF the SMFs that are deployed in the Area of Interest. The NEF may also restrict the discovery to those SMFs that serve some S-NSSAI and DNN combination (this S-NSSAI and DNN combination has been derived from the Application Id or AF Id).

4.	NEF sends an Analytics request/subscribe to NWDAF by invoking a Nnwdaf_AnalyticsInfo_Request or a Nnwdaf_AnalyticsSubscription_Subscribe, including Analytics ID = Service Experience, Application ID, S-NSSAI, AoI, DNAI(s), and target UEs based on the initial list obtained from the AF.

5-7.	 Procedures as specified in clause 6.4.6 of TS 23.288 [x] are followed.

8.	The NWDAF provides the data analytics, i.e., the observed Service Experience (which can be a range of values) to the consumer NF by means of either Nnwdaf_AnalyticsInfo_Request response or Nnwdaf_AnalyticsSubscription_Notify, depending on the service used in step 4.

9.	Based on the Analytics report received from the NWDAF, NEF consolidates results and derives the list(s) of candidate UE(s). The NEF may use the Service Experience type provided by the AF consumer in the filtering information and use operator policies to interpret a customized MoS. The NEF may use the contribution weight associated to an application and a Service Experience Type (e.g., AI/ML traffic) and apply them to a location and time window, as provided by the AF consumer, to be used as reporting thresholds when selecting UE candidate members.E.g.,The NEF may select a UE with a specific Service Experience Type as a UE member candidate, if the associated Service Experience fulfils the threshold for Service Experience filtering criteria provided by the AF.

10.	NEF sends a Nnef_ UEMemberSelectionAssistance_Notify request to the AF including the list(s) of candidate UE(s) and additional information.

* * * End of Changes * * *