3GPP TSG SA WG2 Meeting #157	S2-2307469

# May 22 - 26, 2023, Berlin, Germany					      (Revision of S2-2306823,S2-2304970)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1303 | rev | 2 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of DCMF service to support media processing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC |  |  |  |  |  | Date: |  |  | 2023-04-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | It has been discussed whether AR media function is also integrated into DCMF, with corresponding editor’s notes in current sepcifciation. The writer of this contribution supports to integrate AR media function into DCMF for the simplicity. The AR media function should be separate from the functions of DC resource mangement, because: 1) the resources for AR media handling are separate from the DC resources; 2) the timing of establishing the resources are different, i.e. AR media handling should be correlated to specific DC media streams, so the resouces for AR media handling are established after the DC resources are reserved; 3) the AR media processing is separate from DC media routing, i.e. the media for AR processing is taken from ingress DC termination and sent back to egress termination. Therefore it is proposed to upate DCMF services to manage media resources for AR media processing to avoid mixing the media processing with DC management. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove editor’s note about AR media fucntions and DCMF services is updated to manage media resources for AR media processing. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AA.2.5.1, AA.2.5.2, AA.2.5.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

## AA.2.5	MF Services

### AA.2.5.1		General

# The following table illustrates the MF Services and Service Operations.

# Table AA.2.5.1-1: NF services provided by the DCMF

| NF service | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nmf_MediaResourceManagement (MRM) | Create | Request/Response | IMS AS |
|  | Update | Request/Response | IMS AS |
|  | Delete | Request/Response | IMS AS |

### AA.2.5.2		Nmf_MediaResourceManagement (MRM) service

#### AA.2.5.2.1	General

Service description: This service enables the consumer to create, update and delete media resources related to IMS Data Channel.

The media resource represents a media context including one or multiple media terminations. A media termination includes media resources for one or multiple media streams on the Mb interface.

# Each media stream is identified by a media ID.

NOTE:	Termination is a concept widely used in media protocols like H.248. When a media stream, e.g. application data channel, pass through the DCMF, there is one termination for the input stream and one termination for the output stream. In this case, the two terminations can use the same media ID.

#### AA.2.5.2.2	Nmf_MRM_Create service operation

# Service operation name: Nmf_MRM_Create

Description: The consumer NF requests the MF to create a media context including one or multiple media terminations and reserve media resources for one or multiple media streams of Mb interface in each termination. The consumer NF may also include media resource descriptions requested by the upper layer, e.g. DCSF, to be reserved on the MF.

# Inputs, Required: List of Media Termination Descriptors.

# Each Media Termination Descriptor of the list includes:

-	List of Media Stream Descriptors belonging to the Termination.

# Each Media Stream Descriptor of the list includes:

-	Media ID: A unique identity of the media stream within the media context instance. The MediaID value is set by the consumer.

NOTE 1:	Different Termination IDs associated with the same Media ID represent the media stream flow for data forwarding.

-	Remote Mb Specification: Media Specification specifying SDP parameters of a media endpoint e.g. IP address and ports of the UE. This parameter is used to establish IMS DC or audio/video media path between MF and UE.

NOTE 2:	In the case of reserving termination media resource before receiving media negotiation information from the UE, Remote Mb Specification should be set to Null.

-	DC Media Specification: Description of additional media specification information needed for data channel media stream from application layer, which includes:

-	Media proxy configuration (HTTP or UDP) applicable to the media flow.

-	MDC1/MDC2 media endpoint address.

-	Replacement HTTP URL allocated by the application layer representing the application list (e.g. graphical user interface) offered to the IMS subscriber via the MDC1 interface.

NOTE 3:	In case of P2P application data channel without connecting to the application layer network function, DC Media Specification should be set to Null.

-	AR Media Specification: Description of additional media specification information needed for AR communication services from application layer, which includes:

-	Media Processing Specification: It specifies the how the media stream should be processed.

NOTE:	The detail of Processing Specification is defined by SA4 and needs to be further aligned.

# Inputs, Optional: None.

Outputs, Required: Result indication, Media Resource Context ID, List of allocated Media Termination Descriptor Resource information.

Media Resource Context ID uniquely identifies the media resources created by the DCMF. This identity shall be used as reference when updating or deleting the created media resource.

List of allocated Media Termination Descriptor Resource information includes one or multiple Media Termination Descriptor resources allocated. The list will include same number of terminations, in the same order as provided in the request.

# Media Termination Descriptor Resource includes:

-	Termination ID: Unique identity allocated by the consumer. The identity shall be used when updating/deleting the Media Termination.

-	List of Media Stream Descriptor Resource information elements allocated within the media termination.

# Media Stream Descriptor resource information includes:

-	Media ID: Unique identity of the media stream within the media context instance allocated by the consumer.

-	Local Mb Specification: Media specification specifying SDP parameters representing the media endpoint of the provider e.g. IP address and ports used in SDP response etc.

-	DC Media Resource Sescription: Optional application specific media resource specification specifying the information of data channel, which includes:

-	MF MDC1/MDC2 media point information (IP address/port number, etc.) which shall be relayed to the DCSF by the consumer NF.

# Outputs, Optional: None.

#### AA.2.5.2.3	Nmf_MRM_Update service operation

# Service operation name: Nmf_MRM_Update

Description: The consumer NF requests the MF to update one or multiple media resources within a specific media resource context. Terminations and/or Media Streams can be added, modified or deleted within an Update request.

# Inputs, Required: Media Resource Context ID, List of Media Termination Descriptors to be updated.

# Media Resource Context ID specifies the media context resource to be updated.

List of Media Termination Descriptors includes one or multiple updated Media Stream Descriptors, which provides the media stream information that needs to be updated.

# Inputs, Optional: None.

# Outputs, Required: Result indication, List of Media Termination Descriptor Resource information.

List of Media Termination Descriptor Resource information includes one or multiple Media Stream Descriptor Resources impacted by the Update request.

# Outputs, Optional: None.

#### AA.2.5.2.4	Nmf_MRM_Delete service operation

# Service operation name: Nmf_MRM_Delete

Description: The consumer NF requests the DCMF to delete a specific media context resource including all existing terminations and media streams.

# Inputs, Required: Media Context Resource ID.

# Media Context Resource ID specifies the media context resource to be deleted.

# Inputs, Optional: None.

# Outputs, Required: Result indication.

# Outputs, Optional: None.

**** End of Changes ****