3GPP TSG-SA2 Meeting #157	S2-2307650

# Berlin, Germany, May 22 – 26, 2023                                                 revision of S2-2306757

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0286 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of 5G ProSe UE-to-UE Relay reselection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO, Interdigital |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2023-05-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the TR conclusion,we have agreed both U2U relay selection and negotiated U2U relay reselection can be used for U2U relay reselection. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify reusing U2U relay selection and negotiated U2U relay reselection are two alternatives for U2U relay reselection. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not clear there are two alternatives used for U2U relay reselection. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change* * * *

6.7.4	5G ProSe UE-to-UE Relay reselection

6.7.4.1	General

After being connected to the 5G ProSe UE-to-UE Relay, the 5G ProSe End UEs keep performing the measurement of the signal strength of PC5 unicast link with the 5G ProSe UE-to-UE Relay for relay reselection.

For 5G ProSe UE-to-UE Relay reselection, the 5G ProSe UE-to-UE Relay discovery procedures defined in clause 6.3.2.4 or the negotiated 5G ProSe UE-to-UE Relay reselection procedure defined in clause 6.7.4.2 or clause 6.7.4.3 may be used.

In the negotiated UE-to-UE Relay reselection defined in clause 6.7.4.2 or clause 6.7.4.3, one 5G ProSe End UE initiates the UE-to-UE Relay reselection procedure, the 5G ProSe End UEs can negotiate a new 5G ProSe UE-to-UE Relay using the existing connection and to establish the communication via the reselected 5G ProSe UE-to-UE Relay prior to releasing the communication via the current 5G ProSe UE-to-UE Relay.

6.7.4.2	Negotiated 5G ProSe Layer-2 UE-to-UE Relay reselection

Depicted in Figure 6.7.4.2-1 is the procedure for the negotiated 5G ProSe Layer-2 UE-to-UE Relay reselection.

Figure 6.7.4.2-1: Negotiated 5G ProSe Layer-2 UE-to-UE Relay reselection

1.	A PC5 unicast link is established between the 5G ProSe End UEs via a 5G ProSe UE-to-UE Relay, based on the procedure defined in clause 6.7.2.

2.	The initiating 5G ProSe End UE determines, e.g. based on PC5 signal strength, to perform UE-to-UE Relay reselection and obtains a list of candidate UE-to-UE Relays. The initiating 5G ProSe End UE may receive UE-to-UE Relay Discovery Announcement messages from 5G ProSe UE-to-UE Relays or initiate the 5G ProSe UE-to-UE Relay discovery procedures to find the candidate 5G ProSe UE-to-UE Relays. The initiating 5G ProSe End UE determines the candidate 5G ProSe UE-to-UE Relays e.g., based on the PC5 signal strength of the received UE-to-UE Relay Discovery Announcement message, RSC within the UE-to-UE Relay Discovery Announcement message.

3.	The initiating 5G ProSe End UE sends a Link Modification Request message to the responding 5G ProSe End UE which includes a Relay re-selection indication, the User Info ID(s) of the candidate 5G ProSe UE-to-UE Relay(s), and optionally the Layer-2 ID(s) of the candidate 5G ProSe UE-to-UE Relay(s) and security information.

4.	The responding 5G ProSe End UE selects a new 5G ProSe UE-to-UE Relay from the candidate 5G ProSe UE-to-UE Relays, based on the Relay re-selection indication in the Link Modification Request message. If the responding 5G ProSe End UE has not received a UE-to-UE Relay Discovery Announcement message from a candidate 5G ProSe UE-to-UE Relay or does not have a PC5 connection with the candidate 5G ProSe UE-to-UE Relay associated with the same RSC, then the responding 5G ProSe End UE may perform the 5G ProSe UE-to-UE Relay discovery procedure with the User Info ID of a candidate 5G ProSe UE-to-UE Relay in discovery message, and may set the Layer-2 ID of the candidate 5G ProSe UE-to-UE Relay, if received at step 3, as the Destination Layer-2 ID to carry the discovery message. The PC5 signal strength of the UE-to-UE Relay Discovery Announcement message or UE-to-UE Relay Discovery Response message may be used to select the new 5G ProSe UE-to-UE Relay.

5.	The responding 5G ProSe End UE sends a Link Modification Accept message to the initating 5G ProSe End UE, including the User Info ID of the new 5G ProSe UE-to-UE Relay and security information.

6.	5G ProSe End UEs set up PC5 unicast links, if not already set up, with the new 5G ProSe UE-to-UE Relay, by reusing the procedure defined in clause 6.7.2 and the PC5 unicast is link established between 5G ProSe End UEs via the new 5G ProSe UE-to-UE Relay. The 5G ProSe End UEs switch the data traffic via the new 5G ProSe UE-to-UE Relay. The security information is used to verify that the new link has been set up successfully.

NOTE 1:	The security information contents and usage will be defined by SA WG3.

NOTE 2:	Whether a 5G ProSe End UE releases a PC5 unicast link with the original 5G ProSe UE-to-UE Relay after reselection depends on whether the PC5 unicast link is still required and UE implementation.

6.7.4.3	Negotiated 5G ProSe Layer-3 UE-to-UE Relay reselection

Depicted in Figure 6.7.4.3-1 is the procedure for the negotiated 5G ProSe Layer-3 UE-to-UE Relay reselection.

Figure 6.7.4.3-1: Negotiated 5G ProSe Layer-3 UE-to-UE Relay reselection

1.	5G ProSe End UEs have set up PC5 unicast links with a 5G ProSe UE-to-UE Relay, based on the procedure defined in clause 6.7.1.

2.	The 5G ProSe End UEs have additionally set up PC5 unicast links with a 5G ProSe UE-to-UE Relay, based on the procedure defined in clause 6.7.1.

3.	The 5G ProSe End UEs are transferring traffic via the 5G ProSe UE-to-UE Relay.

4.	The initiating 5G ProSe End UE determines, e.g. based on PC5 signal strength, to perform UE-to-UE Relay reselection and obtains a list of candidate UE-to-UE Relays. The initiating 5G ProSe End UE may receive UE-to-UE Relay Discovery Announcement messages from 5G ProSe UE-to-UE Relays or initiate the 5G ProSe UE-to-UE Relay discovery procedures to find the candidate 5G ProSe UE-to-UE Relays. The initiating 5G ProSe End UE determines the candidate 5G ProSe UE-to-UE Relays e.g., based on the PC5 signal strength of the received UE-to-UE Relay Discovery Announcement message, RSC within the UE-to-UE Relay Discovery Announcement message.

5.	The initiating 5G ProSe End UE sends a Link Modification Request message to the responding 5G ProSe UE-to-UE Relay, which includes a Relay re-selection indication, the User Info ID(s) of the candidate 5G ProSe UE-to-UE Relay(s), the IP addresses of the responding 5G ProSe End UEs and optionally the Layer-2 ID(s) of the candidate 5G ProSe UE-to-UE Relay(s).

Multiple 5G ProSe End UEs IP addresses may be included when the initiating 5G ProSe End UE is communicating with multiple 5G ProSe End UEs via the 5G ProSe UE-to-UE Relay.

6.	5G ProSe UE-to-UE Relay determines the responding 5G ProSe End UE based on the IP address received from the initiating 5G ProSe End UE and sends a Link Modification Request message to the responding 5G ProSe End UE. The Link Modification Request message includes a Relay re-selection indication, User Info ID(s) of the candidate 5G ProSe UE-to-UE Relay(s), IP address of the initiating 5G ProSe End UE and optionally the Layer-2 ID(s) of the candidate 5G ProSe UE-to-UE Relay(s).

If multiple 5G ProSe End UEs are specified in the Link Modification Request message received from the initiating 5G ProSe End UE, the 5G ProSe UE-to-UE Relay sends a PC5 Link Modification Request to each of the 5G ProSe End UEs.

7.	The responding 5G ProSe End UE selects a new 5G ProSe UE-to-UE Relay from the candidate 5G ProSe UE-to-UE Relays, based on the Relay re-selection indication in the Link Modification Request message. If the responding 5G ProSe End UE has not received a UE-to-UE Relay Discovery Announcement message from a candidate 5G ProSe UE-to-UE Relay or does not have a PC5 connection with the candidate 5G ProSe UE-to-UE Relay associated with the same RSC, then the responding 5G ProSe End UE may perform the 5G ProSe UE-to-UE Relay discovery procedure with the User Info ID of a candidate 5G ProSe UE-to-UE Relay in discovery message, and may set the Layer-2 ID of the candidate 5G ProSe UE-to-UE Relay, if received at step 6, as the Destination Layer-2 ID to carry the discovery message and initiates the procedure to set up PC5 unicast links with the new 5G ProSe UE-to-UE Relay, by reusing the procedure defined in clause 6.7.1. The PC5 signal strength of the UE-to-UE Relay Discovery Announcement message or UE-to-UE Relay Discovery Response message may be used to select the new 5G ProSe UE-to-UE Relay.

8.	The responding 5G ProSe End UE sends a Link Modification Accept message to the 5G ProSe UE-to-UE Relay, including the User Info ID of the new 5G ProSe UE-to-UE Relay, IP address of the initiating 5G ProSe End UE, IP address of the responding 5G ProSe End UE for communication via the newly selected 5G ProSe UE-to-UE Relay, and Relay re-selection indication.

9.	5G ProSe UE-to-UE Relay sends a Link Modification Accept message to the initating 5G ProSe End UE, including the User Info ID of the new 5G ProSe UE-to-UE Relay, IP address of the responding 5G ProSe End UE, IP address of the responding 5G ProSe End UE for communication via the newly selected 5G ProSe UE-to-UE Relay and Relay re-selection indication.

10-11.	Link Modification Ack is sent from the initating 5G ProSe End UE to the responding the 5G ProSe End UE via the 5G ProSE UE-to-UE Relay, including the IP address of the initiating 5G ProSe End UE for communication via the newly selected 5G ProSe UE-to-UE Relay, the IP address of the responding 5G ProSe End UE and Relay re-selection indication.

12.	The 5G ProSe End UEs transfer traffic via the newly selected 5G ProSe UE-to-UE Relay.

NOTE:	Whether a 5G ProSe End UE releases a PC5 unicast link with the original 5G ProSe UE-to-UE Relay after reselection depends on whether the PC5 unicast link is still required and UE implementation.

# * * * * End of changes * * * *