3GPP TSG-SA WG2 Meeting #157	S2-2307651

# May 22 - 26, 2023, Berlin, Germany										      (revision of S2-2306816)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0288 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Layer-3 U2U Relay link release and maintenance procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, Interdigital, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2023-05-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The Layer-2 link release over Layer-3 UE-to-UE Relay procedure needs to be corrected to align with the already specified unicast link management procedures. The Layer-2 link maintenance over Layer-3 UE-to-UE Relay procedures is still misssing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the Layer-2 link release over Layer-3 UE-to-UE Relay procedure, and add the Layer-2 link maintenance over Layer-3 UE-to-UE Relay procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Layer-3 U2U Relay management procedures are incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.1.3, 6.7.1.5 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * 1st Change * * * *

6.7.1.3	Layer-2 link release for PC5 communication via 5G ProSe Layer-3 UE-to-UE Relay

Figure 6.7.1.3-1 shows the Layer-2 link release procedure via Layer-3 UE-to-UE Relay.

Figure 6.7.1.3-1: Layer-2 Link Release procedure via Layer-3 UE-to-UE Relay

0.	UE-1 and UE-to-UE Relay, and UE-to-UE Relay and UE-2 have a unicast link established as described in clause 6.7.1.1.

1.	UE-1 sends a Disconnect Request message to UE-to-UE Relay. It is the same as step 1 of clause 6.4.3.3.

2.	Upon reception of the Disconnect Request message, UE-to-UE Relay shall respond with a Disconnect Response. It is the same as step 2 of clause 6.4.3.3.

3a.	Upon reception of the Disconnect Request message, UE-to-UE Relay sends a Disconnect Request message to the UE(s) (here UE-2), which has a unicast link for the communication with UE-1 if there are no other UEs than UE-1 using the layer-2 link. It is the same as step 1 of clause 6.4.3.3.

4a.	Upon reception of the Disconnect Request message, UE-2 shall respond with a Disconnect Response message. It is the same as step 2 of clause 6.4.3.3.

3b.	Upon reception of the Disconnect Request message, UE-to-UE Relay sends a Link Modification Request message to the UE(s) (here UE-2), which has a unicast link for the communication with UE-1 if there are other UEs than UE-1 using the layer-2 link in order to release the PC5 QoS Flow(s) and deletes the context data associated with the PC5 QoS Flow(s).

4b.	Upon reception of the Link Modification Request message, UE-2 shall respond with a Link Modification Accept message and deletes the context data associated with the PC5 QoS Flow(s) used for the communication with UE-1.

The ProSe layer of UE-to-UE Relay and UE-2 provides information about the unicast link modification to the AS layer, which enables the AS layer to update the context related to the modified unicast link.

* * * * 2nd Change * * * *

6.7.1.5	Layer-2 link maintenance for PC5 communication via 5G ProSe Layer-3 UE-to-UE Relay

The Layer-2 link maintenance over the PC5 reference point procedure as described in clause 6.4.3.5 can be used for the Layer-2 link established between UE-1 and UE-to-UE Relay and between UE-to-UE Relay and UE-2.

* * * * End of Changes * * * *