3GPP TSG-WG SA2 Meeting #157	S2-2307732

May 22 - 26, 2023, Berlin, Germany	(revision of S2-2307657)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 3944 | rev | 5 | Current version: | 18.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Co-existence of Small Data Transmission and CN based MT communication handling for UE in RRC_INACTIVE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel, Qualcomm Inc., Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RedCap_Ph2 |  |  |  |  |  | Date: |  |  | 2023-05-25 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | As discussed in S2-2303999, when Connection Resume is performed for Small Data Transmission (SDT) the UE stays in RRC_INACTIVE, whereas the current call flows for Connection Resume imply that the UE is always moved to RRC_CONNECTED state. Remove EN related to N2 message name based on RAN3 LS S2-2306310/R3-231951. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adjust the call flows for Connection Resume so that the UE can end up in either RRC_CONNECTED state (for non-SDT) or RRC_INACTIVE (for SDT). Figure 4.8.2.2-1: - Re-drew the call flow in Visio - Step 4b.1: changed “RRC_CONNECTED” to “Connection Resume” - Removed the states at the end of the call flow - Adjusted the text accordingly Figure 4.8.2.2b-1: - Step 1a.3: changed “req” to “rsp” - Step 2: changed “Request fro RRC_CONNECTED” to “Request for Connection Resume” - Adjusted the text accordingly Update N2 message names. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear co-existence between Small Data Transmission and CN based MT communication handling for UE in RRC_INACTIVE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.8.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*********** Start change **************

#### 4.8.2.2	UE Triggered Connection Resume in RRC_INACTIVE procedure

The Connection Resume procedure is used by the UE in RRC_INACTIVE state, e.g. to transition to RRC_CONNECTED state or for Small Data Transmission while in RRC_INACTIVE as specified in TS 38.300 [9].. Triggers for the UE to initiate this procedure are defined in clause 5.3.3.2.5 of TS 23.501 [2].

Figure 4.8.2.2-1: Connection Resume in RRC_INACTIVE

1.	UE to NG-RAN: RRC message (Resume ID).

The UE initiates connection resume from RRC_INACTIVE state, see TS 38.300 [9]. The UE provides its Resume ID needed by the NG-RAN to access the UE's stored Context.

2.	[Conditional] NG-RAN performs UE Context Retrieval.

UE Context Retrieval is performed when the UE Context associated with the UE attempting to resume its connection is not locally available at the accessed NG-RAN. The UE Context Retrieval procedure via NG-RAN is specified in TS 38.300 [9].

3.	NG-RAN to UE: RRC messages.

NG-RAN determines whether the UE shall be transitioned to RRC_CONNECTED state or kept in RRC_INACTIVE (e.g. the latter in case of Small Data Transmission as defined in TS 38.300 [9]).

4a.	[Conditional] N2 Path switch procedure.

If the accessed NG-RAN is able to retrieve the UE Context, the accessed NG-RAN node initiates N2 Path Switch procedure, i.e. steps 1 to 8 of clause 4.9.1.2.2 and including Xn data forwarding.

If the Connection Resume procedure is a response to RAN paging which is triggered by 5GC due to an N2 interface procedure, NG-RAN and 5GC handle the N2 interface procedure as a collision described in clause 4.9.1.2.

4b.	[Conditional] N2 Notification,

4b.1	If the accessed NG-RAN is the same as the NG-RAN that configured RRC_INACTIVE and still has the UE context, NG-RAN sends

- 	an N2 Notification to the AMF indicating the UE is in RRC_CONNECTED, if an AMF requested N2 Notification (see clause 4.8.3); or,

-	an MT Communication Handling request to the AMF indicating the UE is now reachable for downlink data and/or signalling if Connection Inactive procedure with CN based MT communication handling (see clause 4.8.1.1a) has been performed previously.

4b.2	The AMF invokes Nsmf_PDUSession_UpdateSMContext Request towards SMF indicating the Downlink data delivery for each PDU session with active user plane, if the AMF has requested data buffering as described in clause 4.8.1.1a.

4b.3	N4 session modification procedure is triggered by the SMF. If data buffering is handled in the UPF, the SMF updates the UPF with appropriate rules to trigger data delivery.

4b.4	The SMF sends the Nsmf_PDUSession_UpdateSMContext response.

4b.5	The AMF sends the N2 MT Communication Handling response message to NG-RAN.

If NG-RAN determines that the connection resume is for Small Data Transmission as defined in TS 38.300 [9] and steps 4b.1 to 4b.5 have been performed, then NG-RAN keeps the UE in RRC_INACTIVE state. If the UE is configured with RRC Inactive with eDRX>10.24s, the NG-RAN may send an N2 message to 5GC as described in clause 4.8.1.1a step 2 so the CN can then handle mobile terminated (MT) communication.

*********** End change **************