3GPP TSG-WG SA2 Meeting #157	S2-2307733

# May 22 – 26, 2023, Berlin, Germany		(revision of S2-2307658)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4182 | rev | 2 | Current version: | 18.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NGAP message clarification |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RedCAP_Ph2 |  |  |  |  |  | Date: |  |  | 2023-05-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on LS from RAN3 S2-2306310/R3-231951, RAN3 has decided the NGAP message names related to the CN based MT communicaiton handling procedues for UE in RRC_INACTIVE with long eDRX. The 3 ENs in the procedure can be removed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | -Update the NGAP message names in clause 4.8.1.1a. -Remove corresponding ENs related to the NGAP message. -Clarify further on the NG-RAN requested CN based MT handling |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unresolved ENs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.8.1.1a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 4.8.1.1a	Connection Inactive procedure with CN based MT communication handling

This procedure may be initiated by the serving NG-RAN node when CN based mobile terminating (MT) communication handling is requested for a UE that is configured with eDRX cycle value longer than 10.24 seconds for RRC_INACTIVE state and has at least one PDU session with active user plane as defined in clause 5.31.7 of TS 23.501 [2].

Figure 4.8.1.1a-1: NG-RAN initiated Connection Inactive procedure with CN based MT communication handling

0.	The UE is registered in the network with negotiated eDRX parameters for CM-IDLE state and is in CM-CONNECTED with RRC_CONNECTED state. The AMF provides the eDRX cycle value for CM-IDLE state to NG-RAN as part of the RRC Inactive Assistance Information as defined in clause 5.3.3.2.5 of TS 23.501 [2].

1.	NG-RAN determines eDRX cycle value for UE in RRC_INACTIVE state and decides to initiate Connection Inactive with CN based MT communication handling as specified in clause 5.31.7.2.4 of TS 23.501 [2].

1a.	Optionally the NG-RAN may initiate state transition from RRC_CONNECTED to RRC_INACTIVE with RRC configuring eDRX value as specified in TS 38.300 [9]. The NG-RAN may send the request in step 2 towards CN immediately following step 1a or the NG-RAN may delay this request towards CN as specified in clause 5.31.7.2.1 of TS 23.501 [2].

2.	Either immediately following step 1a or after having delayed the request for NG-RAN based on implementation, the NG-RAN sends N2 MT Communication Handling request message to AMF indicating the UE is transitioning to RRC_INACTIVE state. The CN takes the NG-RAN request into consideration and handles MT communication. The NG-RAN also provides the eDRX cycle value for RRC_INACTIVE to AMF.

If the NG-RAN receives DL NAS message and the UE is in RRC_INACTIVE with RRC configured eDRX cycle and is considered not reachable, NG-RAN indicates to the AMF a NAS non-delivery and then initiates for the CN to handle mobile terminated (MT) communication.

3.	For each of the PDU sessions with user plane resources have been activated, the AMF invokes Nsmf_PDUSession_UpdateSMContext Request (PDU Session ID, Cause, Operation type, User Location Information, Age of Location Information, N2 SM Information (Secondary RAT usage data), CN based MT handling indication) towards SMF. The Operation Type is set to a value that indicates to stop user plane DL data transmissions towards the UE and enable data buffering. The SMF starts data buffering for MT data if the data buffering is handled in SMF.

4.	If data buffering is handled in the UPF, the SMF updates the UPF with proper rules for MT data handling.

5.	The SMF sends the Nsmf_PDUSession_UpdateSMContext response.

6.	The AMF sends N2 MT Communication Handling response message to NG-RAN acknowledging the NG-RAN request and indicating the AMF has taken the NG-RAN request into account. The AMF considers the UE is in CM-CONNECTED with RRC_INACTIVE state.

7.	If the UE connection is not released as specified in step 1a, the NG-RAN initiates state transition from RRC_CONNECTED to RRC_INACTIVE with RRC configuring the eDRX cycle value as specified in TS 38.300 [9].

# * * * * End of changes * * * *