SA WG2 Meeting #157	S2-2306338

Berlin, Germany, May 22 – 26, 2023	revision of S2-230xxxx

Title:	[Draft] Reply LS on MBS service area update clarification

Response to:	LS on MBS service area update clarification (S2-2306330 /S6-231606)

Release:	Rel-17

Work Item:	5MBS

Source:	3GPP TSG SA WG2

To:	3GPP TSG SA WG6, 3GPP TSG CT WG3

Cc:	3GPP TSG CT WG4

Contact person:	Thomas Belling

Thomas (dot) belling (at) Nokia (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# Overall description

SA2 thanks SA6 for their LS, where SA4 wrote:

In public safety and verticals scenarios, the service area may be changed due to the mobility of the group members. SA6 has concerns that failure may occur once the AF triggers the MBS session update with a new MBS service area. …

Q1. SA6 would like to ask SA2 whether the AF/NEF will interact with the NRF to re-select the capable MB-SMF during the MBS session update with requested MBS service area?

Q2. SA6 would like to ask CT3 and CT4 to clarify the scenario(s) when the error “UNKNOWN_MBS_SERVICE_AREA 404 Not Found The requested MBS service area (e.g. identified by the Area Session ID) cannot be found.” in 3GPP TS 29.522 can occur? Is the MBS service area not found in MB-SMF, NRF or NEF? Can it occur during MBS session update?

Q3. SA6 would like to ask SA2 to confirm whether the scenarios described above from SA6, CT4 and CT3 can occur during MBS session update. If yes, what are the mechanism(s) defined by SA2 to be employed to overcome an MBS session update failure?

SA2 would like to answer the questions as follows:

Q1: In Rel-17 MBS, there are restrictions for deployments where SMFs and/or MB-SMFs only serve specific service areas. This is documented in TS 23.247:

## 4.1	Overview of multicast and broadcast communication

…

Interaction between Multicast/Broadcast Service and support of deployments topologies with specific SMF Service Areas is not specified in this Release.

As a consequence, an MBS session creation where an MBS service area is not covered by the MB-SMF service area of a single MB-SMF, and an MBS session modification, where an MBS service area is modified in such a manner that it is no longer covered by the MB-SMF service area of a single MB-SMF cannot be supported. Related AF requests will be rejected. Also no MB-SMF reselection for an ongoing MBS session is supported. During the MBS session update with requested MBS service area, the NEF/MBSF will forward the MBS session update request to the serving MB-SMF corresponding to the MBS session ID, without interacting the NRF.

Q3: The scenario described from SA6 can happen. SA2 will further work on recommendations how this issue with MBS session update failures can be mitigated.

# 2	Actions

To SA6

ACTION: 	SA2 asks SA6 to kindly take the provided feedback into account.

# 3	Dates of next TSG SA WG 2 meetings

SA2#158                 21st August – 25th August 2023 	Gothenburg, Sweden