# 3GPP TSG-SA WG2 Meeting #157																S2-2307763

# May 22 – 26, 2023, Berlin, Germany									    (revision of S2-2307129)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.256 | CR | 0098 | rev | 3 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | N2 and Xn based HO for UAV |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UAS_Ph2 |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | N2 and Xn based HO procedures are not specified in TS 23.256 specification |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to clarify that target NG-RAN will receive required parameters like "A2X services authorized" indication, UE-PC5-AMBR, per-PC5 RAT control authorization, and PC5 QoS parameters for successful HO of UAV. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | N2 and Xn based HO may not work for UAV. In Rev#2 submitted to SA2#157, align the wording related to per PC5 RAT with CR#0094R1. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 5.11.4 (clause# modification to 5.11.6), 5.11.5 (clause# modification to 5.11.7), 5.11.6 (clause# modification to 5.11.8), 5.11.4 (new), 5.11.5 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | SA2#156E: Rev#1 approved SA2#157: Rev#2 submitted |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[3]	3GPP TS 23.502: "Procedures for the 5G System (5GS)".

[4]	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs".

[5]	3GPP TS 22.125: "Unmanned Aerial System (UAS) support in 3GPP".

[6]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[7]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[8]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[9]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[10]	3GPP TS 33.256: "Security aspects of Uncrewed Aerial Systems (UAS)".

[11]	3GPP TS 23.287: "Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services".

[12]	3GPP TS 23.285: "Architecture enhancements for V2X services".

[13]	ASTM F3411.19: "Standard Specification for Remote ID and Tracking".

[14]	ASD-STAN prEN 4709-002:2022-03: "Aerospace series - Unmanned Aircraft Systems - Part 002: Direct Remote Identification".

[15]	3GPP TS 38.300: "NR; NR and NG-RAN Overall description; Stage-2".

[1x]                      3GPP TS 38.423: "NG-RAN; Xn Application Protocol (XnAP)".

* * * Next Change * * *

5.11.4	N2 Handover procedure

The N2 based handover or the Inter-RAT to NG-RAN handover procedures for UE are performed as defined in TS 23.502 [3] with the following additions:

-	If the UE is PC5 capable for A2X, and the UE is authorized to use A2X communication over PC5 reference point, then the target AMF shall send the "A2X services authorized" indication per PC5 RAT, UE-PC5-AMBR per PC5 RAT and PC5 QoS parameters to the target NG-RAN as follows:

-	For the intra AMF handover, the "A2X services authorized" indication per PC5 RAT, UE-PC5-AMBR per PC5 RAT and PC5 QoS parameters are included in the NGAP Handover Request message.

-	For the inter AMF handover or Inter-RAT handover to NG-RAN, the "A2X services authorized" indication per PC5 RAT, UE-PC5-AMBR per PC5 RAT and PC5 QoS parameters are included in the NGAP Handover Request message sent to the target NG-RAN.

5.11.5	Xn Handover procedure

The Xn based handover procedures for UE are performed as defined in TS 23.502 [3] with the following additions:

-	If the "A2X services authorized" indication is included in the UE context (as described in TS 38.423 [1x]), then the source NG-RAN shall include a "A2X services authorized" indication per PC5 RAT, UE-PC5-AMBR per PC5 RAT and PC5 QoS parameters in the XnAP Handover Request message to the target NG-RAN.

-	If the UE is PC5 capable for A2X, and the UE is authorized to use A2X communication over PC5 reference point, then the AMF shall send the "A2X services authorized" indication per PC5 RAT, the UE-PC5-AMBR per PC5 RAT and PC5 QoS parameters to the target NG-RAN in the Path Switch Request Acknowledge message.

5.11.6	Subscriber Data Update Notification to AMF

The procedure of Subscriber Data Update Notification to AMF is performed as defined in TS 23.502 [3] with the following additions:

-	The Nudm_SDM_Notification service operation may contain the "A2X services authorized" indication or the UE-PC5-AMBR, or cross-RAT PC5 control authorization or any combination;

-	The AMF updates the UE Context with the above new A2X subscription data.

-	When the AMF updates UE context stored at NG-RAN, the UE context contains the A2X subscription data.

5.11.7	Delivery of PC5 QoS parameters to NG-RAN

The UE Policy Association Establishment procedure and UE Policy Association Modification procedure, as defined in TS 23.502 [3], is used to provide to the AMF PC5 QoS parameters used by NG-RAN. When receiving Npcf_UEPolicyControl_Create Request from the AMF which includes the PC5 capability for A2X or when receiving the updated subscription data from UDR, the PCF generates the PC5 QoS parameters used by NG-RAN corresponding to a UE.

The (V-)PCF provides the information to the AMF as follows:

-	In the roaming case, the H-PCF includes the PC5 QoS parameters used by NG-RAN in the Npcf_UEPolicyControl_Create Response message or Npcf_UEPolicyControl UpdateNotify Request message sent to V-PCF in an N2 PC5 policy container, and V-PCF relays this N2 PC5 policy container as the N2 container in the Namf_Communication_N1N2MessageTransfer message sent to AMF.

-	In the non-roaming case, the PCF includes the PC5 QoS parameters used by NG-RAN in an N2 container in Namf_Communication_N1N2MessageTransfer message sent to AMF.

When the AMF determines that the N2 PC5 policy container comes from (V-)PCF, the AMF stores such information as part of the UE context.

In the UE Configuration Update procedure triggered by UE Policy Association Establishment or UE Policy Association Modification:

-	The AMF forwards the PC5 QoS parameters in the NGAP message to the NG-RAN if a N2 PC5 policy container is received in the Namf_Communication_N1N2MessageTransfer message.

-	The AMF forwards the PC5 QoS parameters in the NAS message to UE by using the UE Configuration Update procedure for transparent UE Policy delivery procedure defined in clause 4.2.4.3 of TS 23.502 [3] if a UE Policy Container is received in the Namf_Communication_N1N2MessageTransfer message.

NOTE 1:	If the PC5 QoS parameters are provided to both NG-RAN and UE, both the N2 PC5 Policy Container and the UE Policy Container are included in the Namf_Communication_N1N2MessageTransfer message.

NOTE 2:	Non-UE specific PC5 QoS parameters, e.g. default PC5 QoS parameters, can also be locally configured in NG-RAN. How such configuration is performed is out of scope of this specification.

5.11.8	PC5 Capability for A2X indication and A2X related information per PC5 RAT

A UE may support multiple PC5 RATs (i.e. LTE PC5 and NR PC5). For such UE, the PC5 Capability for A2X indication and A2X related information can be per PC5 RAT as described in clause 5.11.2.

The cross-RAT PC5 control authorization indicates whether LTE Uu controls LTE PC5 and/or NR PC5 from the cellular network, and whether NR Uu controls LTE PC5 and/or NR PC5 from the cellular network.

NOTE:	Stage 3 does not define an explicit cross-RAT PC5 control authorization IE and it is indicated in an implicit manner.

* * * End of Changes * * *