3GPP TSG-WG SA2 Meeting #157	S2-2307794

Berlin, Germany, May 22 – 26, 2023	(revision of S2-2307897)

Title:	Reply LS on carrier mapping for unicast SL CA

Response to:	LS on carrier mapping for unicast SL CA (S2-2306317/R2-2304236)

Release:	Rel-18

Work Item:	NR_SL_enh2

Source:	SA2

To:	RAN2

Cc:	CT1

Contact Person:

Name:	LaeYoung Kim

E-mail Address:	laeyoung.kim@lge.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

SA2 would like to thank RAN2 regarding the LS on carrier mapping for unicast SL CA.

SA2 discussed the questions asked by RAN2 and would like to provide the following answers.

- Question 1: According to TS 24.588, V2X layer is only provisioned with a mapping between service identifier and initial L2 address used for unicast. But service identifier is invisible to AS-layer, and the initial L2 ID will only be used in DCR and be replaced by a self-chosen L2 ID in PC5-S link establishment procedure. Then, after L2 ID changes, whether/how can UE's AS layer obtain the mapping between L2 ID and frequencies?

- Answer 1 with question for clarification:

According to discussion in SA2, most companies' understanding is Question 1 is about how the AS layer can know frequency information associated with the PC5-S messages sent during the PC5 unicast link establishment procedure. Anyhow some companies think that Question 1 is related to the scenario after the PC5 unicast link is established. Therefore, SA2 would like to ask RAN2 about what the scenario Question 1 is related to.

SA2 also discussed how to let the AS layer know frequency information associated with the PC5-S messages sent for PC5 unicast link establishment, by providing frequency information together with the PC5-S message to the AS layer from the V2X layer, however there was no corresponding CR approved yet in SA2.

- Question 2: According to TS 24.587, PC5 unicast allows UEs to add/modify/remove V2X services/PC5 QoS flows to the same L2 ID pair. Then, given service info is invisible to AS layer, how can the UE ensure the modified V2X services to be transmitted only on the corresponding frequencies mapped in the V2X layer?

- Answer 2: Based on the following operations as described in clauses 5.4.1.1.3 and 6.3.3 of TS 23.287, it is considered that the UE can ensure the modified V2X services to be transmitted only on the corresponding frequencies mapped in the V2X layer.

i) The AS layer maintains the PC5 unicast link related context for the lifetime of the PC5 unicast link based on the various information provided by the V2X layer, e.g. PC5 Link Identifier, source L2 ID, destination L2 ID, etc.

ii) The V2X layer updates the AS layer about PC5 QoS Flow addition/modification/removal for the established PC5 unicast link. For PC5 QoS Flow addition, the V2X layer provides one or more radio frequencies associated with the added PC5 QoS Flow to the AS layer. The V2X layer can determine the radio frequencies based on the mapping of V2X service type(s) associated with the PC5 QoS Flow to V2X frequencies by using the related configuration as specified in clause 5.1.2.1 of TS 23.287 (i.e. "The mapping of V2X service types to V2X frequencies with Geographical Area(s)").

iii) The V2X layer ensures that V2X service types associated with different radio frequencies are classified into distinct PC5 QoS Flows.

2. Actions:

To RAN2 group.

ACTION: 	SA2 kindly asks RAN2 to provide answer about what the scenario Question 1 is related to, and take above answers into consideration for further work.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting#158	August 21 – 25, 2023	Gothenburg, Sweden

TSG-SA2 Meeting#159	October 09 – 13, 2023	Xiamen, China